3GPP TSG-RAN WG4 Meeting # 107												R4-2308227

Incheon, KR, May 22 – May 26, 2023

Source: 	vivo

Title: 	Discussion on PTRS configuration for FR2 256QAM

Agenda Item:	8.7.2

Document for:	Approval

- Introduction

In the last meeting, the FR2 UL 256QAM discussion was stalled by PTRS configuration and the simulation assumption [1] was agreed for further evaluation. In this contribution, we provide our views on the PTRS configuration rules.

- Discussion

  - Simulation assumption

The simulation assumption in this contribution is listed in Table I

Table I Simulation assumption for PTRS configuration

| Parameters | Value |
| --- | --- |
| Frequency | 29, 39 GHz |
| SCS | 120kHz |
| BW | 100MHz |
| RB Size | 64RB for 120kHz |
| Background AWGN | No additional noise |
| Time offset/Frequency offset | 0 |
| Antenna configuration | 1T1R |
| Modulation | 256QAM |
| Waveform type | CP-OFDM / DFT-s-OFDM |
| DMRS | 3 symbols per slot (UL DMRS add-pos = 2) |
| PTRS configuration | OFF/ON For CP-OFDM: L-PTRS (Time density) = 1 (every 1 symbol) K-PTRS (Freq density) =2 (every 2 RBs) For DFTs-OFDM: L-PTRS (Time density) = 1 (every symbol) N_group = 8, N_samp = 4 Other PTRS configurations can be considered for narrow RB allocations, especially for DFT-s-OFDM. |
| EVM measurement | Data-aided EVM calculation, based on ideal data signal |
| Phase noise profiles | For 29GHz: Option 1: Example 1 in TR38.803 for 29 GHz. Option 2: Example 2 in TR38.803. Option 3: min (Example 1, Example 2) Option 4: For 39GHz: Option 1: Example 1 in TR38.803 with following parameter. Option 2: Example 2 in TR38.803. Option 3: min (Example 1, Example 2) Option 4: |

In the simulation, distortion other than phase noise is not considered, e.g., PA non-linearity, transceiver impairment, etc., to ensure the results only show the feasibility of different phase noise profiles.

  - PTRS configuration for 29GHz band

The simulation results for 29GHz bands are shown in Table II

Table II simulation results for 29GHz band

| 64RB |  | EVM no correction | EVM with correction | benefit |
| --- | --- | --- | --- | --- |
| CP-OFDM | Option 1 | -28.3288 | -28.9129 | 0.5841 |
|  | Option 2 | -25.3244 | -25.5883 | 0.2639 |
|  | Option 3 | -31.7997 | -33.0798 | 1.2801 |
|  | Option 4 | -33.0942 | -33.3608 | 0.2666 |
| DFT-s-OFDM | Option 1 | -28.8567 | -27.1551 | -1.7016 |
|  | Option 2 | -25.4507 | -28.9614 | 3.5107 |
|  | Option 3 | -32.8215 | -29.8556 | -2.9659 |
|  | Option 4 | -33.2358 | -30.0408 | -3.195 |

Based on the simulation results, we have the following observations:

Observation 1: The phase noise profile described in option 1 and option 2 is not feasible for the 29GHz band.

Observation 2: CP-OFDM can benefit from PTRS correction in UL 256QAM but DFT-s-OFDM can’t.

To further investigate PTRS configuration under different RB allocations, the simulation is performed and the results are shown in Table III. Considering that Option 1 and Option 2 are not feasible, only Option 3 and Option 4 are evaluated.

Table III PTRS correction benefit evaluation for different RB allocation

|  |  |  | EVM no correction | EVM with correction | benefit |
| --- | --- | --- | --- | --- | --- |
| 32RB | CP-OFDM | Option 3 | -32.5213 | -34.0762 | 1.5549 |
|  |  | Option 4 | -33.2888 | -34.1761 | 0.8873 |
|  | DFT-s-OFDM | Option 3 | -32.6484 | -30.7554 | -1.893 |
|  |  | Option 4 | -33.4813 | -31.4505 | -2.0308 |
| 16RB | CP-OFDM | Option 3 | -33.4905 | -33.9333 | 0.4428 |
|  |  | Option 4 | -33.9908 | -34.2379 | 0.2471 |
|  | DFT-s-OFDM | Option 3 | -35.5055 | -33.3672 | -2.1383 |
|  |  | Option 4 | -34.1009 | -32.5789 | -1.522 |
| 8RB | CP-OFDM | Option 3 | -33.3388 | -33.9874 | 0.6486 |
|  |  | Option 4 | -34.4442 | -34.8899 | 0.4457 |
|  | DFT-s-OFDM | Option 3 | -33.3038 | -32.9937 | -0.3101 |
|  |  | Option 4 | -34.5513 | -34.0163 | -0.535 |
| 4RB | CP-OFDM | Option 3 | -33.9173 | -35.0024 | 1.0851 |
|  |  | Option 4 | -35.6699 | -36.7056 | 1.0357 |
|  | DFT-s-OFDM | Option 3 | -34.7217 | -34.3119 | -0.4098 |
|  |  | Option 4 | -36.4809 | -35.7798 | -0.7011 |

Observation 3: Even under narrow RB allocation, the DFT-s-OFDM still hard to benefit from PTRS correction.

Take the 8RB case as an example, we further evaluated DFT-s-OFDM with different PTRS configurations, and the simulation results are shown in Table IV

Table IV PTRS correction benefit evaluation for DFT-s-OFDM with narrow RB allocation under different PTRS configurations.

| PTRS configuration |  | EVM no correction | EVM with correction | benefit |
| --- | --- | --- | --- | --- |
| L-PTRS = 1 N_group = 4, N_samp = 4 | Option 3 | -33.3038 | -33.946 | 0.6422 |
|  | Option 4 | -34.5513 | -34.7963 | 0.245 |
| L-PTRS = 1 N_group = 2, N_samp = 4 | Option 3 | -33.3038 | -32.8345 | -0.4693 |
|  | Option 4 | -34.5513 | -34.022 | -0.5293 |
| L-PTRS = 1 N_group = 2, N_samp = 2 | Option 3 | -33.3038 | -32.9937 | -0.3101 |
|  | Option 4 | -34.5513 | -34.0163 | -0.535 |

Observation 4: Only configuration L-PTRS = 1, (N_group = 4, N_samp = 4) can make narrow RB allocation benefit from PTRS correction.

  - PTRS configuration for 39GHz band

The simulation results for 39GHz are shown in Table V.

Table V simulation results for the 39GHz band

| 64RB |  | EVM no correction | EVM with correction | benefit |
| --- | --- | --- | --- | --- |
| CP-OFDM | Option 1 | -25.1733 | -25.3235 | 0.1502 |
|  | Option 2 | -22.7246 | -22.9743 | 0.2497 |
|  | Option 3 | -29.8719 | -30.2049 | 0.333 |
|  | Option 4 | -29.9468 | -30.182 | 0.2352 |
| DFT-s-OFDM | Option 1 | -25.3171 | -23.5189 | -1.7982 |
|  | Option 2 | -22.8672 | -26.3839 | 3.5167 |
|  | Option 3 | -30.0035 | -28.8569 | -1.1466 |
|  | Option 4 | -30.0888 | -28.5366 | -1.5522 |

It is noted that only phase noise is considered in the simulation and when other distortion needs to be counted in MPR evaluation, even under option 3 or option 4 phase noise, it is still challenging to achieve the 29.1 dB EVM requirement for UE.

Observation 5: Either option 3 or option 4 may still be infeasible for MPR evaluation in the 39GHz band.

Based on the analysis above, we propose:

Proposal 1: Either option 3 or option 4 can be used for MPR evaluation in 29GHz bands.

Proposal 2: Adopt L-PTRS = 1 K-PTRS =2 as PTRS configuration for CP-OFDM, and do not configure PTRS for DFT-s-OFDM.

Proposal 3: Focus on MPR evaluation in 29GHz bands first, and further discuss feasible phase noise profile in 39GHz bands.

- Conclusion

In this contribution, we provide our views on PTRS configuration for FR2 256QAM, and our proposals are listed below:

Observation 1: The phase noise profile described in option 1 and option 2 is not feasible for the 29GHz band.

Observation 2: CP-OFDM can benefit from PTRS correction in UL 256QAM but DFT-s-OFDM can’t.

Observation 3: Even under narrow RB allocation, the DFT-s-OFDM still hard to benefit from PTRS correction.

Observation 4: Only configuration L-PTRS = 1, (N_group = 4, N_samp = 4) can make narrow RB allocation benefit from PTRS correction.

Observation 5: Either option 3 or option 4 may still be infeasible for MPR evaluation in the 39GHz band.

Proposal 1: Either option 3 or option 4 can be used for MPR evaluation in 29GHz bands.

Proposal 2: Adopt L-PTRS = 1 K-PTRS =2 as PTRS configuration for CP-OFDM, and do not configure PTRS for DFT-s-OFDM.

Proposal 3: Focus on MPR evaluation in 29GHz bands first, and further discuss feasible phase noise profile in 39GHz bands.

- References

- R4-2306603	WF on FR2-1 UL 256QAM, xiaomi, RAN4#106bis