3GPP TSG-RAN WG4 Meeting # 107												R4-2308228

Incheon, KR, May 22 – May 26, 2023

Source: 	vivo

Title: 	Discussion on beam correspondence requirement applicability

Agenda Item:	8.7.3.1

Document for:	Approval

- Introduction

In the last meeting, the requirement applicability of beam correspondence in initial access is discussed but no conclusion is achieved. In this contribution, we further provide our views on this issue.

- Discussion

In [1], the UE capability issue was raised:

Sub-topic 1-6 UE capability

- Proposals

  - Option 1: Only the UE support both beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 is considered can support msg1 beam correspondence.

  - Option 2: For supporting UE beam correspondence requirements for RRC_INACTIVE and initial access UE needs to support both beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 UE capabilities.

  - Option 3: RAN4 specs to clarify that Rel-18 or newer UEs supporting beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 is considered as a Msg1 beam correspondence capable UE.

Way forward/Agreements:

- Follow the previous RAN4 decision that new UE capability is not introduced for initial access.

- FFS for the options above.

The first issue here is whether beam correspondence in initial access should be mandatory for all UE. Recall that in RRC_CONNETED, even though different capabilities exist, beam correspondence is a mandatory feature for all UE. However, to accommodate different UE implementations, the assistance of beam sweeping is allowed for some “weak” UE.

Observation 1: The beam correspondence is mandatory for all UE in the RRC_CONNECTED state but differentiated by UE capability considering not all UE can achieve the same UE requirement.

In addition, as discussed in the previous meeting that the UE capability is not introduced. This makes sense because the NW can only obtain UE capability information when the UE has completed initial access, so a capability for initial access seems quite meaningless. It is also hard to indicate that beam correspondence in initial access is an optional feature.

Observation 2: UE can only acquire UE capability information after initial access, and it is hard to indicate that beam correspondence in initial access is optional.

In our view, a compromise that is similar to the BC requirement in the RRC_CONNECTED state here can be that different requirements apply to UE which can be differentiated by UE capabilities in RRC_CONNECTED. We provide more details in [2]

Proposal 1: Beam correspondence in initial access should be mandatory for all UE, but the requirement can be differentiated based on the capability in RRC_CONNECTED.

Another issue here is whether this feature can be release independent. In our view, whether the UE support beam correspondence in the previous release is a UE implementation issue. Even for the UE cannot support beam correspondence in IA, UE still can finish the IA by performing beam sweeping only if the delay is acceptable, so we prefer this feature to be only introduced for R18 onward UE.

Observation 3: The legacy UE may not be designed to mandatory support beam correspondence during initial access

Proposal 2: The feature of supporting beam correspondence in initial access is only introduced for R18 onward UE.

- Conclusion

In this contribution, we provide our views on beam correspondence requirement applicability, and our proposals are listed below:

Observation 1: The beam correspondence is mandatory for all UE in the RRC_CONNECTED state but differentiated by UE capability considering not all UE can achieve the same UE requirement.

Observation 2: UE can only acquire UE capability information after initial access, and it is hard to indicate that beam correspondence in initial access is optional.

Observation 3: The legacy UE may not be designed to mandatory support beam correspondence during initial access

Proposal 1: Beam correspondence in initial access should be mandatory for all UE, but the requirement can be differentiated based on the capability in RRC_CONNECTED.

Proposal 2: The feature of supporting beam correspondence in initial access is only introduced for R18 onward UE.

- References

- R4-2306600	WF on beam correspondence in initial access and RRC_INACTIVE, Nokia, RAN4#106e-bis

- R4-2308229	Discussion on beam type and relaxation in initial access, 	vivo, RAN4#107