3GPP TSG-RAN WG4 Meeting # 107												R4-2308230

Incheon, KR, May 22 – May 26, 2023

Source: 	vivo

Title: 	Discussion on beam type and relaxation in initial access

Agenda Item:	8.7.3.2

Document for:	Approval

- Introduction

In the past meeting, the discussion for beam correspondence requirement in initial access is stalled since it is hard to find a decent requirement framework to accommodate different beam types. In this contribution, we share a possible way to solve this issue and make progress on the WI.

- Discussion

In [1], the BC tolerance is discussed and a possible relaxation between the rough beam and the fine beam was raised；

Sub-topic 1-5 BC tolerance

- Proposals

  - Option 1: BC tolerance is applicable.

    - Option 1a: The same as Rel-16.

    - Option 1b: New tolerance is introduced.

    - Option 1c: it’s suggested to study the tolerance requirements especially for UE supporting BC with beam sweeping in RRC_CONNECTED.

  - Option 2: BC tolerance is not applicable.

  - Option 3: If these two types of UE capability will be reused in R18, which means UE supporting BC without beam sweeping has better performance, tolerance requirement is needed for UE with relatively bad BC performance like R16 UE with beam sweeping. If there is only a kind of UE compared with R16, tolerance requirement is no need.

Way forward/Agreements:

- BC tolerance requirement defined in Rel-15 is not applicable to initial access and RRC_INACTIVE state, according to WID “Specify UE beam correspondence requirements for initial access and RRC_INACTIVE state, for SSB-based beam correspondence without UL beam sweeping.”

- Encourage companies to provide justification if relaxation is needed in IA for UE not supporting BC without UL beam sweeping.

Generally, there are two types of UE in the previous release – support beam correspondence with or without UL beam sweeping, which is indicated by capability beamCorrespondenceWithoutUL-BeamSweeping. Further, based on the different RS, two different capabilities are defined in R16: beamCorrespondenceSSB-based-r16 and beamCorrespondenceCSI-RS-based-r16. In the initial access, only SSB exists and the beam sweeping is not allowed as we agreed in the previous meeting, so only the UE support both “beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16” is capable to achieve the same requirement as RRC_CONNECTED state in initial access.

Observation 1: For the UE support of both “beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16”, it is possible to achieve exactly the same requirement as RRC_CONNECTED state in initial access.

For other UE, it is hard to achieve such high requirements since the prerequisites are not met, so the requirement relaxation is needed.

Observation 2: For the UE does not support “beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16”, the requirement relaxation is needed.

Based on the analysis above, we think it is better to define different requirements based on the capabilities. The relaxation is 7 dB which is from RRM spec for rough beam.

Proposal 1: Develop RF requirements based on different UE capabilities for beam correspondence in initial access:

- For UE support “beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16”, same beam correspondence requirement in RRC_CONNECTED is applied.

- For UE doesn’t support “beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16”, 7 dB relaxation is applied to both min peak EIRP and spherical coverage.

Based on the framework above, the side condition also needs to be different.

Proposal 2: Different side conditions are applied based on the UE capability during the verification:

- For UE support “beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16”, SSB SNR≥ 6 dB can be reused.

- For UE doesn’t support “beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16”, SSB SNR should be adjusted to ≥ 13 dB

- Conclusion

In this contribution, we provide a possible way to define the requirement and our proposals are listed below:

Observation 1: For the UE support of both “beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16”, it is possible to achieve exactly the same requirement as RRC_CONNECTED state in initial access.

Observation 2: For the UE does not support “beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16”, the requirement relaxation is needed.

Proposal 1: Develop RF requirements based on different UE capabilities for beam correspondence in initial access:

- For UE support “beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16”, same beam correspondence requirement in RRC_CONNECTED is applied.

- For UE doesn’t support “beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16”, 7 dB relaxation is applied to both min peak EIRP and spherical coverage.

Proposal 2: Different side conditions are applied based on the UE capability during the verification:

- For UE support “beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16”, SSB SNR≥ 6 dB can be reused.

- For UE doesn’t support “beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16”, SSB SNR should be adjusted to ≥ 13 dB

- References

- R4-2306600	WF on beam correspondence in initial access and RRC_INACTIVE, Nokia, RAN4#106bis