3GPP TSG-RAN WG4 Meeting #107	R4-2308258

# Incheon, KR, May 22 – May 26, 2023

Agenda item:	 11.1

Source:	 vivo

Title:	Further discussions on RedCap HPUE

Document for:	 Approval

- Introduction

In RAN#99 meeting, RAN task RAN4 on the following discussion and report back to RAN#100:

- To study how to enable HPUE including PC2 for RedCap in the band agnostic way

- To study the applicable form factor, e.g., sensor, camera

In RAN4#106bis meeting, the group makes progress on how to enable HPUE RedCap in a band agnostic way, but no decision on applicable form factor [1].

In this contribution, we share our views on RedCap HPUE form factor.

- Discussions

In the WF [1], there are two options:

Issue 1: Form factor for PC2 RedCap HPUE

Candidate options for further discussion:

- Option 1: Not restrict the applicable form factor for PC2 RedCap HPUE in the specification

- Option 2: Clearly add the restriction on the applicable form factor for PC2 Redcap in the specification

- Form factor

Regarding the UE assumption for RedCap form factor discussions, in RAN1 studies, the ~3dB antenna efficiency loss was assumed only for wearable devices due to form factor limitation, in TR 38.875:

9.1	Coverage recovery evaluation

9.1.1	Urban scenario at 2.6 GHz

<skip unnecessary part>

It should be noted that the 3 dB loss is resulted from the UE antenna efficiency loss assumed for the wearable use cases. Furthermore, the same target data rate of 100 kbps for PUSCH is assumed for both RedCap UE and the reference NR UE (see evaluation methodology described in clause 6.3). A smaller or no coverage loss for PUSCH is expected if the target data rate for RedCap UE is reduced.

Observation 1: 3dB antenna loss is only assumed for RedCap wearable devices.

In addition, even with the antenna efficiency loss, lower cell edge target data rate can be acceptable for wearables (e.g. smart watch) compared with smart phones (1Mbps cell edge target data rate assumed in RAN1 studies). Given different target data rate can be accepted, the justification on UL coverage issue for RedCap UEs is not clear to the group.

Observation 2: Even with the antenna efficiency loss, lower cell edge target data rate can be acceptable for wearables (e.g. smart watch) compared with smart phones (1Mbps cell edge target data rate assumed in RAN1 studies).

Observation 3: The justification on UL coverage issue for RedCap UEs is not clear to the group.

Regarding other UE types, there is no such antenna loss assumption for other RedCap devices which does not have form factor limitation, e.g. industrial wireless sensors, video surveillance.

Observation 4: For sensor and camera form factor, there is no 3dB antenna loss assumption.

On the other hand, based on the TRP OTA measurement results from LTE watch and smartphone under the same configuration, there is no big OTA performance difference. Now move to NR, the real OTA performance for RedCap has not been verified given the OTA test method is still under discussion in Rel-18 TRP TRS WI.

Observation 5: The OTA test method for NR RedCap TRP measurement is under development in Rel-18 TRP TRS WI.

From UE implementation perspective, higher output power will cause not only cost issue, but also power consumption and overheating issues which are quite sensitive to wearable devices. Therefore, we have the following two proposals:

Proposal 1: Following RAN guidance, RAN4 should focus on the applicable form factor on sensor and camera.

Proposal 2: The restriction on the applicable form factor for PC2 Redcap should be reflected in the CR directly, or in the new WID.

- Conclusion

In this contribution, we provide our views on RedCap HPUE, and have the following proposals:

Observation 1: 3dB antenna loss is only assumed for RedCap wearable devices.

Observation 2: Even with the antenna efficiency loss, lower cell edge target data rate can be acceptable for wearables (e.g. smart watch) compared with smart phones (1Mbps cell edge target data rate assumed in RAN1 studies).

Observation 3: The justification on UL coverage issue for RedCap UEs is not clear to the group.

Observation 4: For sensor and camera form factor, there is no 3dB antenna loss assumption.

Observation 5: The OTA test method for NR RedCap TRP measurement is under development in Rel-18 TRP TRS WI.

Proposal 1: Following RAN guidance, RAN4 should focus on the applicable form factor on sensor and camera.

Proposal 2: The restriction on the applicable form factor for PC2 Redcap should be reflected in the CR directly, or in the new WID.

# References

- R4-2306650	WF on RedCap HPUE.