# 3GPP TSG-RAN WG4 Meeting #107	R4-2308271

# Incheon, KR, May 22 – May 26, 2023

Source: 	vivo

Title: 	Further discussion on remaining Tx requirements for SL unlicensed operation for single CC

Agenda Item:	8.31.2.1.2

Document for:	Approval

Introduction

In the last meeting, we had very effective discussion on SL-U single carrier Tx requirements considering reusing NR-U/NR V2X requirements as much as possible. Most of the Tx requirements have been decided for SL-U single carrier operation in [1]. A TP [2] for SL-U Tx requirements has been endorsed. In this contribution, we continued the discussion on remaining Tx requirements for SL-U single carrier operation.

Discussion

In the last meeting, most of Tx requirements were concluded reusing NR-U/NR V2X requirements. For ON/OFF time mask for SL-U single carrier operation, we only agreed 10us ending transient period located in the guard period. From our understanding, the additional 5us is introduced for sensing whether the subband channel is idle to use, and it should be happening before the start of the transmission to allow for channel sensing. In this sense, we think the additional 5us should be before CP-E, and it can not be contained in the CPE. We think Option 2 is more aligned with our understanding, except for the on-off time mask for SL-U S-SSB. For now, the S-SSB is not concluded in RAN1, we are not sure whether the S-SSB structure is reused for SL-U. So we suggest to apply the general General ON/OFF time mask for S-SSB.

Issue 3-8-1: ON/OFF time mask for SL-U single CC

- Option 1: The time mask for NR SL-U needs to consider both the CP-extension and the guard period at the end of the slot. [LGE]

- Option 2: Adopt below figures ON/OFF time mask for SL-U with transient period same as NR-U in the front, and same as V2X in the end. [OPPO]

  - For the leading transient period, it is defined as 15us including 5us before CP-E and 10us within the CP-E (Same as NR-U, 5us transient period was defined in the front to allow UE ramp-up earlier).

  - For the ending transient period, it is proposed to keep the 10us within the guard period symbol instead of pushing inside the useful symbols for SL-U.

General ON/OFF time mask for SL-U PSSCH and PSCCH

ON/OFF time mask for SL-U S-SSB

- Option 3: To locate the 15us transient period inside the CPE. To locate the 10us transient period inside the last symbol (as guard symbol) of sidelink transmission. Adopt on/off time mask as below figure. [Xiaomi]

Proposal 1: The additional 5us should be before CP-E for SL-U ON-OFF time mask and consider the following mask for SL-U PSSCH, PSCCH, S-SSB.

General ON/OFF time mask for SL-U PSSCH, PSCCH, S-SSB

For the power control for SL-U single CC, we proposed the wording for absolute power control requirements for SL-U single CC. The absolute power tolerance requirements of clause 6.3.4.2 apply at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 40 ms.

Table 6.3.4.2-1: Absolute power tolerance

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 9.0 dB |

Issue 3-9-1: Absolute power control for SL-U single CC

- Option 1: Only absolute tolerance requirements are defined for NR SL-U. The duration of the transmission gap needs to be aligned with NR SL-U transmit scheme.

- Option 2: NR single CC requirements of Power control can be reused for SL-U considering there is no difference among NR V2X and NR-U.

Agreement：

Option 1 is agreed, further discuss Option 2 in next meeting.

Proposal 2: The absolute power tolerance requirements of table 6.3.4.2-1 apply at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 40 ms.

Table 6.3.4.2-1: Absolute power tolerance

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 9.0 dB |

Conclusion

This contribution discusses remaining Tx requirements for SL-U single carrier operation. The following observations and proposals are made:

Proposal 1: The additional 5us should be before CP-E for SL-U ON-OFF time mask and consider the following mask for SL-U PSSCH, PSCCH, S-SSB.

General ON/OFF time mask for SL-U PSSCH, PSCCH, S-SSB

Proposal 2: The absolute power tolerance requirements of table 6.3.4.2-1 apply at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 40 ms.

Table 6.3.4.2-1: Absolute power tolerance

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 9.0 dB |

- References

[1] R4-2306631, WF on SLU single CC Tx requirements, Xiaomi, RAN4#106bis-e.

[2] R4-2306634, TP to TR on the SL-e TX requirement, Xiaomi, RAN4#106bis-e.