3GPP TSG-RAN4 Meeting #107	R4-2308275

# Incheon, Korea, May 22 – May 26, 2023

Agenda item:			8.28.2.3

Source:	vivo

Title:	RF simulation results for transparent schemes

Document for:	Discussion

- Introduction

In RAN4#106bis-e meeting, some RF simulation parameters and simulation targets about power domain enhancements have reached an agreement [1], captured as follows:

<Way forward/Agreement>: - RAN4 agrees to analyse provided simulation results as attached to this WF via Excel and provide conclusions based on these at RAN4#107. - It is expected a decision on selecting the Rel-18 MPR/PAR reduction solution(s) according to the WID can be taken at RAN4#107 - RAN4 agrees that companies shall fill both the ‘OBO’ and ‘10% BLER SINR’ columns in the RAN4 simulation templates for MPR/PAR result collection. - Use the following equation as the baseline Net Gain dB=PoutFilter i-PoutRef+SNR10%Ref-SNR10%Filter i Where PoutFilter i is the achieved (TX) output power of the filtered waveform being compared against the reference, PoutRef is the output power of the reference, SNR10%Ref is the required SNR to achieve 10% BLER with the reference, and SNR10%Filter i is the required SNR to achieve 10% BLER using the filtered waveform being compared against the reference. - Use the following equation when providing the results for net gain Net Gain dB=-DOPR_M0 +SNR10%Ref-SNR10%Filter i - For evaluations, RAN4 should consider the spectrum extension factor (α) as: 0 (reference), 0.25, 0.111 (1/9), 0.125 and 0.375. - When expressing the net gain companies shall indicate the BS receiver type used. - RAN4 shall further study whether the ACLR of a PC2 UE can be applied to a PC3 UE when the output power is boosted to the same level of a PC2 UE.

We evaluate the performance of different filter-extension combinations and provide some simulation results for transparent schemes in this contribution, the corresponding results for non-transparent schemes are shown in [2].

- Simulation parameters

Simulation parameters used in these simulations are shown in Table 1.

Table 1. Simulation parameters for FDSS

| Carrier frequency | 4GHz |
| --- | --- |
| Channel BW | 20/100MHz |
| SCS | 15/30 kHz |
| Modulation | QPSK |
| Waveform | DFT-S-OFDM |
| Number of RBs | 16, 32, 64, 96, 128, 256 |
| Extension factors | 0, 0.125, 0.25, 0.375 |
| Channel | PUSCH, 14 OFDM symbols |
| Spectral shaping filter | - 3-tap (0.28 1 0.28) - 2-tap (1 0.28) - Truncated RRC (0.5, 0.1667) - No filter (reference case) |
| Power class | PC 3 |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 4 |
| Number of DMRS symbols | 2 |
| Number of PUSCH data symbols | 12 |
| Carrier Leakage, Image | 28dBc, 28dBc |
| BS receiver type | MRC receiver |

- Simulation results

Based on the LS from RAN1 [3], transparent schemes include Frequency domain spectrum shaping w/o spectrum extension, named as ‘FDSS w/o SE’in this contribution, and reference waveform(DFT-s-OFDM) without filter is named as ‘no filter’, as seen in Figure 1.

Figure 1. Different FDSS schemes

3.1 OBO performance

This section is the resubmission of the previous paper[4].

The physical implementation challenge is not considered in the simulation, the preliminary MPR simulation results are as follows. Three kinds of filters are used: 3tap([0.28 1 0.28]), 2tap([1 0.28]), TRRC(0.5,0.1667). The related simulation results are presented by parameter OBO.

- (b)

- (d)

Figure 2. OBO for 20 MHz channel & 15 kHz SCS,

with 16 PRB (a), 32 PRB (b), 64 PRB (c) and 96 PRB (d)

For the smaller RB allocations with FDSS w/o SE, the power boost taken by 3-tap filter is not very obvious, sometimes the output power is even lower than no filter case. However the performance of 3-tap filter becomes better with the increase of RB allocation. When the total number of RB reaches 96, it can be seen that the 3-tap filter achieves the largest power boost when RB allocation starts from some edge position of CBW. And TRRC filter provides the most stable power boost for FDSS w/o SE among different RB allocation numbers.

Observation1: Overall for FDSS w/o SE, the power improvement of TRRC filter is more stable than other filters.

Figure 2 shows that for FDSS w/o SE, only minor or even no power boost can be seen for small RB allocation, the boost is more obvious with the increasement of RB allocation. And the overall boost for FDSS w/o SE is no more than 1dB compared to legacy DFT-s-OFDM, less than that for FDSS w SE in [2].

Observation2: For FDSS w/o SE, only minor or even no power boost can be seen for small RB allocation, the boost is more obvious with the increasement of RB allocation.

- (b)

- (d)

Figure 3. OBO for 100 MHz channel & 30 kHz SCS,

with 32 PRB (a), 64 PRB (b), 96 PRB (c) and 256 PRB (d)

Figure 3 shows OBO performance for transparent schemes for 100MHz CBW and 30KHz SCS with 32, 64, 96 and 256 PRBs. The trend is very similar to the 20 MHz CBW case: Power bost is more obvious with the increasement of RB allocation number. There are no major differences in OBO performance between 20 MHz CBW and 100 MHz CBW cases.

Observation3: There are no obvious differences in OBO performance between 20 MHz CBW and 100 MHz CBW for transparent cases.

As shown in the Figure 2 and Figure 3, the ability of FDSS w/o SE to increase output power is relatively limited, the overall power boost for FDSS w/o SE is no more than 1dB compared to no filter case, less than that for FDSS w SE in [2].

Observation4: The ability of FDSS w/o SE to increase output power is relatively limited, and the overall power boost for FDSS w SE is no more than 1dB compared to legacy DFT-s-OFDM in both 20 MHz and 100 MHz channels.

3.2 Net gain

This section is the evaluation of some new simulation results of net gain.

Table 2. Net gain results of transparent cases(100M-30KHZ, FR1)

| Total PRBs | MCS (no filter/ transparent) | Spectrum shaping filter | 10% BLER SINR (trans) | OBO (trans) | RB start position | RB allocation, (inner/outer/edge/etc) | Net gain (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0 | [0.28 1 0.28] | -4.6 | / | 0 | outer | / |
|  |  |  |  | / | 132 | inner | / |
| 8 | 2 | [0.28 1 0.28] | -2.66 | / | 0 | outer | / |
|  |  |  |  | / | 132 | inner | / |
| 8 | 6 | [0.28 1 0.28] | 1.22 | / | 0 | outer | / |
|  |  |  |  | / | 132 | inner | / |
| 16 | 0 | [0.28 1 0.28] | -4.39 | 0.01 | 0 | outer | -1.87 |
|  |  |  |  | 0.13 | 128 | inner | -3.09 |
| 16 | 7 | [0.28 1 0.28] | 2.36 | -0.01 | 0 | outer | -2.46 |
|  |  |  |  | 0.11 | 128 | inner | -3.68 |
| 24 | 0 | [0.28 1 0.28] | -4.14 | -0.12 | 0 | outer | -2.02 |
|  |  |  |  | -0.01 | 124 | inner | -3.14 |
| 32 | 2 | [0.28 1 0.28] | -2.21 | 0 | 0 | outer | -2.28 |
|  |  |  |  | 0.5 | 120 | inner | -3.68 |
| 32 | 8 | [0.28 1 0.28] | 3.66 | 0 | 0 | outer | -2.84 |
|  |  |  |  | 0.39 | 120 | inner | -4.31 |
| 40 | 2 | [0.28 1 0.28] | -2.19 | 0 | 0 | outer | -2.21 |
|  |  |  |  | 0.14 | 116 | inner | -3.65 |
| 40 | 6 | [0.28 1 0.28] | 1.83 | -0.22 | 0 | outer | -2.23 |
|  |  |  |  | 0.01 | 116 | inner | -3.76 |

Table 3. Net gain results of transparent cases (20M-15KHZ, FR1)

| Total PRBs | MCS (no filter/ transparent) | Spectrum shaping filter | 10% BLER SINR (trans) | OBO (trans) | RB start position | RB allocation, (inner/outer/edge/etc) | Net gain (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 0 | [0.28 1 0.28] | -9.74 | / | 0 | outer | / |
|  |  |  |  | / | 132 | inner | / |
| 8 | 2 | [0.28 1 0.28] | -7.53 | / | 0 | outer | / |
|  |  |  |  | / | 132 | inner | / |
| 8 | 6 | [0.28 1 0.28] | -2.58 | / | 0 | outer | / |
|  |  |  |  | / | 132 | inner | / |
| 16 | 0 | [0.28 1 0.28] | -8.83 | 0.02 | 0 | outer | -0.78 |
|  |  |  |  | 0.13 | 128 | inner | -2.59 |
| 16 | 7 | [0.28 1 0.28] | -1.03 | -0.125 | 0 | outer | -1.485 |
|  |  |  |  | 0.125 | 128 | inner | -2.935 |
| 24 | 0 | [0.28 1 0.28] | -8.42 | / | 0 | outer | / |
|  |  |  |  | -0.25 | 124 | inner | -2.36 |
| 32 | 2 | [0.28 1 0.28] | -6.42 | -0.2 | 0 | outer | -1.11 |
|  |  |  |  | 0.2 | 120 | inner | -2.51 |
| 32 | 8 | [0.28 1 0.28] | -0.19 | -0.25 | 0 | outer | -1.16 |
|  |  |  |  | -0.25 | 120 | inner | -2.39 |
| 40 | 2 | [0.28 1 0.28] | -6.43 | -0.1 | 0 | outer | -1.12 |
|  |  |  |  | -0.1 | 116 | inner | -2.56 |
| 40 | 6 | [0.28 1 0.28] | -2.2 | -0.24 | 0 | outer | -1.24 |
|  |  |  |  | -0.24 | 116 | inner | -2.32 |

Net gain should be calculated from two parts of data from both baseline and FDSS, the complete data has been provided in the excel[5]. The FDSS transparent schemes seem to have no net gain in both outer and inner allocations based on our simulation results, for some cases with 3tap filter [0.28 1 0.28], the net gain is generally negative.

Observation5: The FDSS transparent schemes seem to have no net gain in both outer and inner allocations, for some cases with 3tap filter [0.28 1 0.28], the net gain is generally negative.

- Conclusion

Based on OBO performance:

Observation1: Overall for FDSS w/o SE, the power improvement of TRRC filter is more stable than other filters.

Observation2: For FDSS w/o SE, only minor or even no power boost can be seen for small RB allocation, the boost is more obvious with the increasement of RB allocation.

Observation3: There are no obvious differences in OBO performance between 20 MHz CBW and 100 MHz CBW for transparent cases.

Observation4: The ability of FDSS w/o SE to increase output power is relatively limited, and the overall power boost for FDSS w SE is no more than 1dB compared to legacy DFT-s-OFDM in both 20 MHz and 100 MHz channels.

Based on net gain:

Observation5: The FDSS transparent schemes seem to have no net gain in both outer and inner allocations, for some cases with 3tap filter [0.28 1 0.28], the net gain is generally negative.

- References

- R4-2306627 WF on remaining open issues for MPRPAR reduction for coverage enhancement, #RAN4 106bis-e.

- R4-2308276 RF simulation results for non-transparent schemes, #RAN4 107.

- R4-2300007 LS to RAN4 for further information on RAN1 assumptions for LLS performance evaluation of MPR/PAR reduction solutions, RAN1, #RAN4 106.

- R4-2305116 RF simulation results for transparent schemes for enhancement to reduce MPR, #RAN4 106bis-e.

- R4-2308110 Collected Simulation Results for MPR/PAR evaluations, #RAN4 107.