3GPP TSG-RAN4 Meeting #107	R4-2308279

# Incheon, Korea, May 22th – May. 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7211 | rev | - | Current version: | 15.19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to inter-RAT NR measurement requirements_R15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2023-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When the per-FR gap was introduced in Rel-15, the relationship between "UE supporting per-FR gap" and "UE configured with per-FR gap" was widely discussed. And the following principles are agreed in RAN4#87 (see R4-1807997 for more backgrounds): - per-FR gap can only be configured to UE supporting per-FR gap. i.e. “UE configured with per-FR gap” is a subset of "UE supporting per-FR gap". - When per-UE gap is configured to a per-FR gap capable UE, UE still shall fulfil per-UE measurement requirements. - When per-FR gap is configured to a per-FR gap capable UE ,UE shall perform gap-based measurement fulfiling per-FR measurement requirements. - Specificly, if UE has no serving cell in a particular FR, UE shall perform measurement in this FR gap-lessly regardless if explicit per-FR measurement gap is configured in this FR. And a effective MGRP of 40ms/20ms (FR1/FR2) is used to determine measurement requirements. This principle is already applied to gap based measurement requirements for EN-DC/NR SA/NR-DC/NE-DC in 38.133, and it is also applied to inter-RAT NR measurement requirements under LTE SA (36.133 clause 8.1.2.4.21). We have: 36.133 cl.8.1.2.4.21 For per-FR measurement gap capable UE, when serving cells are in E-UTRA and measurement objects are only in FR2, - UE can perform such measurements without gap, and - UE fulfils the requirements for FR2 measurement objects based on effective MGRP = 20 ms.. However, there exists several issues to be clarified: - Per-FR gap is a NR UE capability rather than LTE UE capability. So saying a LTE UE “per-FR gap capable” causes confusion. A reference to 38.306 is needed. - 36.133 clause 8.1.2.4.21 is the requirements under LTE SA, so clearly all serving cells are LTE cells. The wording “serving cells are in E-UTRA” is redundant. - According to 36.331, NW must config a MO for LTE PCell as long as it provides MeasConfig to UE. So it’s impossible to have “measurement objects are only in FR2”. the intention here obviously should be “UE is configured with FR2 NR measurement object”. 36.331 cl.5.5.2.1 #### 5.5.2.1 General E-UTRAN applies the procedure as follows: - to ensure that, whenever the UE has a measConfig, it includes a measObject for each LTE serving frequency;. - Last but not least, the LTE PCell can only configure per-UE gap under LTE SA as specified in 36.331. 36.331 cl.6.3.2 fr1-Gap Indicates whether the gap is only applicable for measurements on FR1. E-UTRAN sets this field to TRUE only when the UE is configured with (NG)EN-DC.. which means if a gap pattern is configured by LTE PCell, the UE shall always perform FR2 NR measurements and determine requirements based on the configured gap pattern. A per-FR gap capable UE only perform FR2 NR measurements gaplessly under LTE SA when LTE PCell doesn’t config any measurement gap. Assosicated CRs are also submitted to update LTE-FR2 NR measurement TCs A.8.4.2.5/6/7/8 and A.18.3.1.5/6/7/8 respectively. |
| Summary of change: |  | Inter-RAT NR measurement requirements under LTE SA are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.4.21.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.1.2.4.21.1.1	Identification of a new NR cell

When measurement gaps are scheduled, the UE shall be able to identify a new detectable cell within Tidentify_irat_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise, UE shall be able to identify a new detectable inter-RAT cell within Tidentify_irat_with_index. The UE shall be able to identify a new detectable inter-RAT SS block of an already detected cell within Tidentify_irat_without_index.

Tidentify_irat_without_index = (TPSS/SSS_sync_irat + T SSB_measurement_period_irat) ms

Tidentify_irat_with_index = (TPSS/SSS_sync_irat + T SSB_measurement_period_irat + TSSB_time_index_irat) ms

Where:

TPSS/SSS_sync_irat: it is the time period used in PSS/SSS detection given in table 8.1.2.4.21.1.1-1 and table 8.1.2.4.21.1.1-2.

TSSB_time_index_irat: it is the time period used to acquire the index of the SSB being measured given in table 8.1.2.4.21.1.1-3 and table 8.1.2.4.21.1.1-4.

TSSB_measurement_period_irat: equal to a measurement period of SSB based measurement given in table 8.1.2.4.21.1.1-5 and table 8.1.2.4.21.1.1-6.

Mpss/sss_sync_irat: For a UE supporting FR2 power class 1, Mpss/sss_sync_irat = 64 samples. For a UE supporting FR2 power class 2 (vehicle mounted), Mpss/sss_sync_irat = 40 samples. For a UE supporting FR2 power class 3 (handheld), Mpss/sss_sync_irat = 40 samples. For a UE supporting FR2 power class 4, Mpss/sss_sync_irat = 40 samples.

MSSB_index_irat: For a UE supporting FR2 power class 1, MSSB_index_irat = 40 samples. For a UE supporting FR2 power class 2 (vehicle mounted), MSSB_index_irat = 24 samples. For a UE supporting FR2 power class 3 (handheld), MSSB_index_irat = 24 samples. For a UE supporting FR2 power class 4, MSSB_index_irat = 24 samples.

Mmeas_period_irat: For a UE supporting FR2 power class 1, Mmeas_period_irat = 64 samples. For a UE supporting FR2 power class 2 (vehicle mounted), Mmeas_period_irat = 40 samples. For a UE supporting FR2 power class 3 (handheld), Mmeas_period_irat = 40 samples. For a UE supporting FR2 power class 4, Mmeas_period_irat = 40 samples.

Nfreq is defined in clause 8.1.2.1.1

For per-FR measurement gap specified in 38.306 [49] capable UE, when FR2 NR measurement object(s) is configured,

-	if measurement gap is configured, UE fulfils the per-UE measurement requirements for FR2 NR measurement object(s) based on the configured measurement gap pattern. Nfreq is counted on a per-UE basis.

-	if measurement gap is not configured, UE performs measurements on FR2 NR measurement object(s) without gap and

fulfils the requirements for FR2 measurement object(s) based on effective MGRP = 20 ms. Nfreq is counted on a per-FR basis.

Table 8.1.2.4.21.1.1-1: Time period for PSS/SSS detection (Frequency range FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_irat |
| --- | --- |
| No DRX | Max(600ms, 8 Max(MGRP, SMTC period)) Nfreq |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(81.5) Max(MGRP, SMTC period, DRX cycle)) Nfreq |
| DRX cycle > 320ms | 8 DRX cycle Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. |  |

Table 8.1.2.4.21.1.1-2: Time period for PSS/SSS detection (Frequency range FR2)

| Condition NOTE1,2 | TPSS/SSS_sync_irat |
| --- | --- |
| No DRX | Max(600ms, Mpss/sss_sync_irat Max(MGRP, SMTC period)) Nfreq |
| DRX cycle ≤ 320ms | Max(600ms, (1.5 Mpss/sss_sync_irat) Max(MGRP, SMTC period, DRX cycle)) Nfreq |
| DRX cycle > 320ms | Mpss/sss_sync_irat DRX cycle Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 5. |  |

Table 8.1.2.4.21.1.1-3: Time period for time index detection (Frequency range FR1)

| Condition NOTE1,2 | TSSB_time_index_irat |
| --- | --- |
| No DRX | Max(120ms, 3 Max(MGRP, SMTC period)) Nfreq |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(3 1.5) Max(MGRP, SMTC period, DRX cycle)) Nfreq |
| DRX cycle > 320ms | 3 DRX cycle Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 5. |  |

Table 8.1.2.4.21.1.1-4: Time period for time index detection (Frequency range FR2)

| Condition NOTE1,2 | TSSB_time_index_irat |
| --- | --- |
| No DRX | Max(200ms, MSSB_index_irat Max(MGRP, SMTC period)) Nfreq |
| DRX cycle ≤ 320ms | Max(200ms, (1.5 MSSB_index_irat) Max(MGRP, SMTC period, DRX cycle)) Nfreq |
| DRX cycle > 320ms | MSSB_index_irat DRX cycle Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 5. |  |

In the requirements, an NR cell is considered detectable when:

-	NR SS-RSRP related conditions in the accuracy requirements in Section 9.11.1 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 of TS 38.133 [50],

-	NR SS-RSRQ related conditions in the accuracy requirements in Section 9.11.2 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 of TS 38.133 [50],

-	NR SS-SINR related conditions in the accuracy requirements in Section 9.11.3 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 of TS 38.133 [50].

When measurement gaps are scheduled for NR measurements the UE physical layer shall be capable of reporting NR SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in clause 9.11, with measurement period as shown in table 8.1.2.4.21.1.1-5 and 8.1.2.4.21.1.1-6:

Table 8.1.2.4.21.1.1-5: Measurement period for inter-RAT measurements (Frequency range FR1)

| Condition NOTE1,2 | TSSB_measurement_period_irat |
| --- | --- |
| No DRX | Max(200ms, 8 Max(MGRP, SMTC period)) Nfreq |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle)) Nfreq |
| DRX cycle > 320ms | 8 DRX cycle Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 5 |  |

Table 8.1.2.4.21.1.1-6: Measurement period for inter-RAT measurements (Frequency range FR2)

| Condition NOTE1,2 | TSSB_measurement_period_irat |
| --- | --- |
| No DRX | Max(400ms, Mmeas_period_irat Max(MGRP, SMTC period)) Nfreq |
| DRX cycle ≤ 320ms | Max(400ms, (1.5 Mmeas_period_irat) Max(MGRP, SMTC period, DRX cycle)) Nfreq |
| DRX cycle > 320ms | Mmeas_period_irat DRX cycle Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 5. |  |

The UE shall be capable of performing NR SS-RSRP, SS-RSRQ and SS-SINR for up to 7 NR carrier frequencies.

For each RAT E-UTRAN FDD-NR layer on FR1 or FR2, the UE shall be capable of monitoring at least 4 cells.

For each RAT E-UTRAN FDD-NR layer on FR1, during each layer 1 measurement period, the UE shall be capable of monitoring at least 7 SSBs with different SSB index and/or PCI on the RAT E-UTRAN FDD-NR layer.

For each RAT E-UTRAN FDD-NR layer on FR2, during each layer 1 measurement period, the UE shall be capable of monitoring at least 10 SSBs with different SSB index and/or PCI on the RAT E-UTRAN FDD-NR layer. The UE shall be capable of monitoring at least one SSB per cell.

The NR SS-RSRP measurement accuracy for all measured NR cells shall be as specified in clause 9.11.1. The NR SS-RSRQ measurement accuracy for all measured NR cells shall be as specified in clause 9.11.2. The NR SS-SINR measurement accuracy for all measured NR cells shall be as specified in clause 9.11.3.

NOTE: When inter-frequency RSTD measurements are configured and the UE requires measurement gaps for performing such measurements, gap pattern 0 is assumed and requirements in this clause are derived assuming MGRP=80ms is used.

<End of modified section 1>