GPP TSG-RAN WG4 Meeting # 107	R4-2308311

# Incheon, KR, May 22 – May 26, 2023

Agenda item:	9.7.5

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM requirements maintenance for IoT NTN

Document for:	Discussion

# Introduction

The Rel-18 RAN4 WID was approved in [1] to define requirements for Rel-17 IoT NTN [2]. In Rel-17 IoT NTN, RAN1/2/3 specifications were defined to enable NB-IoT and eMTC operating over NTN. The core requirements have been completed with some left issues to be discussed in maintenance stage [3]. In this contribution, we further provide our views on the remaining issues.

# Discussion

## 2.1 measurement relaxation for NGSO

For measurement relaxation for both serving cell and neighbor cell, the status are summarized as follows:

#### Issue 2-2: NGSO, serving cell measurement - relaxation factor N - Option 1: The values in tables used for deriving the serving cell measurement relaxation factors are reduced by factor 2 for NGSO for eMTC and NB-IoT in IDLE mode. (Ericsson) - Option 2: Reuse the legacy mechanism of serving cell measurement relaxation factor N. (CMCC, MTK) #### Issue 3-4: NGSO, serving cell measurement upon t-Service-r17 - Option 1: - If the UE is configured with DRX cycle ≥ [1.28] sec, then the UE shall meet the serving cell evaluation requirements defined for DRX cycle of [640] ms starting from at least 2 configured DRX cycles before ‘t-Service-r17’. - If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_IDLE, then the UE shall meet the requirements defined for DRX cycle length of [2.56] s in Table 4.7A.2.1.1-1 starting from at least [2] DRX cycles before ‘t-Service-r17’. - Option 2: Postpone the discussion on enhancements of serving cell measurements to the Rel-18 WI (IoT_NTN_enh).

The problem of current conditions for cell measurement relaxation for serving cell and neighbor cell will lead to very poor performance as UE may start neighbor cell measurement and evaluate serving cell for cell reselection long after t-Service. We illustrate the case in following Fig. 1.

Fig. 1 Serving cell and neighbor cell measurement relaxation for IoT NTN.

Due to the nature of NTN, the change of RSRP of NGSO may not be significant. It will lead to the result that: UE may easily enter relaxation mode, but it is hard to exit relaxation mode even the cell is going to stop serving the UE. As shown in the upper part of the figure, UE will most likely enter relaxation at point A (N*DRX for serving and no measurement for neighbour cell). Based on the reason explained above, even approaching or after t-servie, UE is still in relaxation mode. UE may realize that relaxed monitoring criteria is not met at point C. Only from this point, UE starts measure serving cell in normal manner and start neighbour cell measurement.  It could be observed that it will introduce unnecessary delay for cell re-selection.

Observation 1: Legacy relaxation criteria will lead to very long delay for cell re-selection after t-service.

Thus, we propose to have additional conditions that UE shall exist cell relaxation before t-service for both serving cell and neighbor cell as shown in the lower part of Fig. 1. For serving cell evaluation, UE shall exist relaxation mode to evaluate serving cell in normal manner (e.g. every DRX) before t-Service. For neighbour cell measurement, companies point out that it is already stated in the spec that UE shall perform neighbour cell measurement before t-service regardless of the rules currently limiting the UE measurement behaviour, which is shown as follows:

If all the relaxed monitoring criteria defined in clause 5.2.4.12 [1] are fulfilled then the UE’s intra-frequency measurement is not required to meet Tdetect,NB_Intra_NC, Tmeasure,NB_Intra_NC and Tevaluate,NB_intra_NC as defined in Table 4.6A.2.2-1 and Table 4.6A.2.2-2. If t-Service is broadcasted and applicable, UE shall be able to detect, measure, and evaluate neighbour cells before t-Service is reached, regardless of the rules currently limiting the UE measurement activities, and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation. This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of t-Service is started to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger.

We tend to agree with the observation that it serve the same purpose that UE shall not stop neighbour cell measurement as defined in TS 36.304, which is shown as follows:

However, as shown above, the description about Ttrigger is directly copied from NR NTN requirements without much discussion for IoT NTN. Based on current requirements, the two highlighted parts are not very clear which one can overwrite the other one. We think it can be clarified in Rel-18 IoT NTN enh.

Observation 2: In current requirements for neighbour cell measurement, it needs to be clarified that whether it requires UE to exit relaxed neighbour cell measurement Ttrigger before t-service.

Based on observation 2, we can assume that UE is already doing neighbour cell measurement before t-service, then there is no need for relaxed serving cell measurement. Companies commented that there is still power saving gain for serving cell measurement relaxation. However, since UE is already doing neighbour cell measurement, when UE is required to wake up at least every DRX, we couldn't see the sense to allow serving cell relaxation anymore.

Observation 3: If UE is already performing neighbour cell measure normally before t-service, it does not make much sense for serving cell measurement relaxation for power saving purpose since UE is already wake up for measurement for each DRX.

Thus, we propose to add following conditions for serving cell relaxation for IoT NTN:

- The time span to T-service is longer than k, where k = 4 DRX cycle if not configured with eDRX_IDLE cycle, and k = 1 eDRX cycle if configured with eDRX_IDLE cycle.

Proposal 1:

In addition to the existing serving cell relaxation conditions, add following conditions for IoT NTN:

- The time span to T-service is longer than k, where k = 4 DRX cycle if not configured with eDRX_IDLE cycle, and k = 1 eDRX cycle if configured with eDRX_IDLE cycle.

## 2.2 Requirements for Discontinuous coverage

There are remaining issues related to discontinuous coverage, which are shown as follows:

#### Issue 3-1: Discontinuous Coverage - clarification - Option 1: RAN4 to agree that in discontinuous coverage scenarios, the UE shall resume measurements when the UE is in expected coverage as defined by SystemInformationBlockType32 - Option 1a: RAN4 to agree that in discontinuous coverage scenarios, the UE shall resume measurements when the UE is in expected in-coverage after UE acquired the SystemInformationBlockType32. #### Issue 3-2: Discontinuous Coverage - wording alignment to align with the definitions in TS36.304 and the design of t-service, in TS 36.133, Section 4.6A, revise the wording “If SystemInformationBlockType32 [2] has been received and if the UE has determined that it is out of coverage, the UE is not required to perform cell measurements from the last slot of SI transmission which indicates that UE will be in out of coverage after t-Service [2] when the serving cell stop serving the area.” as follows: - “If SystemInformationBlockType32 [2] has been received and the UE has determined it will be out of coverage after t-service, the UE is not required to perform neighbor cell measurements during the last one SI modification periods, before t-Service [2]” #### Issue 3-3: Discontinuous Coverage - revision on paging requirement - Option 1: Do not create additional requirements after t-serviceStart regarding UE paging interruption. - Option 2: Paging requirements apply when UE is in coverage, which is the time when UE has acquired the SI of the target cell and the time is after t-serviceStart-r17 if the UE supports discontinuous coverage and SystemInformationBlockType32 has been received in quasi-Earth fixed scenario.

As commented by many companies, basically the UE behaviour once in coverage is like cell selection procedure. In RAN4 specification, we never define requirements for cell selection procedure. Thus, similarly, it is preferred not to define requirements for discontinuous coverage upon UE is in coverage, which is up to UE implementation.

Proposal 2: RAN4 not to define requirements for discontinuous coverage upon UE is in coverage, which is similar as Cell selection procedure.

# Conclusions

Observation 1: Legacy relaxation criteria will lead to very long delay for cell re-selection after t-service.

Observation 2: In current requirements for neighbour cell measurement, it needs to be clarified that whether it requires UE to exit relaxed neighbour cell measurement Ttrigger before t-service.

Observation 3: If UE is already performing neighbour cell measure normally before t-service, it does not make much sense for serving cell measurement relaxation for power saving purpose since UE is already wake up for measurement for each DRX.

Proposal 1:

In addition to the existing serving cell relaxation conditions, add following conditions for IoT NTN:

- The time span to T-service is longer than k, where k = 4 DRX cycle if not configured with eDRX_IDLE cycle, and k = 1 eDRX cycle if configured with eDRX_IDLE cycle.

Proposal 2: RAN4 not to define requirements for discontinuous coverage upon UE is in coverage, which is similar as Cell selection procedure.

# References

[1] RP-221556 New WID: WID on NB-IoT/eMTC core & performance requirements for Non-Terrestrial Networks

[2] RP-211601 NB-IoT/eMTC support for Non-Terrestrial Networks

[3] R4-2306370 WF on NB-IoT/eMTC RRM requirements for NTN, MediaTek inc