GPP TSG-RAN WG4 Meeting # 107	R4-2308315

# Incheon, KR, May 22 – May 26, 2023

Agenda item:	9.8.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM requirements for IoT NTN enhancement

Document for:	Discussion

# Introduction

The Rel-18 WID IoT NTN enhancements was approved in [1] to improve the mobility/throuput/GNSS operation performance compared with Rel-17 IoT NTN. The RRM impacts were initially discussed in last meeting with agreements captured in [2]. In this contribution, we further provide our views on remaining issues.

# Discussion

## 2.1 Requirements which not applicable due to lack of neighbour cell ephemeris

The requirements involving neighbour cell measurement are not applicable due to lack of neighbour cell ephemeris in Rel-17 IoT NTN, and it is agreed to introduce the requirements in Rel-18 and take the requirements defined during R17 can be the starting point for further discussion. The latest status of RAN2 regarding the neighbor cell assistant information is shown as follows:

Agreements:

- New SIBxx is introduced to broadcast the neighbor cell/satellite information.

Agreements via email – from offline 114:

- Common TA parameters are broadcast as assistance information for neighbor cell measurements.

- Kmac is broadcast as neighbor cell assistance information.

- For moving cell, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (like in NR-NTN)

Based on the discussion in Rel-17 IoT NTN, companies have different views on when UE is allowed to perform relaxed neighbour cell measurement (not perform intra-frequency and inter-frequency measurement). If only consider the relaxed neighbour cell measurement criteria defined in TS 36.304, UE can stop measurement. We have raised the concern in Rel-17 discussion that the change of NRSRP of serving cell maybe not significant and UE may stop neighbour cell measurement the serving cell is going to stop serving the area.

Based on the discussion in last meeting, companies pointed out that there is already statement which is similar as NR NTN that UE shall be able to detect, measure, and evaluate neighbour cells before t-Service is reached, which is shown as follow:

If all the relaxed monitoring criteria defined in clause 5.2.4.12 [1] are fulfilled then the UE’s intra-frequency measurement is not required to meet Tdetect,NB_Intra_NC, Tmeasure,NB_Intra_NC and Tevaluate,NB_intra_NC as defined in Table 4.6A.2.2-1 and Table 4.6A.2.2-2. If t-Service is broadcasted and applicable, UE shall be able to detect, measure, and evaluate neighbour cells before t-Service is reached, regardless of the rules currently limiting the UE measurement activities, and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation. This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of t-Service is started to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger. Ttrigger = max(Tdetect,NB_Intra_NC , Pcarrier * Tdetect,NB_Inter_NC), where - Pcarrier is the number of inter-frequency carriers for which carrier frequency information was provided by the serving NB-IoT cell, - Tdetect,NB_Intra_NC is defined in [4.6A.2.2], and Tdetect,NB_Inter_NC is defined in [4.6A.2.5].

We tend to agree with the observation though the corresponding requirements are directly reused from NR NTN. However, current requirements also contain this part, and it is not clear which one can overwrite the other one. Thus, it is suggested to be clarified as follows where UE shall not continue neighbour cell relaxation when The time span to T-service is shorter than Ttrigger.

If all the relaxed monitoring criteria defined in clause 5.2.4.12 [1] are fulfilled then the UE’s intra-frequency measurement is not required to meet Tdetect,NB_Intra_NC, Tmeasure,NB_Intra_NC and Tevaluate,NB_intra_NC as defined in Table 4.6A.2.2-1 and Table 4.6A.2.2-2. If t-Service is broadcasted and applicable, UE shall be able to detect, measure, and evaluate neighbour cells before t-Service is reached, regardless of the rules currently limiting the UE measurement activities, and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation. This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of t-Service is started to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger. Ttrigger = max(Tdetect,NB_Intra_NC , Pcarrier * Tdetect,NB_Inter_NC), where - Pcarrier is the number of inter-frequency carriers for which carrier frequency information was provided by the serving NB-IoT cell, - Tdetect,NB_Intra_NC is defined in [4.6A.2.2], and Tdetect,NB_Inter_NC is defined in [4.6A.2.5]. If all the relaxed monitoring criteria defined in clause 5.2.4.12 [1] are fulfilled and the time span to before t-Servic is longer than Ttrigger then the UE’s intra-frequency measurement is not required to meet Tdetect,NB_Intra_NC, Tmeasure,NB_Intra_NC and Tevaluate,NB_intra_NC as defined in Table 4.6A.2.2-1 and Table 4.6A.2.2-2.

Proposal 1: Clarify that UE shall be able to detect, measure, and evaluate neighbour cells before t-Service is reached, and the relaxed neighbour cell measurement is only allowed when the relaxed monitoring criteria defined in clause 5.2.4.12 [1] are fulfilled and the time span to before t-Servic is longer than Ttrigger

## 2.2 Neighbor cell measurement in connected mode for NB-IoT

For the objective related to mobility enhancement, the descriptions are summarized as follows:

The following mobility enhancements objectives are listed. - Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2] - Support signalling in system information of neighbour cell ephemeris, for eMTC and NB-IoT [RAN2] - Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2] - Define UE RRM core requirements for the above mobility enhancement features [RAN4].

Based on the discussion in last meeting, the status is summarized as follows:

#### Issue 3-1: Neighbour cell measurements in connected mode, time-based - For quasi-earth fixed cell for NB-IoT, RAN4 to discuss whether to define requirements for t-service triggered neighbour cell measurement before RLF. FFS the following options: - Option 1: The NB-IoT UE shall start the intra/inter-frequency measurements at least before time T1 before start of t-Service, where T1 is the time required to perform one measurement. - Option 2: RAN4 to define time-based connected mode measurement initiation condition in core spec. The details such as exact measurement starting time should wait further progress of RAN2. - Option 3: not to define. - Other options are not precluded. #### Issue 3-2: Neighbour cell measurements in connected mode, location-based - For quasi-earth fixed cell and earth-moving cell for NB-IoT, RAN4 to define location-based connected mode measurement initiation condition in core spec. The details such as exact measurement starting time should wait further progress of RAN2

For Rel-18 IoT NTN enhancement, the related status in RAN2 are summarized as follows:

Agreements:

- For NB-IoT we support a trigger for neighbour cell measurements based on T-service (in the quasi-Earth fixed case) (this does not preclude anything for eMTC discussion)

Agreements:

- At least for NB-IoT NTN, for quasi-earth fixed cells, UE shall start intra/inter frequency measurement in connected mode before the t-Service if present. The exact time to start measurements in connected mode before t-Service can be left to UE implementation” (can revisit if we agree other proposal based on neighbour cell coverage)

- RAN2 will not specify the condition of stopping UE measurement before t-Service

- For earth-moving cell, the UE derives when loss of coverage of current cell happens (how to derive this information is FFS)

- For earth-moving cell, UE shall start intra/inter frequency measurements in RRC connected mode before losing coverage. The exact time to start measurements can be left to UE implementation

Agreements:

- Location-based connected mode measurement initiation is supported in quasi-Earth-fixed cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for quasi-Earth-fixed cell. FFS on whether the R17 IEs are reused or not. FFS if the same mechanism can also be used in idle (like in NR-NTN)

- Location-based connected mode measurement initiation is supported in earth-moving cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for earth-moving cell. FFS on whether the R17 IEs are reused or not. FFS on whether additional information needs to be broadcast to inform the UE how the reference location moves over time or if this can be derived from other information (e.g. Epoch time and ephemeris). FFS if the same mechanism can also be used in idle (like in NR-NTN)

It could be observed that RAN2 is discussing new triggering for neighbour cell measurement before RLF. As mentioned above, in Rel-17 NB-IoT, the measurement is triggered by NRSRP level of the serving cell. However, in NTN, the NRSRP change is less significant, thus news triggering method is to be considered. Based on RAN2’s discussion, it can be further divided into following two cases:

- Quasi-earth fixed cell:

  - RAN2 is discussing triggering the neighbour cell measurement by t-Service.

  - RAN2 is discussing location-based measurement initiation.

- Earth-moving cell:

  - RAN2 is discussing location-based measurement initiation.

Based on the analysis above, the triggering method for quasi-earth fixed cell and earth-moving cell can be summarized as follows:

- Quasi-earth fixed cell:

  - Legacy NRSRP based triggering

  - T-service based triggering (the exact time to start measurements is up to UE implementation)

  - Location based triggering: reference location + distance threshold

- Earth-moving cell:

  - Legacy NRSRP based triggering

  - Location based triggering: reference location + distance threshold

Observation 1: The triggering methods for neighbour cell measurement in connected mode before RLF are as follows:

- Quasi-earth fixed cell:

  - Legacy NRSRP based triggering

  - T-service based triggering (the exact time to start measurements is up to UE implementation)

  - Location based triggering: reference location + distance threshold

- Earth-moving cell:

  - Legacy NRSRP based triggering

  - Location based triggering: reference location + distance threshold

For Quasi-earth fixed cell, the t-Service triggered measurement is different from what is defined in Rel-17 NR NTN. In NR NTN, t-Service triggered measurement are only used in IDLE mode. In connected mode, it is up to NW configuration to trigger the measurement and determine the frequency carriers which is same as TN. For NTN IoT, now the target scenario is neighbour cell measurement in connected mode, and the exact delay of neighbour cell measurement is not deterministic since it is kind of “best effort” measurement which can only utilized the vacant time slot.

Observation 2: The inter-frequency measurement in connected mode before RLF is “best efforts” measurement using vacant time slot, where the exact time for inter-frequency neighbour cell detection and measurement is unpredictable.

In NR NTN, RAN4 had defined Ttrigger which works as the condition that UE shall be able to detect, measure, and evaluate neighbour cells before t-service when the time span from the time when t-Service is acquired by the UE to the time when the cell is stop serving the cell is longer than t-service. However, for the reason mentioned about, it is hard to define the “Ttrigger” for inter-frequency measurement. Thus, for t-service trigger measurement, RAN4 should discuss whether to define requirements since the exact start time for measurement is up to UE implementation.

Based on the discussion in last meeting, companies have different views on whether to define requirements for time-based neighbour cell measurement. Based on the analysis above, following options could be considered:

Option 1: Do not define requirements for t-service triggered neighbour cell measurement.

Option 2: The UE shall be able to measure only intra-frequency neighbour cell before t-service provided that the time span from the SI broadcasting t-service to t-service is longer than Tidentify_intra, and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation.

Proposal 2: For for t-service triggered neighbor cell measurement before RLF for NB-IoT:

Option 1: Do not define requirements for t-service triggered neighbour cell measurement.

Option 2: The UE shall be able to measure only intra-frequency neighbour cell before t-service provided that the time span from the SI broadcasting t-service to t-service is longer than Tidentify_intra, and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation.

For location-based triggering, the case is simpler. In existing requirements for TN, UE shall be able to identify a new detectable neighbour cell when the NRSRP based criteria is met as defined in 36.331 which is shown as below. Then, for location-based triggering, it is straightforward to add the criteria for location based triggered and the same requirements as NRSRP based triggering can apply.

The latest RAN2 status are shown as follows:

Agreements:

- Location-based connected mode measurement initiation is supported in quasi-Earth-fixed cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for quasi-Earth-fixed cell. FFS on whether the R17 IEs are reused or not. FFS if the same mechanism can also be used in idle (like in NR-NTN)

- Location-based connected mode measurement initiation is supported in earth-moving cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for earth-moving cell. FFS on whether the R17 IEs are reused or not. FFS on whether additional information needs to be broadcast to inform the UE how the reference location moves over time or if this can be derived from other information (e.g. Epoch time and ephemeris). FFS if the same mechanism can also be used in idle (like in NR-NTN)

Proposal 3: For location-based triggering, add the corresponding criteria (reference location and distance threshold) and the same requirements as NRSRP based triggering can apply, and the details should wait for more progress in RAN2.

## 2.2 CHO for eMTC

Issue 4: For eMTC, CHO requirements - Define RRM requirements for time-based and location-based CHO for both normal and enhanced coverage for eMTC. For the specific delay requirement development, wait further progress of RAN2. - Note: the enhanced coverage is legacy function

RAN2 has not made further progress on eMTC CHO during April meeting. CHO requirements for Rel-17 NR NTN are defined based on the principle that time or location-based condition is always configured together with RSRP based condition. However, in RAN2 February meeting there was no consensus whether this principle still holds for IoT NTN or not. In this situation, we believe RAN4 should further wait for RAN2 progress before discussing the exact CHO requirements.

Proposal 4: RAN4 should further wait for RAN2 progress before discussing exact requirements for time and [location based] CHO for eMTC.

## 2.4 Location-based cell reselection

RAN2 agreed the following for location triggered cell reselection measurements.

- For moving cell, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (like in NR-NTN)

- For eMTC NTN, for fixed cell, location-based measurement initiation can also be used in RRC_IDLE for cell re-selection purposes (like in NR-NTN)

- For eMTC NTN, for moving cell, location-based measurement initiation can also be used in RRC_IDLE for cell re-selection purposes (like in NR-NTN). FFS whether to consider solution that does not require UE to update the GNSS for this same as in connected mode

In NR NTN, for fixed cell the following are defined for location triggered cell reselection measurement. For moving cell, the requirements are under discussion in Rel-18 NR NTN WI, but likely it would be similar to fixed cell.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, or the distance between UE and serving cell reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. The requirements apply provided that the distance exceeds the distanceThresh by a margin of 50 m. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

For eMTC and fixed cell, it is straightforward to re-use the requirements from NR NTN because the mechanism is same. For eMTC and moving cell, we suggest to wait for further progress in Rel-18 NR NTN WI. Whether the requirements are also applicable to NB-IoT depends on further RAN2 agreement.

Proposal 5: For location triggered cell reselection measurement,

- For eMTC and fixed cell, re-use the requirements from NR NTN

- For eMTC and moving cell, wait for further progress in Rel-18 NR NTN WI

- FFS whether the requirements are also applicable to NB-IoT pending on further RAN2 agreement

## 2.3 GNSS re-acquisition

The status of GNSS re-acquisition of RRM impacts is shown as follows.

#### Issue 5: GNSS re-acquisition - RAN4 to postpone the discussions on GNSS re-acquisition until more progress has been reached on this objective in other WGs.

Based on RAN1 discussion, there are potentially two method for UE to re-acquire GNSS. Alternative one is that NW will configure a gap for UE explicitly for GNSS re-acquisition. The other alternative is UE to acquire GNSS autonomously.

For NW triggered Gap, based on latest RAN1 progress which is shown below, NW will configure a gap according to the reported GNSS fix time duration. And it is triggered aperiodically via MAC CE. From our understanding, there is not much RAN4 RRM impacts since the triggering method/gap length will be clearly defined by RAN1/2. The potential impact could be the gap pattern to be captured in RAN4 spec if needed.

Agreement For the GNSS measurement gap aperiodically triggered with MAC CE, the duration for the GNSS measurement gap can be configured by eNB. - The gap duration is equal to the latest reported GNSS position fix time duration for measurement when the duration for GNSS measurement gap is not included in the configuration by eNB. Conclusion From RAN1 perspective, UE is not forbidden to autonomously re-acquire GNSS position fix during inactive state of Connected DRX. - Note: The configured DL/UL transmissions during inactive state of Connected DRX should not be impacted - Note: details are up to RAN2 Send an LS to RAN2 for the conclusion. Agreement Draft LS in R1-2304125 is endorsed. Final LS in R1-2304126. Agreement On when the aperiodic GNSS measurement gap starts, which is aperiodically triggered by eNB with MAC CE, the start time should be at n+ X, where n is the end of MAC CE receiving subframe/slot - FFS: details of X, e.g. predefined value or configured value, considering HARQ feedback for the MAC CE, etc Agreement UE reports one GNSS position fix time duration for GNSS measurement via a N-bit field at least including [1,2] seconds as component values. - FFS: value of N, other component value(s) of GNSS position fix time duration (e.g. N=3, with value in [3,7,13,19,25, X] seconds, and X is FFS). - FFS: whether RAN4 input is needed.

Regarding the impacts on other RRM requirements, from our understanding, it may have impacts on connected mode measurement. For instance, for inter-frequency neighbor cell measurement for NB-IoT, the measurement occasion shall also fulfill the condition that it is the resource where UE is not performing GNSS fix. For other measurement in connected mode, the measurement delay could be longer if GNSS fix happens during the measurement period.

Observation 3: For inter-frequency neighbor cell measurement for NB-IoT, the measurement occasion shall also fulfill the condition that it is the resource where UE is not performing GNSS fix. For other measurement during connected mode, the measurement delay could be longer if GNSS fix happens during measurement period.

# Conclusions

Observation 1: The triggering methods for neighbour cell measurement in connected mode before RLF are as follows:

- Quasi-earth fixed cell:

  - Legacy NRSRP based triggering

  - T-service based triggering (the exact time to start measurements is up to UE implementation)

  - Location based triggering: reference location + distance threshold

- Earth-moving cell:

  - Legacy NRSRP based triggering

  - Location based triggering: reference location + distance threshold

Observation 2: The inter-frequency measurement in connected mode before RLF is “best efforts” measurement using vacant time slot, where the exact time for inter-frequency neighbour cell detection and measurement is unpredictable.

Proposal 2: For for t-service triggered neighbor cell measurement before RLF for NB-IoT:

Option 1: Do not define requirements for t-service triggered neighbour cell measurement.

Option 2: The UE shall be able to measure only intra-frequency neighbour cell before t-service provided that the time span from the SI broadcasting t-service to t-service is longer than Tidentify_intra, and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation.

Proposal 3: For location-based triggering, add the corresponding criteria (reference location and distance threshold) and the same requirements as NRSRP based triggering can apply, and the details should wait for more progress in RAN2.

Proposal 4: RAN4 should further wait for RAN2 progress before discussing exact requirements for time and [location based] CHO for eMTC.

Proposal 5: For location triggered cell reselection measurement,

- For eMTC and fixed cell, re-use the requirements from NR NTN

- For eMTC and moving cell, wait for further progress in Rel-18 NR NTN WI

- FFS whether the requirements are also applicable to NB-IoT pending on further RAN2 agreement

Observation 3: For inter-frequency neighbor cell measurement for NB-IoT, the measurement occasion shall also fulfill the condition that it is the resource where UE is not performing GNSS fix. For other measurement during connected mode, the measurement delay could be longer if GNSS fix happens during measurement period.

# References

[1] RP-213596 New WID on IoT NTN enhancements

[2] R4-2306387 WF on IoT_NTN_enh