3GPP TSG-RAN WG4 Meeting # 107	R4-2308316

Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.9.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on general aspects for SCell activation delay reduction

Document for:	Discussion

# Introduction

The Rel-18 WI of even further RRM enhancement was approved in [1] and further revised in [2]. One of the objectives is to study the SCell activation delay reduction in FR2. Considerable progress has been made based on previous discussion [3] [4] [5]. In this contribution, we provide our views on work plan for further discussion.

# Discussion

In RAN4#104e, the scope and work procedures of the WI was discussed with following agreements:

- Sub-topic 2-1 General Issue 2-1-1: Baseline scope for FR2 SCell activation enhancement Agreement: - 1st phase: RAN4 starts the discussion of the R18 unknown FR2 SCell activation enhancement based on baseline FR2 SCell activation requirement in TS38.133 section 8.3.2 - Discuss the feasibility and method of SCell activation enhancement based on AP-CSI-RS and/or A-TRS - 2nd phase: the other SCell activation cases, e.g., multiple SCell activation, direct SCell activation, and PUCCH SCell activation, to be discussed after baseline case (i.e. single FR2 SCell activation) is completed. - 2nd phase will start from RAN4 #107 meeting. Issue 2-1-2: Prioritization for initial phase Agreement: - RAN4 to prioritize at least FR2 unknown Scell delay reduction in the 1st phase of the WI. - FR2 unknown Scell without intra-band serving cell is considered for 1st phase. - The extension of the enhancement solutions to FR1 can also be discussed.

It could be observed that the discussion about multiple SCell activation, direct SCell activation and PUCCH SCell activation shall be discussed from RAN4#107 meeting. From our understanding, the enhancement is mainly focusing on enhancement on DL, e.g. L3 reporting after SCell activation, beam sweeping factor reduction, which are the common components for other SCell activation procedures are mentioned above. For instance, Tactivation_time are directly referred to section 8.3.2 for other SCell activations.

Observation 1: The FR2 enhancement under discussion are common for different SCell activation procedure containing Tactivation_time.

However, for multiple SCell activation, more discussion in needed before directly apply the enhancement. In legacy requirements for multiple SCell activation, it is assumed that Cell detection is not needed for all FR2 to-be-activated SCell. However, based on current progress, the enhancements are mainly focusing unknown FR2 SCell without active serving cell or known to-be-activated SCell in the same band, which means AGC/cell search is needed at least for one cell in each FR2 bands.

Observation 2: In current multiple SCell activation requirements, Cell search is not needed for FR2 to-be-activated SCell.

If applying the conclusion of enhancement under discussion for multiple SCell activation, RAN4 shall consider sharing the cell including FR2 SCells. Since the details of the enhancement is not completed yet, it is more efficiency to discuss the requirement for multiple SCell activation after the requirements for single Cell activation is completed.

Proposal 1: The enhancement under discussion can also apply to other single Cell activation including direct SCell activation and PUCCH SCell activation.

Proposal 2: RAN4 to discuss whether to define requirements for multiple SCell activation involving unknown FR2 to-be-activated SCell without active serving cell or known to-be-activated SCell in the same band. If RAN4 decide to define requirements, it should be discussed after the requirements for single SCell activation is completed.

Another issue worth clarification is whether FR2-2 is also included. The general principle in previous release is that FR2 include FR2-1 and FR2-2 unless explicitly stated otherwise. However, based on the discussion so far, it seems companies assumed only FR2-1 is included, as only beam sweeping factor of 8 is considered. Regarding the RRM impacts, if FR2-2 is also considered, at least the beam sweeping factor related enhancement shall be considered separately.

Proposal 3: RAN4 to clarify whether SCell activation requirements for FR2-2 are included or not.

# Conclusions

Observation 1: The FR2 enhancement under discussion are common for different SCell activation procedure containing Tactivation_time.

Observation 2: In current multiple SCell activation requirements, Cell search is not needed for FR2 to-be-activated SCell.

Proposal 1: The enhancement under discussion can also apply to other single Cell activation including direct SCell activation and PUCCH SCell activation.

Proposal 2: RAN4 to discuss whether to define requirements for multiple SCell activation involving unknown FR2 to-be-activated SCell without active serving cell or known to-be-activated SCell in the same band. If RAN4 decide to define requirements, it should be discussed after the requirements for single SCell activation is completed.

Proposal 3: RAN4 to clarify whether SCell activation requirements for FR2-2 are included or not.

# References

[1] RP-220977 New WID: Even Further RRM enhancement for NR and MR-DC

[2] RP-221696 Revised WID: Even Further RRM enhancement for NR and MR-DC

[3] R4-2220440, WF on R18 eFeRRM - FR2 SCell activation enhancement

[4] R4-2303228, Way forward on R18 RRM enhancement (part 1)

[5] R4-2306315, WF on R18 RRM enhancement- FR2 SCell activation delay reduction

[6] R4-2214345, WF on R18 eFeRRM