3GPP TSG-RAN WG4 Meeting # 107	R4-2308333

Incheon, KR, May 22 – May 26, 2023

Title: 	Consideration on enhanced CHO configurations

Source: 	Huawei, HiSilicon

Agenda item:	8.25.5

Document for:	Discussion

Introduction

This contribution provides some considerations on enhanced CHO from RRM perspective.

Discussion

In last meeting, it is agreed to specify requirements for both scenarios in objectives 3 and 4, that is,

	Scenario 1: CHO including target MCG and target SCG in NR-DC. (obj. 3)

	Scenario 2: CHO including target MCG and candidate SCG for CPC/CPA in NR-DC (obj. 4)

- Requirements for CHO including target MCG and target SCG in NR-DC. (obj. 3)

There are two remaining issues for CHO with target SCG [1].

Issue 3-2-1: PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) - Agreements: - PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) is defined as: - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution. - Definition of each component, except Tprocessing, is same as that defined in CHO requirement in TS38.133 clause 6.4.1.2. - Definition of Tprocessing: - Option 1: same as that defined in requirements of handover with PSCell. - Option 2: same as that defined in requirements of CHO. Issue 3-2-2: PSCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) - Candidate solutions: - Companies are encouraged to further discuss the following issues: - When does UE execute PSCell addition/change - Option 1: after PCell CHO is executed but before it is completed. - Option 2: after PCell CHO is completed. - Whether Tsearch_PCell_Conditional is neeed in DCHOwithPSCell_PSCell?

As we know, Tprocessing for PCell handover in existing handover with PSCell is specified as below, which adds some processing margin considering UE SW processing and RF warm-up for both parallel and sequential PCell HO and PSCell addition/change.

- Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 30 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 25 ms.

Tprocessing in existing conditional handover is specified as below, where only PCell CHO is performed.

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

Regarding PCell handover delay in CHO including target MCG and target SCG in FR1+FR2, it is more like HO with PSCell.

Proposal 1: Tprocessing for PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) is the same as that defined in requirements of handover with PSCell (i.e., option 1).

In existing requirement, PSCell change delay for handover with PSCell is defined as below, where PCell search time is considered.

##### 6.1.5.4.2 HO with PSCell – PSCell change delay The requirements in this section shall apply for PSCell change during handover with PSCell from NR DC to NR-DC. When handover with PSCell from NR-DC to NR-DC is commanded, the PSCell change time shall be less than DHOwithPSCell_PSCell: - DHOwithPSCell_PSCell = TRRC_delay + Tprocessing + Tsearch_PCell + Tsearch_PSCell + T∆ + TPSCell_ DU + 2 ms

In PSCell change in CHO with target PSCell procedure, UE shall first identify PCell and PSCell, therefore both PCell and PSCell search time are needed. After that UE evaluate condition of PCell, when PCell condition is fulfilled, UE performs PSCell addition procedure. It shall be noted that there is no need to evaluate PSCell condition as target PSCell is pre-configured/ bundled with target PSCell.

Proposal 2: PSCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) is defined as

DCHOwithPSCell_PSCell = TRRC + TEvent_DU + Tmeasure + Tsearch_PSCell + Tprocessing + TPSCell_ DU + T∆ + Tmargin + TCHO_execution

Where TEvent_DU Tmeasure and TCHO_execution are for PCell.

- Requirements for CHO including target MCG and candidate SCG in NR-DC. (Obj. 4)

It is agreed in RAN2 #121 bis meeting that the baseline is UE waits until both CHO and CPC conditions are met. Alternative is still under discussion. The following analysis focus on the baseline.

- For the CHO+CPC case: - When both CHO and CPC conditions are met, both CHO and CPC cell change is executed. - Baseline: The UE waits until both CHO and CPC conditions are met (always). (furthermore, it is assumed that if needed the network can provide a complementary CHO-only configuration, to avoid failures in deployments where failure would otherwise be likely to happen). - Alternative: FFS if When CHO condition is met, but CPC condition is not met, CHO execution is triggered (and somehow source SCG can be released). IF allowed in the new configuration the UE may continue evaluation of CPC/CPA conditions.

Once the execution condition of one candidate PCell is met and the execution condition of an associated candidate PSCell is met, the UE synchronizes to the selected/target PCell. Then the TEvent_DU would be updated, such as,

“TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until conditions fulfilled for PCell and associated candidate PSCell at the measurement reference point which will trigger the conditional handover. ”

Tprocessing is also supposed to be updated, which is the same as what discussed in obj.3 (Proposal 1).

PCell delay for CHO including target MCG and candidate SCG in NR-DC is:

DCHOwithcandidatePSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution

Where TEvent_DU and Tprocessing would be updated.

Proposal 3: PCell delay for CHO including target MCG and candidate SCG in NR-DC (Obj.4) is:

DCHOwithcandidatePCell_PCell = TRRC + TEvent_DU + Tmeasure + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution

where

TEvent_DU is updated as “TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until conditions fulfilled for PCell and associated candidate PSCell at the measurement reference point which will trigger the conditional handover.”

Tprocessing is the same as that defined in requirements of handover with PSCell.

Regarding the PSCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA, additional Tmeasure_PCell is supposed to be added.

Proposal 4: PSCell delay for CHO including target MCG and candidate SCG in NR-DC (Obj.4) is:

DCHOwithcandidatePSCell_PCell = = TRRC + TEvent_DU + Tmeasure_PCell+ Tmeasure_PSCell + Tprocessing + TPSCell_ DU + T∆ + Tmargin + TCHO_execution.

where

TEvent_DU is updated as “TEvent_DU is updated as “TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until conditions fulfilled for PCell and associated candidate PSCell at the measurement reference point which will trigger the conditional handover.”

Tprocessing is the same as that defined in requirements of handover with PSCell.

Conclusions

This contribution provides some consideration on CHO including target MCG and candidate SCGs for CPC/CPA.

Proposal 1: Tprocessing for PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) is the same as that defined in requirements of handover with PSCell (i.e., option 1).

Proposal 2: PSCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) is defined as

DCHOwithPSCell_PSCell = TRRC + TEvent_DU + Tmeasure + Tsearch_PSCell + Tprocessing + TPSCell_ DU + T∆ + Tmargin + TCHO_execution

Where TEvent_DU Tmeasure and TCHO_execution are for PCell.

Proposal 3: PCell delay for CHO including target MCG and candidate SCG in NR-DC (Obj.4) is:

DCHOwithcandidatePCell_PCell = TRRC + TEvent_DU + Tmeasure + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution

where

TEvent_DU is updated as “TEvent_DU is updated as “TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until conditions fulfilled for PCell and associated candidate PSCell at the measurement reference point which will trigger the conditional handover.”

Tprocessing is the same as that defined in requirements of handover with PSCell.

Proposal 4: PSCell delay for CHO including target MCG and candidate SCG in NR-DC (Obj.4) is:

DCHOwithcandidatePSCell_PCell = = TRRC + TEvent_DU + Tmeasure_PCell+ Tmeasure_PSCell + Tprocessing + TPSCell_ DU + T∆ + Tmargin + TCHO_execution.

where

TEvent_DU is updated as “TEvent_DU is updated as “TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until conditions fulfilled for PCell and associated candidate PSCell at the measurement reference point which will trigger the conditional handover.”

Tprocessing is the same as that defined in requirements of handover with PSCell.

Reference

- R4-2306396, WF on NR Mobility Enhancements RRM requirements (part 2), Apple