3GPP TSG-RAN WG4 Meeting # 107	R4-2308338

# Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.14.4.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on mobility requirements for ATG

Document for:	Discussion

- Introduction

A new Rel-18 WI to support NR Air-to-ground (ATG) network for NR was approved in [1] and further revised in [2]. In RAN4#106bis meeting, the RRM impacts on ATG were discussed with status captured in WF [3]. In this paper, we further provide our views on remaining issues.

- Discussion

- Mobility in RRC Idle/Inactive mode

For IDLE/Inactive state mobility requirements, one remaining open issue is cell re-selection mechanism:

Issue 2-1-1-1: Cell re-selection mechanism - Option 1: Introduce distance-based triggering as an additional condition for intra- and inter-frequency cell measurement as NTN, whether and how to configure the feature is up to network implementation. - If Srxlev > S(non)IntraSearchP and Squal > S(non)IntraSearchQ, and the distance between UE and serving cell reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter/intra-frequency layers of higher priority. - If Srxlev ≤ S(non)IntraSearchP or Squal ≤ S(non)IntraSearchQ, or the distance between UE and serving cell reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter/intra-frequency layers of higher, equal or lower priority in preparation for possible reselection. - Option 4: - If ATG UE altitude higher than certain threshold, ATG UE perform measurement - If ATG UE altitude lower than certain threshold, ATG UE may not perform measurement - Option 5: No need to specify distance, altitude or speed-based enhancements to cell re-selection mechanism during Release 18. - If needed, such mechanisms may be discussed in future releases. - Assistance information comprises at least changes in the UE altitude, flight path and speed, remaining service time for serving cell and side conditions can be FFS. - Other Options are not precluded.

Option 1 is similar as the scheme introduced in NTN. The justification of introducing distance related condition is mainly because the RSRP fluctuation may not well reflect UE location change in large cell. Taking distance as an additional condition may have benefit to ensure UE perform in-time measurement especially in a large cell.

In our understanding power saving is not essential for ATG UE, as ATG UE is a plug-in equipment. So if ATG UE is at a low altitude, UE can still perform measurement.

Other altitude, flight, or speed based triggered cell re-selection schemes are kinds of further measurement enhancement. At a late phase of the WI, it is not suggested to do further enhancement unless there is an obvious motivation.

Proposal 1: For ATG, RSRP + distance-based triggering for intra- and inter-frequency measurement principle for cell re-selection can be introduced for idle mode ATG UE.

Proposal 2: The benefits of further enhancements on measurement in idle/inactive mode (e.g., based on altitude, flight, or speed assistance information) need to be justified, otherwise these enhancements are not suggested to be specified in this release.

If Option 1 above is supported, we think the legacy referencelocation-r17 can be reused for ATG.

Issue 2-1-1-2: If Cell re-selection mechanism as Option 1/4 is agreed, how does network signal the reference location and distance threshold to UE - First wait for RAN2’s response on the LS in R4-2303684 If location-based cell-reselection is supported, further discuss whether the legacy referencelocation-r17 can be reused for ATG, referencelocation-r17 related information are as follows:

Proposal 3: If proposal 1 is supported, the legacy referencelocation-r17 can be reused for ATG.

In the last meeting, it is agreed that two sets of requirements are introduced for inter-frequency measurement for ATG.

    - Set 1: legacy R15 cell-reselection requirement

    - Set 2: R16 HST cell-reselection requirement

In R17 whether UE is able to support enhanced HST cell-reselection requirements depends on UE capability. To provide UE flexibility, it is suggested to introduce optional UE capability to support enhanced HST requirements.

Proposal 4: Introduce optional ATG UE capability to support enhanced HST requirements.

- Mobility in RRC Connected mode

For CONNECTE state mobility, one remaining open issue is cell re-selection mechanism:

Issue 2-2-1: location-based CHO mechanism - Option 1: For the location-based CHO, reusing the procedure in R17 NTN for R18 ATG (combines RSRP and location based triggering condition) - Option 2: Location based CHO is supported for A2G in Rel-18, and details of requirements are FFS - Option 3: Using BS location information and UE location information, CHO can be used for UE located on the blind area of ATG BS. - Option 4: Not to specify location-based CHO, such mechanism can be specified in future releases

It is agreed in RAN4#106 meeting to introduce conditional handover for ATG. In R16 for TN network, only RSRP based CHO is supported. In R17 RSRP+ location based CHO is introduced in NTN network. So far separate location based CHO is not supported yet. For ATG network, as the airplane trajectory is almost deterministic, location information can be used as one triggerfor handover.

Proposal 5: RSRP+ location based CHO can be considered for connected mode ATG UE.

- Conclusions

This contribution provides analysis on mobility requirements for ATG. The following proposals are proposed:

Proposal 1: For ATG, RSRP + distance-based triggering for intra- and inter-frequency measurement principle for cell re-selection can be introduced for idle mode ATG UE.

Proposal 2: The benefits of further enhancements on measurement in idle/inactive mode (e.g., based on altitude, flight, or speed assistance information) need to be justified, otherwise these enhancements are not suggested to be specified in this release.

Proposal 3: If proposal 1 is supported, the legacy referencelocation-r17 can be reused for ATG.

Proposal 4: Introduce optional ATG UE capability to support enhanced HST requirements.

Proposal 5: RSRP+ location based CHO can be considered for connected mode ATG UE.

References

[1] RP-220962 New WID: Air-to-ground network for NR, CMCC

[2] RP-221369 Revised WID on Air-to-ground network for NR, CMCC

[3] R4-2306344, WF on NR ATG RRM requirements, CMCC