3GPP TSG-RAN WG4 Meeting # 107									                   R4-2308339

Incheon, KR, May 22 – May 26, 2023

Title: 					Discussion on RRM impact for network energy savings for NR

Source: 	Huawei, HiSilicon

Agenda item:	8.35.4

Document for:	Discussion

Introduction

The new WI on network energy savings for NR has been approved in [1]. Technical Report output in SI phase is captured in TR 38.864. The WID has the following objectives:

The objectives of the work item are the following: - Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2] - Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided. - Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements - Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2] - Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2] - Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3]. - Specify the corresponding RRM/RF core requirements, if necessary, for the above features [RAN4]

This contribution provides the preliminary analysis on the possible RRM impacts due to network energy saving.

Discussion

SSB-less SCell operation

- Scenarios

In last meeting, the following scenarios are proposed for SSB-less SCell operation [2].

< Agreement >Issue 2-1-1: Which scenario to be considered for SSB-Less SCell operation for FR1 inter-band collocated CA Online session (Friday April 21, 2023) - Agreements - The following scenarios are considered for inter-band SSB-less SCell requirements definition - Scenario 1: No SSB but with TRS transmission configured on the SSB-less SCell - FFS whether to consider requirements based on TRS transmission during and/or after SCell activation - Scenario 2: No SSB and no TRS transmission configured on the SSB-less SCell - Scenario 2a: No DL transmission but with UL reception at the NW side on the SSB-less SCell. - FFS if requirements will be defined for all or a subset of scenarios subject to feasibility analysis

In scenario 1, there is no SSB but with TRS configured transmission.

During SCell activation procedure, TRS is used for QCL configuration, e.g., CSI-RS for CSI reporting/ beam management is QCL type-A with TRS on the to-be-activated SCell, and the TRS is further QCL-TypeC with SSB of other active serving cell. As we know A-TRS shall QCL type-A to periodic TRS on the same serving cell, and A-TRS is not able to directly QCL-A/C to other cell,  therefore in essence, TRS is also needed even A-TRS is present on the serving cell.

After SCell activation procedure, UE perform TRS based time tracking to guarantee PDCCH/PDSCH performance. In our understanding, scenario 1 is a typical SSB-less Scenario.

Observation 1: Scenario 1 is a typical SSB-less Scenario.

In scenario 2, there are no SSB and no TRS configured transmission. In our understanding, if there is no TRS in the cell, to guarantee demodulation performance, strict restriction of time alignment, PDP distribution and Doppler spread between SSB-less SCell and PCell are required. Moreover to support higher-order modulation, further stricter sync (far less than 260ns timing error between SSB-less SCell and PCell) and almost identical channel QCL conditions are required. We are not sure whether these restrictions can be well guaranteed in realistic network. In practical, to achieve good demodulation performance, an effective way is that UE perform time tracking based on TRS on its own SCell.

Therefore from demodulation performance perspective, SSB-less and TRS-less SCell operation would bring performance degradation.

Observation 2: From demodulation performance perspective, SSB-less and TRS-less SCell operation in Scenario 2 may bring performance degradation.

In scenario 2a, there is no PDCCH/PDSCH but with UL reception at the NW side on the SSB-less SCell. We understand the motivation of scenario 2a is to further enlarge network energy saving by shutting down the DL transmitter of the NW completely. Under such use case, from NW perspective, there is no PDCCH/PDSCH but there can be UL reception in the SSB-less SCell. However in our understanding the “UL only SCell” configuration may have some potential impacts in RAN2.

- As per current RAN2, upon Scell addition, some DL related parameters (e.g., BWP specific parameter) are mandatory present.

- As per RAN2 spec TS38.321, the normal SCell operation includes CSI reporting and PDCCH monitoring. It means that at least PDCCH is to be transmitted.

Moreover the ending point of SCell activation procedure is CSI-RS reporting, then it means that CSI-RS is supposed to be transmitted for CQI on to-be-activated SCell.

In addition, Scenario 2a requires all the UEs have no DL reception requirements, is a kind of an extreme example of Cell DTX where the SCell is always in the non-active duration.

Observation 3: More discussion is needed to analyse whether scenario 2a has some potential impacts on other RAN group.

Proposal 1: For SSB-less operation, Scenario 1 is to be prioritized. Scenario 2 and 2a can be further discussed.

- Conditions for SSB-less inter-band CA SCell operation

Based on above analysis, we propose to firstly focus on scenario 1 (No SSB but with TRS transmission). In last meeting, the below conditions are proposed.

< Agreement >Issue 2-1-2: From RRM perspective from UE side, under which conditions SSB-less SCell operation can work in scenario-1? Online session (Monday April 24, 2023) Agreements - Conditions for SSB-less inter-band CA SCell operation for scenario-1 - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell - Option 1-1: 260ns - Option 1-2: 3us - Option 1-3: smaller than 3us - Other values are not precluded - The difference of the reception power with the FR1 inter-band active serving cell - Option 2-1: 6dB - Option 2-2: 6dB+offset - Other values are not precluded - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell) - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations. - FFS if requirements will be defined subject to feasibility analysis - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a

- RTD

In intra-band continuous CA scenario, the carrier frequency is continuous. Taking CA_n77 as an example, CA_n77, the frequency span can be 200 MHz. We observe that in certain inter-band CA bands, spectrum can be adjacent, and the inter-band span can be smaller than that in some single-band intra-band CA scenarios. For example, for CA_n8-n20（900M+800M）, CA_n8-n28 (900M+700M) , CA_n5-n14(850M+700M), the frequency band spans are separately 169M, 202M, 136M. It means that certain inter-band CA combinations may have a narrower or comparable frequency range than some intra-band contiguous CA combinations.

Although in current specification that the TAE of inter-band CA is 3 μs, at least in certain band combinations, the TAE of inter-band CA can be much better than 3 μs, and comparable to intra-band continuous CA, i.e., 260ns. From BS architecture perspective, FR1 co-located inter-band CA can be implemented by the common transmitter, as shown in Figure 1(a）, or by the independent transmitters, as shown in Figure 1(b).

- Figure 1 Implementation Architecture of co-located BS CA (a) CA by the common transmitter; (b) CA by the independent transmitters

In case (a), FR1 co-located inter-band CA is implemented with the common transmitter, for example, a multi-band TX, which means that the majority of the components are shared by the component carriers. Therefore, the TAE could be controlled in the same way as intra-band contiguous carrier aggregation, ensuring it does not exceed 260ns.

In case (b), FR1 co-located inter-band CA implemented with the independent transmitters. The time delay uncertainty mainly comes from the different analogue components such as SERDES, ADC, transmitter filter which can be controlled under a limited range. Moreover in co-located CA scenario, there is no time deviation between carrier components due to optical fiber extension. Therefore i even implementing FR1 co-located inter-band CA with independent transmitters, in certain band combination, the delay deviation can also be within 260ns.

It shall be noted that it does not intend to revise or tighten the existing TAE. If RAN4 can justify that in certain band combination, the received time difference between two CC can be less than 260ns, then the frequency/time synchronization of SSB-less SCell can utilize that of the activated cell on inter-band. Herein the procedures of cell detection and fine timing can naturally be skipped.

Observation 4: In certain band combinations, the received time difference between SSB-less SCell and the FR1 inter-band active serving cell can be less than 260ns.

In last meeting, some companies proposed that RTD between the SSB-less SCell and the FR1 inter-band active serving cell is 3us. However if SCS is 30KHz, 3us is larger than CP. Then then coarse timing of PCell is not able to be applied to SSB-less SCell. The sequence is that UE is not able to correctly receive TRS if UE refers to PCell’s timing. In other words, SSB shall be transmitted in the to-be-activated SCell if RTD is 3us @30KHz SCS.

Observation 5:  SSB-less operation is not feasible if RTD is 3us @30KHz SCS.

- Power difference

Under certain band combinations, e.g., 700M,800M, 900M or 1.8G,2.1G ,2.6 G, the transmit power configuration for each CC can be the same. From UE perspective, the received power difference can be within 6dB. Then AGC gain of reference Cell can be applied for SSB-less SCell. AGC adjustment is not needed during SCell activation.

Observation 6: In certain band combinations, the received power difference from two CCs can be within 6dB.

- TCI state/beam

Although we think in certain inter-band combination same DL TX beams for two collocated inter-band cells are a typical scenario, from specifying requirement perspective it is not a proper way to define network behaviour. Moreover recalling R15 intra-band continuous CA, no TCI state related condition is included either. In essence, the intention of same or strong correlated TX beam is to guarantee same RTD at UE side. RTD 260ns condition is sufficient for UE to refer to all required information on an already activated serving cell.

As in scenario-1, TRS is configured on the SSB-less SCell, the QCL configuration for is straight forward:

“The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell on different band.”

Observation 7: In scenario-1, the following QCL relation is one condition for SSB-less SCell activation:

“The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell on different band.”

- Inter-band combination/ Frequency domain separation

The band combination or frequency domain separation is indeed an issue and needs to be discussed. We suppose this would be discussed in RF session. From RRM perspective, the discussion of frequency domain separation or band combination is to decide the value of RTD/ power difference, rather than a condition.

Observation 8: Band combination or frequency domain separation will be discussed in RF session, and it would not be captured as a condition in RRM requirements.

Proposal 2: In scenario-1 for certain band combinations, SSB-less SCell operation can be supported under the following conditions:

-	Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell is within ±260ns, and

-	The difference of the reception power with the FR1 inter-band active serving cell is <= 6dB

-	The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell on different band.

- L1/L3 measurement on SSB-less SCell

- Sub-topic 2-4: L1/L3 measurement operation on the SSB-less SCell Issue 2-4-1: Scenario for L1 measurement operation on the SSB-less SCell apart from SCell activation procedure - Candidate Options: - Option 1: No SSB but with CSI-RS resource for L1 measurement on the inter-band SSB-less SCell - Option 2: No SSB and No CSI-RS resource for L1 measurement on the inter-band SSB-less SCell Issue 2-4-2: Scenario for L3 measurement operation on the SSB-less SCell apart from SCell activation procedure - Candidate Options: - Option 1: No SSB but with CSI-RS resource for L3 measurement on the inter-band SSB-less SCell - Option 2: No SSB and No CSI-RS resource for L3 measurement on the inter-band SSB-less SCell Issue 2-4-3: L1/L3 measurement operation on the SSB-less SCell - Candidate Options: - Option 1: given that the SSB-less SCell can fully leverage the information from an already activated serving cell, the corresponding L1/L3 measurements can be skipped. - Option 2: For UE deriving SSB-less SCell L1/L3 measurements results by performing L1/L3 measurements on PCell or another SCell, whether and how to assist UE to determine the coverage of SSB-less SCell should be further investigated. - Option 3: The UE is not required to perform SSB-based L1/L3 measurements on the SSB-less SCells. The CSI-RS based L1/L3 measurement requirements need to be specified for SSB-less SCell operation.

For UE who only supports SSB-based L1 measurement, the scenarios with CSI-RS and without CSI-RS resources are the same.

For UE who can support CSI-RS based L1 measurement,

-When CSI-RS resources for L1 measurement are configured, it is straight forward to perform CSI-RS based L1/L3 measurement. Herein the legacy requirements for CSI-RS based L1 measurement can be reused for SSB-less SCell operation.

- When CSI-RS resources for L1 measurement are not configured, as long as the RTD, power difference conditions are ensured between two carriers, the SSB-less SCell can fully leverage the information from an already activated serving cell, the corresponding L1 measurements can be skipped.

Proposal 3: When CSI-RS resources for L1 measurement are configured, the legacy requirements for CSI-RS based L1 measurement can be reused for SSB-less SCell operation.

Proposal 4: When CSI-RS resources for L1 measurement are not configured, as long as the RTD, power difference conditions are ensured between two carriers, the SSB-less SCell can fully leverage the information from an already activated serving cell, the corresponding L1 measurements can be skipped.

Regarding L3 measurement, as long as the RTD, power difference conditions are ensured between two carriers, the SSB-less SCell can fully leverage the information from an already activated serving cell. Then the L3 measurements results on SCell can leverage the SSB based measurement results on PCell. Similar for neighbour cell measurement, the SSB based measurements on PCC can be leveraged for SSB-less SCC. Therefore it is not necessary to consider CSI-RS based L3 measurement on SSB-less SCell.

Proposal 5: As long as the RTD, power difference conditions are ensured between two carriers, the SSB based L3 measurements on PCC can be leveraged for SSB-less SCC.

DTX/DRX

For the RRM impacts of Cell DTX and DRX, it was agreed to wait for more inputs from RAN1/2.

< Agreement >Issue 3-1: RRM impacts due to Cell DTX/DRX (obj#2) - RRM impacts due to cell DTX can wait for more inputs from RAN1/2.

In this contribution, we further provide the analysis based on latest RAN1/2 progress. In last RAN1 meeting, it was further investigating which signals/channels are not expected to be received from UE’s perspective during non-active period in Cell DTX. The status is summarized as follows:

Agreement From RAN1 point of view, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions. - Periodic/Semi-persistent CSI-RS configured in CSI report configuration in CSI-ReportConfig with reportQuantity including RI (for CSI reporting) - FFS: - PDCCH in USS - UE behaviour for retransmission - if some specific RNTI scrambled PDCCH in USS will be excluded from cell DTX operation - PDCCH in Type-3 CSS - UE behaviour for retransmission - if some specific RNTI scrambled PDCCH in Type-3 CSS will be excluded from cell DTX operation - PRS - CSI-RS configured by measObjectNR (for RRM) - CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDectection (for RLM and BFD) - Periodic CSI-RS configured with trs-Info ‘true’ (for tracking) - Periodic/Semi-persistent CSI-RS (for BM) - FFS on how to differentiate (if needed) with other CSI-RS used for CSI reports for BM - FFS: Whether the same or different UE behaviour is applicable with or without C-DRX - FFS: Whether the list of impacted signals/channels can be configurable - FFS: Whether there will be exception case(s) for UE receiving and/or processing listed signals/channels during non-active periods of DTX - FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period

It could be observed that whether UE needs to monitor CSI-RS for RRM/RLM/BFD/tracking/BM are still under discussion, especially for P/SP CSI-RS. For AP CSI-RS, UE still needs to monitor it following NW scheduling. From RAN4 perspective, if it is finally concluded in RAN1 that UE shall still monitor these CSI-RS in non-active period of Cell DTX, then there are no impacts on RRM requirements. If UE is not required to monitor the CSI-RS, RAN4 may need to further investigate whether to update the current RRM requirements. For instance, considering the periodicity of DTX. Besides, the alignment and interaction between Cell DTX and UE C-DRX is also under discussion in RAN1/RAN2. Based on the analysis above, RAN4 shall further wait for more progress in RAN1 and RAN2.

Observation 9: Whether UE needs to monitor P/SP CSI-RS for RRM/RLM/BFD/tracking/BM during non-active period of Cell DTX is under discussion in RAN1.

Observation 10: The alignment and interaction between Cell DTX and UE C-DRX is under discussion in RAN1/RAN2.

Proposal 6: RAN4 shall wait for more progress in RAN1/2 to investigate whether there is RRM impacts of DTX/DRX.

Spatial and power domain techniques

For spatial/power domain adaptation, it was agreed to wait for more inputs from RAN1/2.

< Agreement >Issue 3-2: RRM impacts due to Spatial and power domain techniques (obj#3) - Discuss the RRM requirement for spatial and power adaptation in NES after RAN1 has more concrete conclusions.

Based on latest progress in RAN1, the discussion is mainly about the framework to allow reporting for different spatial adaptation patterns and power offset. Besides, only CSI-RS for CSI is considered in RAN1, and whether CSI-RS for other usage (e.g. beam management) is also subject to spatial/power adaptation is still FFS. Thus, similar as the Cell DTX/DRX, RAN4 shall wait for more progress of RAN1 on spatial and power adaptation to investigate whether there is RRM impacts.

Observation 11: Only CSI-RS for CSI is considered in RAN1, and whether CSI-RS for other usage (e.g. beam management) is also subject to spatial/power adaptation is still FFS.

Proposal 7: RAN4 shall wait for more progress of RAN1 on spatial and power adaptation to investigate whether there is RRM impact.

Cell selection/reselection

For cell selection/reselection, it was agreed to wait for more inputs from RAN2.

< Agreement >Issue 3-3: RRM impacts due to Cell selection/reselection (obj#4) 4 Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2]. - wait further RAN2 progress

Though there is no discussion in RAN2 in the last meeting, from our understanding, the objective is mainly targeting the enhancement in signaling aspects where UE shall be prevented to camp on a cell in NES mode. There are no significant impacts on RRM requirements for cell selection/reselection.

Proposal 8: Obj#4 (mechanism to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques) has no RRM impact.

CHO

For CHO, it was agreed to wait for more inputs from RAN2.

< Agreement >Issue 3-4: RRM impacts due to CHO (Obj#5) 5 Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2]. - RRM impact due to CHO enhancement in NES mode can wait for more inputs from RAN2.

The latest status of RAN2 is shown as follows:

Agreements

-	RAN2 agree to make enhancement in CHO procedure based on that the source cell entering “NES mode”.  FFS further details

-	For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode”

-	As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration.  FFS what trigger is used for execution of CHO

It could be observed that RAN2 is discussing the CHO which may be triggered by source cell entering “NES mode”. However, the details about the procedure is still premature and RAN2 will further discuss it. Regarding the RRM impacts, RAN4 may need to have requirements for CHO with specific triggering of NES. It is suggested to further investigate the RRM impacts with more concrete conclusion in RAN2.

Observation 12: RAN2 is discussing CHO triggered by source cell entering “NES mode”.

Proposal 9: RAN4 shall wait for more progress of RAN2 on CHO to investigate whether there is RRM impacts.

Conclusions

This contribution provides preliminary analysis on the RRM impact requirements for network energy saving. According to the analysis, we propose the following proposals.

Observation 1: Scenario 1 is a typical SSB-less Scenario.

Observation 2: From demodulation performance perspective, SSB-less and TRS-less SCell operation in Scenario 2 may bring performance degradation.

Observation 3: More discussion is needed to analyse whether scenario 2a has some potential impacts on other RAN group.

Proposal 1: For SSB-less operation, Scenario 1 is to be prioritized. Scenario 2 and 2a can be further discussed.

Observation 4: In certain band combinations, the received time difference between SSB-less SCell and the FR1 inter-band active serving cell can be less than 260ns.

Observation 5:  SSB-less operation is not feasible if RTD is 3us @30KHz SCS.

Observation 6: In certain band combinations, the received power difference from two CCs can be within 6dB.

Observation 7: In scenario-1, the following QCL relation is one condition for SSB-less SCell activation:

“The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell on different band.”

Observation 8: Band combination or frequency domain separation will be discussed in RF session, and it would not be captured as a condition in RRM requirements.

Proposal 2: In scenario-1 for certain band combinations, SSB-less SCell operation can be supported under the following conditions:

-	Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell is within ±260ns, and

-	The difference of the reception power with the FR1 inter-band active serving cell is <= 6dB

-	The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell on different band.

Proposal 3: When CSI-RS resources for L1 measurement are configured, the legacy requirements for CSI-RS based L1 measurement can be reused for SSB-less SCell operation.

Proposal 4: When CSI-RS resources for L1 measurement are not configured, as long as the RTD, power difference conditions are ensured between two carriers, the SSB-less SCell can fully leverage the information from an already activated serving cell, the corresponding L1 measurements can be skipped.

Proposal 5: As long as the RTD, power difference conditions are ensured between two carriers, the SSB based L3 measurements on PCC can be leveraged for SSB-less SCC.

Observation 9: Whether UE needs to monitor P/SP CSI-RS for RRM/RLM/BFD/tracking/BM during non-active period of Cell DTX is under discussion in RAN1.

Observation 10: The alignment and interaction between Cell DTX and UE C-DRX is under discussion in RAN1/RAN2.

Proposal 6: RAN4 shall wait for more progress in RAN1/2 to investigate whether there is RRM impacts of DTX/DRX.

Observation 11: Only CSI-RS for CSI is considered in RAN1, and whether CSI-RS for other usage (e.g. beam management) is also subject to spatial/power adaptation is still FFS.

Proposal 7: RAN4 shall wait for more progress of RAN1 on spatial and power adaptation to investigate whether there is RRM impact.

Proposal 8: Obj#4 (mechanism to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques) has no RRM impact.

Observation 12: RAN2 is discussing CHO triggered by source cell entering “NES mode”.

Proposal 9: RAN4 shall wait for more progress of RAN2 on CHO to investigate whether there is RRM impacts.

References

- RP-223540, New WID: Network energy savings for NR, Huawei

- R4-2306369, WF on RRM requirements for NR network energy saving, Huawei