3GPP TSG-RAN4 Meeting #107	R4-2308355

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on performance aspects of IoT NTN

Agenda item:	9.7.6

Document for:	Discussion

Introduction

In Rel-17, support for NB-IoT and eMTC over NTN was studied in [1] and defined in RAN1, RAN2, and RAN3 under the Work Item described in [2]. RAN4 requirements will be defined in a release-independent manner in Rel-18. The work was initiated in the RAN4 #104 meeting and must be continued following the WF from previous meetings and the WID [3].

In meeting RAN4 #106-bis, the discussion on the WI has progressed and some issues were identified and left for further studies on the Way-Forward approved in that meeting [4].

In this document, Nokia’s views regarding the following topics are presented regarding the open issues 2, 4 and 6-1 in [4] as well as discussing the configuration for eMTC test cases.

Discussion

Discussion on open issues

Power Headroom

In TS 36.133, the Power Headroom is defined as the difference between the UE   configured maximum output power as defined in TS 36.105 (Pcmax) and the estimated power used for UL-SCH or UL-NSCH transmission of the serving cell. It is expected, based on the link budget analysis provided in TR 36.763 that for both NGSO and GSO the UE is likely to be transmitting close to maximum transmit power. It is unlikely that the range for power headroom needs any differentiation for GSO and NGSO operation. The advantage of the PHR report in NTN is rather limited, as the network does not expect the same flexibility on UL allocation of resources as in terrestrial networks. .

Use the same PHR table for GSO and NGSO operations (Option 1).

Common Delay Configuration

The usage of satellite assistance information is based on both: ephemeris and common delay parameters, which respectively correspond to the propagation delay in the service link and feeder link. Both parameters are necessary for UE procedures, in special timing pre-compensation. The feeder link delay parameters are provided by the ta-Common, ta-CommonDrift and ta-CommonDriftVariant in SIB31. And according to TS 36.213[8], they are to be used such that the calculated common delay corresponds to “the distance at time t between the serving satellite and the uplink time synchronization reference point divided by the speed of light”. For developing a configuration for common delay we propose the following:

The common delay parameters provided in SIB31 for the RRM test cases can be obtained assuming a reference point on Earth whose elevation angle to the satellite position given by the ephemeris does not fall below 30 degrees during the test.

Discussion on test configurations

In the previous meetings, RAN4 has agreed that the test cases for eMTC (M1 UEs) will be performed using a band of 1.4 MHz. The problem though is that the reference channels provided  in section A.3.1.3 of TS 36.133 do not include reference channels for this band. In the previous batch of draftCRs to introduce the new test cases for NTN, companies have included the reference channels provided in A.3.1.3, but those do not apply for 1.4 MHz.

RAN4 to introduce reference channels for Cat-M1 UEs in Annex A.3.1.3, for both MPDDCH and MPDSCH.

Table A.3.1.3.1-1: MPDCCH Reference Channel for Cat-M1 FDD UEs in CEModeA

The following set of tables represent one example in how we propose to fix this problem.

Table A.3.1.3.1-1: MPDCCH Reference Channel for Cat-M1 FDD UEs in CEModeA

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MPDCCH Reference channel | - | R.16 FDD | R.17 FDD | R.24 FDD | R.25 FDD | R.46 FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 | 1.4 |
| Number of transmitter antennas | - | 1 | 2 | 1 | 2 | 1 |
| DCI Format | - | 6-1A | 6-1A | 6-1A | 6-1A | 6-1A |
| Transmission Type | - | Distributed | Distributed | Distributed | Distributed | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 4 | 4 | 4 | 4 | [4] |
| Aggregation level | ECCE | 16 | 16 | 16 | 16 | [16] |
| Maximum number of repetitions | - | 8 | 8 | 8 | 8 | 8 |
| Frequency hopping | - | ON | ON | ON | ON | OFF |
| Number of narrowbands | - | 2 | 2 | 2 | 2 | 1 |
| MPDCCH Narrowband | - | 7th | 7th | 4th | 4th | - |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 | - |
| MPDCCH start subframe | subframes | 1 | 1 | 1 | 1 | 1 |
| MPDCCH start symbol | symbols | 2 | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 4 | 4 | 4 | 4 | 4 |
| Payload (without CRC) | Bits | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| Cell ID | - | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: Payload size shall depend upon the test configuration. Note 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |  |

Table A.3.1.3.2-1: MPDCCH Reference Channel for Cat-M1 HD-FDD UEs in CEModeA

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MPDCCH Reference channel | - | R.6 HD-FDD | R.7 HD-FDD | R.14 HD-FDD | R.15 HD-FDD | R.47 HD-FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 | 1.4 |
| Number of transmitter antennas | - | 1 | 2 | 1 | 2 | 1 |
| DCI Format | - | 6-1A | 6-1A | 6-1A | 6-1A | 6-1A |
| Transmission Type | - | Distributed | Distributed | Distributed | Distributed | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 4 | 4 | 4 | 4 | [4] |
| Aggregation level | ECCE | 16 | 16 | 16 | 16 | [16] |
| Maximum number of repetitions | - | 8 | 8 | 8 | 8 | 8 |
| Frequency hopping | - | ON | ON | ON | ON | OFF |
| Number of narrowbands | - | 2 | 2 | 2 | 2 | 1 |
| MPDCCH Narrowband | - | 7th | 7th | 4th | 4th | - |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 | - |
| MPDCCH start subframe | subframes | 1 | 1 | 1 | 1 | 1 |
| MPDCCH start symbol | symbols | 2 | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 4 | 4 | 4 | 4 | - |
| Payload (without CRC) | Bits | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| Cell ID | - | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: Payload size shall depend upon the test configuration. Note 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |  |

Table A.3.1.4.1-1: PDSCH Reference Channel for Cat-M1 FDD in CEModeA

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH Reference channel |  | R.20 FDD | R.21 FDD | R.28 FDD | R.29 FDD | R.32 FDD | R.33 FDD | R.48 FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 | 10 | 10 | 1.4 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 2 | 1 | 2 | 1 |
| Allocated resource blocksNote1 | PRBs | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated subframes per Radio Frame | subframes | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |  |  |  |
| Sub-Frames 0 ~ 9 |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Maximum number of repetitions |  | 16 | 16 | 16 | 16 | 1 | 1 | 16 |
| Frequency hopping |  | ON | ON | ON | ON | ON | ON | OFF |
| Number of narrowbands for frequency hopping | narrowbands | 2 | 2 | 2 | 2 | 2 | 2 | 1 |
| PDSCH Narrowband |  | 2nd | 2nd | 2nd | 2nd | 2nd | 2nd | - |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 | 7 | 7 | - |
| PDSCH start symbol | symbols | 2 | 2 | 2 | 2 | 2 | 2 | 1 |
| Frequency hopping interval | subframes | 4 | 4 | 4 | 4 | 4 | 4 | - |
| Cell ID |  | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |  |  |  |

Table A.3.1.4.2-1: PDSCH Reference Channel for Cat-M1 HD-FDD in CEModeA

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH Reference channel |  | R.10 HD-FDD | R.11 HD-FDD | R.18 HD-FDD | R.19 HD-FDD | R.24 HD-FDD | R.25 HD-FDD | R.49 HD-FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 | 10 | 10 | 1.4 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 2 | 1 | 2 | 1 |
| Allocated resource blocksNote1 | PRBs | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated subframes per Radio Frame | subframes | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |  |  |  |
| Sub-Frames 0 ~ 9 |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Maximum number of repetitions |  | 16 | 16 | 16 | 16 | 1 | 1 | 16 |
| Frequency hopping |  | ON | ON | ON | ON | ON | ON | OFF |
| Number of narrowbands for frequency hopping | narrowbands | 2 | 2 | 2 | 2 | 2 | 2 | 1 |
| PDSCH Narrowband |  | 1st | 1st | 1st | 1st | 1st | 1st | - |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 | 7 | 7 | - |
| PDSCH start symbol | symbols | 2 | 2 | 2 | 2 | 2 | 2 | 1 |
| Frequency hopping interval | subframes | 4 | 4 | 4 | 4 | 4 | 4 | - |
| Cell ID |  | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |  |  |  |

Conclusion

This paper has analyzed some open issues on the performance part and the test configuration for NB-IoT/eMTC over NTN. Based on the discussions the following set of observations and proposals were made:

Proposal 1: Use the same PHR table for GSO and NGSO operations (Option 1).

Proposal 2: The common delay parameters provided in SIB31 for the RRM test cases can be obtained assuming a reference point on Earth whose elevation angle to the satellite position given by the ephemeris does not fall below 30 degrees during the test.

Proposal 3: RAN4 to introduce reference channels for Cat-M1 UEs in Annex A.3.1.3, for both MPDDCH and MPDSCH.

- References

- TR 36.763, “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)”

- RP-211601 – NB-IoT/eMTC support for Non-Terrestrial Networks

- RP-220938 – New WID on NB-IoT/eMTC core & performance requirements for NTN

- R4-2306370, WF on NTN NB-IoT/eMTC RRM requirements, Mediatek, RAN WG4 #106-bis-e, electronic meeting

- R4-2302343, Discussion on RRM mobility aspects for IoT over NTN, Nokia, Nokia Shanghai Bell, RAN WG4 #106, Athens, Greece.

- TS 36.133, “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”, 3GPP, Release 18.0.0

- TS 36.304, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode, 3GPP, Release 17.3.0