3GPP TSG-RAN4 Meeting #107	R4-2308356

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on remaining open issues for the core part of IoT NTN

Agenda item:	9.7.5

Document for:	Discussion

Introduction

In Rel-17, support for NB-IoT and eMTC over NTN was studied in [1] and defined in RAN1, RAN2, and RAN3 under the Work Item described in [2]. RAN4 requirements will be defined in a release-independent manner in Rel-18. The work was initiated in the RAN4 #104 meeting and must be continued following the WF from previous meetings and the WID [3].

In meeting RAN4 #106-bis, the discussion on the WI has progressed and some issues were identified and left for further studies on the Way-Forward approved in that meeting [4].

In this document, Nokia’s views regarding the following topics are presented regarding the open issues on discontinuous coverage aspects including the measurement relaxation of the measurements on the serving cell.

Discussion

Measurement Relaxation

The issue 2-1 in R4-2306370 [4], discusses whether the measurement relaxation for the serving cell measurements in RRC_IDLE should be further enhanced by new relaxation triggers.

#### Issue 2-1: NGSO, serving cell measurement - Option 1: Postpone the discussion on enhancements of serving cell measurements to the Rel-18 WI - Option 2: For serving cell: In addition to the existing serving cell relaxation conditions, add following conditions for IoT NTN: - The time span to T-service is longer than k, where k = 4 DRX cycle if not configured with eDRX_IDLE cycle, and k = 1 eDRX cycle if configured with eDRX_IDLE cycle.

From the RRM point of view, the mobility is impacted by the measurement relaxation, as it will make the interval between two consecutive measurements on the serving cell larger. But, from the mobility point of view in RRC_IDLE there are two main reasons to perform Serving Cell measurements:

- To initate measurements of neighbor cells if the criterion S is not met

- To rank the serving cell for the cell reselection procedure

The main reason to execute serving cell measurements is to trigger neighbor cell measurements.

Regardless of whether the conditions of the serving cell right before t-service are good or bad, the UE knows (by the configuration of t-service) that the serving cell will disappear at t-service. Based on the average scenario is even more likely that the conditions on the serving cell are good before the cell is discontinued.  Any last effort to collect serving cell measurements becomes unnecessary, as these measurements are soon to become outdated.

The main justification to collect serving cell measurements is also not justified, as the configuration of t-service is already a trigger on itself to initiate neighbor cell measurements. This has been agreed in RAN2 and already captured in specification by RAN4:

RAN2 #120 - Agreements: 1. At least for NB-IoT NTN, for quasi-earth fixed cells, UE shall start intra/inter frequency measurement in connected mode before the t-Service if present. The exact time to start measurements in connected mode before t-Service can be left to UE implementation” (can revisit if we agree other proposal based on neighbour cell coverage) TS 36.133: If t-Service is broadcasted and applicable, UE shall be able to detect, measure, and evaluate neighbour cells before t-Service is reached, regardless of the rules currently limiting the UE measurement activities, and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation. This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of t-Service is started to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger. Ttrigger = max(Tdetect,NB_Intra_NC , Pcarrier * Tdetect,NB_Inter_NC),

Therefore, it is our understanding that:

Measurement relaxation does not apply within T-trigger when t-service is broadcasted and applicable.

Collecting serving cell measurements before t-service will not lead to any change of action by the UE regardless whether the measurements satisfy or not the criterion S. Those measurements will only lead to additional expenditure of energy by the UE. Likewise, collecting neighbor cell measurements from cells belonging to the same satellite (likely to disappear) may also be only a waste of energy.  Because of this, and assuming there is already an automatic trigger for neighbor cell measurements (Ttrigger):

After Ttrigger UE is allowed to skip serving cell measurements and measurements of neighbor cells associated to the same serving satellite.

The main advantage of this solution is that for the cases where the UE has to measure cells on a neighbor satellite, if the UE can skip the measurements on the serving cell the K_satellite factor may be reduced from 2 to 1, saving UE energy and speeding up the reaction time of the UE.

The trade-off imposed by Option 2 in [4] was to potentially trigger a faster cell reselection procedure closer to t-service at the cost of expenditure of UE energy. We believe that the solution above provides an even better compromise between them both.

Another alternative solution to further enhance the UE reaction time and the cell reselection procedure is to allow the UE to perform cell selection procedure instead of cell reselection procedure immediately before t-service, as the UE already knows the suitable  cell will be gone at t-service.

When the UE has reached measurement relaxation, and t-service is configured, the UE might perform cell selection procedure instead of cell reselection procedure, when the time span to T-service is smaller than k, where k = 2 DRX cycle if not configured with eDRX_IDLE cycle, and k = 1 eDRX cycle if configured with eDRX_IDLE cycle.

Relaxation Factor

Another issue discussed was relative to the relaxation factor to be adopted in GEO and LEO scenarios. As previously discussed in [5],  the measurement relaxation factor is a function of the DRX cycle length and the “relaxation factor” defined by the network: numDRX-CyclesRelaxed. As demonstrated by the tables below [6]:

Table 4.6A.2.1A-1: The relaxation factor N for a UE not configured with eDRX IDLE cycle DRX cycle length [s]; Value. 0.32; Min(n , 8). 0.64; Min(n , 8). 1.28; Min(n , 8). 2.56; Min(n , 4). 5.12; Min(n , 2). 10.24; 1. Note: n is signalled by the network by using numDRX-CycleRelaxed-r15 defined in TS 36.331 [2].. Table 4.6A.2.1A-2: The relaxation factor N for a UE configured with eDRX IDLE cycle DRX cycle length [s]; Value. 2.56 ≤ PTW length [s] < 5.12; 5.12 ≤ PTW length [s] < 7.68; 7.68 ≤ PTW length [s] < 12.8; 12.8 ≤ PTW length [s] < 23.04; 23.04 ≤ PTW length [s]. 0.32; Min(n , 2); Min(n , 4); Min(n , 8); Min(n , 8); Min(n , 8). 0.64; 1; Min(n , 2); Min(n , 4); Min(n , 8); Min(n , 8). 1.28; N/A; 1; Min(n , 2); Min(n , 4); Min(n , 8). 2.56; N/A; N/A; 1; Min(n , 2); Min(n , 4). 5.12; N/A; N/A; N/A; 1; Min(n , 2). 10.24; N/A; N/A; N/A; N/A; 1. Note: n is signalled by the network by using numDRX-CycleRelaxed-r15 defined in TS 36.331 [2]..

So, as the parameter numDRX-CyclesRelaxed can be configured in 4 different values (1, 2, 4 and 8), the proposal to divide the relaxation factor by a factor of 2, does not introduce any new range of relaxation that cannot be already configured by NW, as presented in Table 1.

Table 1. Analysis of the relaxation factor if reduced by a factor of 2

| DRX cycle length [s] | Allowed values for the relaxation factor | Allowed values for the relaxation factor if reduced by a factor of 2 |
| --- | --- | --- |
| 0.32 | 1, 2, 4, 8 | 0.5, 1, 2, 4 |
| 0.64 | 1, 2, 4, 8 | 0.5, 1, 2, 4 |
| 1.28 | 1, 2, 4, 8 | 0.5, 1, 2, 4 |
| 2.56 | 1, 2, 4 | 0.5, 1, 2 |
| 5.12 | 1, 2 | 0.5, 1 |
| 10.24 | 1 | 0.5 |

The values highlighted in yellow on Table 1 can already be configured without the introduction of the scaling factor. Whereas the value in red (0.5) does the exact opposite of “relaxing” the measurements, as it reduces the spacing between measurements, which is not in the spirit of the feature. Therefore, introducing the relaxation factor brings no benefit in terms of configurable relaxation factor, and introduces problems by “unrelaxing” measurements under certain conditions. Similar exercise can be made for the relaxation factor configured with an eDRX cycle.

The network is capable of controlling the interval between measurements by choosing the (e)-DRX cycle length and the parameter numDRX-CyclesRelaxed.

Introducing a scaling factor for the relaxed cell measurements does not introduce any relaxation factor that can’t be already configured today. Moreover, it introduces additional complications by “unrelaxing” the measurements in certain scenarios.

Do not introduce a scaling factor for the relaxation factor for NGSO compared to GSO. The relaxation factor can already be modulated by the RRC parameter numDRX-cyclesRelaxed specified by RAN2.

Discontinuous coverage

Another set of issues discussed last meeting regard “discontinuous coverage” operations, where the UE knows it will be out of coverage based on the parameters provided by SIB31 and SIB32.

#### Issue 3-1: Discontinuous Coverage - clarification - Option 1: RAN4 to agree that in discontinuous coverage scenarios, the UE shall resume measurements when the UE is in expected coverage as defined by SystemInformationBlockType32 - Option 1a: RAN4 to agree that in discontinuous coverage scenarios, the UE shall resume measurements when the UE is in expected in-coverage after UE acquired the SystemInformationBlockType32. #### Issue 3-3: Discontinuous Coverage - revision on paging requirement - Option 1: Do not create additional requirements after t-serviceStart regarding UE paging interruption. - Option 2: Paging requirements apply when UE is in coverage, which is the time when UE has acquired the SI of the target cell and the time is after t-serviceStart-r17 if the UE supports discontinuous coverage and SystemInformationBlockType32 has been received in quasi-Earth fixed scenario.

In issue 3-1, Option 1a is similar to Option 1, however, it presents the drawback that the acquisition of SI by the UE cannot be verified as a requirement and should not be used as the starting point for resuming the UE measurements. Besides, it is not clear if the SIB32 acquisition corresponds to the serving cell or the target cell SIB. If it is the serving cell, it becomes the same option as Option 1a. If it is the target cell, then it becomes a conundrum: how would the UE acquire the SIB32 before resuming measurements?  The “in-coverage” in this context means the point in time where the UE is expected to perform measurements. And this cannot be dependent on the UE have already executed measurements.

RAN4 to agree that in discontinuous coverage scenarios, the UE shall resume measurements when the UE is in expected coverage as defined by the previously acquired SystemInformationBlockType32.

For issue 3-3, it is important to clarify that paging requirements only apply after the UE is camped in a cell. According to TS 36.304 [7]:

- If a UE is in “camped normally” or “camped in any cell” (not applicable for NB-IoT UEs) states, the UE shall monitor the paging channels.

- A UE that is not camped in any cell, is in “any cell selection” state, and there is no requirement for monitoring paging channels.

The UE is only camped in a cell, if that cell has been selected by the UE. Therefore, the UE in discontinuous coverage, cannot be considered camped in a cell. This means that for the duration of the discontinuous coverage gap, paging requirements do not apply to this UE.

Paging requirements only apply for UEs camped in a detected cell.

Do not create additional requirements after t-serviceStart regarding UE paging interruption.

Serving Cell Measurement upon t-service.

Another open issue from the previous meeting that requires to be addressed by RAN4 regards the DRX operation when the UE is configured with t-service.

#### Issue 3-4: NGSO, serving cell measurement upon t-Service-r17 - Option 1: - If the UE is configured with DRX cycle ≥ [1.28] sec, then the UE shall meet the serving cell evaluation requirements defined for DRX cycle of [640] ms starting from at least 2 configured DRX cycles before ‘t-Service-r17’. - If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_IDLE, then the UE shall meet the requirements defined for DRX cycle length of [2.56] s in Table 4.7A.2.1.1-1 starting from at least [2] DRX cycles before ‘t-Service-r17’. - Option 2: Postpone the discussion on enhancements of serving cell measurements to the Rel-18 WI (IoT_NTN_enh).

This problem is very similar to the discussion presented in Section 2.1.1 of the present document. As we mention there, this seems an attempt to provide the UE with more serving cell measurements right before t-service, when they are less needed. Besides the high likelihood of the UE still be in good conditions before t-service, those measurements are known to be soon deprecated. We do not see value to expend UE energy in collecting more measurements than the ones configured by the NW, in special considering a NTN UE in NGSO scenarios will probably be required to perform several cell reselections a day.

Do not change DRX requirements based on t-service configuration.

Conclusion

In this paper we analyzed the remaining open issues on the core discussions for NB-IoT and eMTC operations over NTN.

Observation 1: The main reason to execute serving cell measurements is to trigger neighbor cell measurements.

Observation 2: Measurement relaxation does not apply within T-trigger when t-service is broadcasted and applicable.

Proposal 1: After Ttrigger UE is allowed to skip serving cell measurements and measurements of neighbor cells associated to the same serving satellite.

Proposal 2: When the UE has reached measurement relaxation, and t-service is configured, the UE might perform cell selection procedure instead of cell reselection procedure, when the time span to T-service is smaller than k, where k = 2 DRX cycle if not configured with eDRX_IDLE cycle, and k = 1 eDRX cycle if configured with eDRX_IDLE cycle.

Observation 3: The network is capable of controlling the interval between measurements by choosing the (e)-DRX cycle length and the parameter numDRX-CyclesRelaxed.

Observation 4: Introducing a scaling factor for the relaxed cell measurements does not introduce any relaxation factor that can’t be already configured today. Moreover, it introduces additional complications by “unrelaxing” the measurements in certain scenarios.

Proposal 3: Do not introduce a scaling factor for the relaxation factor for NGSO compared to GSO. The relaxation factor can already be modulated by the RRC parameter numDRX-cyclesRelaxed specified by RAN2.

Proposal 4: RAN4 to agree that in discontinuous coverage scenarios, the UE shall resume measurements when the UE is in expected coverage as defined by the previously acquired SystemInformationBlockType32.

Observation 5: Paging requirements only apply for UEs camped in a detected cell.

Proposal 5: Do not create additional requirements after t-serviceStart regarding UE paging interruption.

Proposal 6: Do not change DRX requirements based on t-service configuration.

- References

- TR 36.763, “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)”

- RP-211601 – NB-IoT/eMTC support for Non-Terrestrial Networks

- RP-220938 – New WID on NB-IoT/eMTC core & performance requirements for NTN

- R4-2306370, WF on NTN NB-IoT/eMTC RRM requirements, Mediatek, RAN WG4 #106-bis-e, electronic meeting

- R4-2302343, Discussion on RRM mobility aspects for IoT over NTN, Nokia, Nokia Shanghai Bell, RAN WG4 #106, Athens, Greece.

- TS 36.133, “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”, 3GPP, Release 18.0.0

- TS 36.304, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode, 3GPP, Release 17.3.0