3GPP TSG-RAN4 Meeting #107	R4-2308359

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7217 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on reference configurations for Cat-M1 in Satellite Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is missing the reference configuration for Cat-M1 UEs with channel bandwidth of 1.4 MHz |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing the reference configurations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification of test cases for Cat-M1 UEs in Satellite Access cannot be completed according to the agreed setup for the channel bandwidth |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.1.3, A.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Change #1>

### A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs

MPDCCH reference measurement channels in this section can be used in tests for Cat-M2 UEs.

#### A.3.1.3.1	FDD in CEModeA

Table A.3.1.3.1-1: MPDCCH Reference Channel for Cat-M1 FDD UEs in CEModeA

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MPDCCH Reference channel | - | R.16 FDD | R.17 FDD | R.24 FDD | R.25 FDD | R.46 FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 | 1.4 |
| Number of transmitter antennas | - | 1 | 2 | 1 | 2 | 1 |
| DCI Format | - | 6-1A | 6-1A | 6-1A | 6-1A | 6-1A |
| Transmission Type | - | Distributed | Distributed | Distributed | Distributed | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 4 | 4 | 4 | 4 | [4] |
| Aggregation level | ECCE | 16 | 16 | 16 | 16 | [16] |
| Maximum number of repetitions | - | 8 | 8 | 8 | 8 | 8 |
| Frequency hopping | - | ON | ON | ON | ON | OFF |
| Number of narrowbands | - | 2 | 2 | 2 | 2 | 1 |
| MPDCCH Narrowband | - | 7th | 7th | 4th | 4th | - |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 | - |
| MPDCCH start subframe | subframes | 1 | 1 | 1 | 1 | 1 |
| MPDCCH start symbol | symbols | 2 | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 4 | 4 | 4 | 4 | 4 |
| Payload (without CRC) | Bits | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| Cell ID | - | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: Payload size shall depend upon the test configuration. Note 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |  |

#### A.3.1.3.2	HD-FDD in CEModeA

Table A.3.1.3.2-1: MPDCCH Reference Channel for Cat-M1 HD-FDD UEs in CEModeA

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MPDCCH Reference channel | - | R.6 HD-FDD | R.7 HD-FDD | R.14 HD-FDD | R.15 HD-FDD | R.47 HD-FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 | 1.4 |
| Number of transmitter antennas | - | 1 | 2 | 1 | 2 | 1 |
| DCI Format | - | 6-1A | 6-1A | 6-1A | 6-1A | 6-1A |
| Transmission Type | - | Distributed | Distributed | Distributed | Distributed | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 4 | 4 | 4 | 4 | [4] |
| Aggregation level | ECCE | 16 | 16 | 16 | 16 | [16] |
| Maximum number of repetitions | - | 8 | 8 | 8 | 8 | 8 |
| Frequency hopping | - | ON | ON | ON | ON | OFF |
| Number of narrowbands | - | 2 | 2 | 2 | 2 | 1 |
| MPDCCH Narrowband | - | 7th | 7th | 4th | 4th | - |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 | - |
| MPDCCH start subframe | subframes | 1 | 1 | 1 | 1 | 1 |
| MPDCCH start symbol | symbols | 2 | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 4 | 4 | 4 | 4 | - |
| Payload (without CRC) | Bits | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| Cell ID | - | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: Payload size shall depend upon the test configuration. Note 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |  |

#### A.3.1.3.3	TDD in CEModeA

Table A.3.1.3.3-1: MPDCCH Reference Channel for Cat-M1 TDD UEs in CEModeA

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| MPDCCH Reference channel | - | R.14 TDD | R.15 TDD |
| Carrier bandwidth | MHz | 10 | 10 |
| Number of transmitter antennas | - | 1 | 2 |
| DCI Format | - | 6-1A | 6-1A |
| Transmission Type | - | Distributed | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 4 | 4 |
| Aggregation level | ECCE | 16 | 16 |
| Maximum number of repetitions | - | 8 | 8 |
| Frequency hopping | - | ON | ON |
| Number of narrowbands | - | 2 | 2 |
| MPDCCH Narrowband | - | 7th | 7th |
| Frequency HoppingOffset | narrowbands | 7 | 7 |
| MPDCCH start subframe | subframes | 1 | 1 |
| MPDCCH start symbol | symbols | 2 | 2 |
| Frequency hopping interval | subframes | 10 | 10 |
| Payload (without CRC) | Bits | Note 1 | Note 1 |
| Cell ID | - | Note 2 | Note 2 |
| Note 1: Payload size shall depend upon the test configuration. Note 2: Cell ID shall depend upon the test configuration. |  |  |  |

#### A.3.1.3.4	FDD in CEModeB

Table A.3.1.3.4-1: MPDCCH Reference Channel for Cat-M1 FDD UEs in CEModeB

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| MPDCCH Reference channel | - | R.18 FDD | R.19 FDD | R.26 FDD | R.27 FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 |
| Number of transmitter antennas | - | 1 | 2 | 1 | 2 |
| DCI Format | - | 6-1B | 6-1B | 6-1B | 6-1B |
| Transmission Type | - | Distributed | Distributed | Distributed | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 6 | 6 | 6 | 6 |
| Aggregation level | ECCE | 24 | 24 | 24 | 24 |
| Maximum number of repetitions | - | 128 | 128 | 128 | 128 |
| Frequency hopping | - | ON | ON | ON | ON |
| Number of narrowbands | - | 2 | 2 | 2 | 2 |
| MPDCCH Narrowband | - | 7th | 7th | 4th | 4th |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 |
| MPDCCH start subframe | subframes | 1 | 1 | 1 | 1 |
| MPDCCH start symbol | symbols | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 8 | 8 | 8 | 8 |
| Payload (without CRC) | Bits | 18 | 18 | 17 | 17 |
| Cell ID | - | Note 1 | Note 1 | Note 1 | Note 1 |
| Note 1: Cell ID shall depend upon the test configuration. |  |  |  |  |  |

#### A.3.1.3.5	HD-FDD in CEModeB

Table A.3.1.3.5-1: MPDCCH Reference Channel for Cat-M1 HD-FDD UEs in CEModeB

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| MPDCCH Reference channel | - | R.8 HD-FDD | R.9 HD-FDD | R.16 HD-FDD | R.17 HD-FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 |
| Number of transmitter antennas | - | 1 | 2 | 1 | 2 |
| DCI Format | - | 6-1B | 6-1B | 6-1B | 6-1B |
| Transmission Type | - | Distributed | Distributed | Distributed | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 6 | 6 | 6 | 6 |
| Aggregation level | ECCE | 24 | 24 | 24 | 24 |
| Maximum number of repetitions | - | 128 | 128 | 128 | 128 |
| Frequency hopping | - | ON | ON | ON | ON |
| Number of narrowbands | - | 2 | 2 | 2 | 2 |
| MPDCCH Narrowband | - | 7th | 7th | 4th | 4th |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 |
| MPDCCH start subframe | subframes | 1 | 1 | 1 | 1 |
| MPDCCH start symbol | symbols | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 8 | 8 | 8 | 8 |
| Payload (without CRC) | Bits | 18 | 18 | 17 | 17 |
| Cell ID | - | Note 1 | Note 1 | Note 1 | Note 1 |
| Note 1: Cell ID shall depend upon the test configuration. |  |  |  |  |  |

#### A.3.1.3.6	TDD in CEModeB

Table A.3.1.3.6-1: MPDCCH Reference Channel for Cat-M1 TDD UEs in CEModeB

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| MPDCCH Reference channel | - | R.16 TDD | R.17 TDD |
| Carrier bandwidth | MHz | 10 | 10 |
| Number of transmitter antennas | - | 1 | 2 |
| DCI Format | - | 6-1B | 6-1B |
| Transmission Type | - | Distributed | Distributed |
| Number of PRB pairs per M-PDCCH set | - | 6 | 6 |
| Aggregation level | ECCE | 24 | 24 |
| Maximum number of repetitions | - | 128 | 128 |
| Frequency hopping | - | ON | ON |
| Number of narrowbands | - | 2 | 2 |
| MPDCCH Narrowband | - | 7th | 7th |
| Frequency HoppingOffset | narrowbands | 7 | 7 |
| MPDCCH start subframe | subframes | 1 | 1 |
| MPDCCH start symbol | symbols | 2 | 2 |
| Frequency hopping interval | subframes | 20 | 20 |
| Payload (without CRC) | Bits | 18 | 18 |
| Cell ID | - | Note 1 | Note 1 |
| Note 1: Cell ID shall depend upon the test configuration. |  |  |  |

### A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs

#### A.3.1.4.1	FDD in CEModeA

Table A.3.1.4.1-1: PDSCH Reference Channel for Cat-M1 FDD in CEModeA

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH Reference channel |  | R.20 FDD | R.21 FDD | R.28 FDD | R.29 FDD | R.32 FDD | R.33 FDD | R.48 FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 | 10 | 10 | 1.4 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 2 | 1 | 2 | 1 |
| Allocated resource blocksNote1 | PRBs | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated subframes per Radio Frame | subframes | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |  |  |  |
| Sub-Frames 0 ~ 9 |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Maximum number of repetitions |  | 16 | 16 | 16 | 16 | 1 | 1 | 16 |
| Frequency hopping |  | ON | ON | ON | ON | ON | ON | OFF |
| Number of narrowbands for frequency hopping | narrowbands | 2 | 2 | 2 | 2 | 2 | 2 | 1 |
| PDSCH Narrowband |  | 2nd | 2nd | 2nd | 2nd | 2nd | 2nd | - |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 | 7 | 7 | - |
| PDSCH start symbol | symbols | 2 | 2 | 2 | 2 | 2 | 2 | 1 |
| Frequency hopping interval | subframes | 4 | 4 | 4 | 4 | 4 | 4 | - |
| Cell ID |  | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |  |  |  |

Table A.3.1.4.2-1: PDSCH Reference Channel for Cat-M1 HD-FDD in CEModeA

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH Reference channel |  | R.10 HD-FDD | R.11 HD-FDD | R.18 HD-FDD | R.19 HD-FDD | R.24 HD-FDD | R.25 HD-FDD | R.49 HD-FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 | 10 | 10 | 1.4 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 2 | 1 | 2 | 1 |
| Allocated resource blocksNote1 | PRBs | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated subframes per Radio Frame | subframes | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 | 32 | 32 | 32 | 32 | 32 | 32 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |  |  |  |
| Sub-Frames 0 ~ 9 |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Maximum number of repetitions |  | 16 | 16 | 16 | 16 | 1 | 1 | 16 |
| Frequency hopping |  | ON | ON | ON | ON | ON | ON | OFF |
| Number of narrowbands for frequency hopping | narrowbands | 2 | 2 | 2 | 2 | 2 | 2 | 1 |
| PDSCH Narrowband |  | 1st | 1st | 1st | 1st | 1st | 1st | - |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 | 7 | 7 | - |
| PDSCH start symbol | symbols | 2 | 2 | 2 | 2 | 2 | 2 | 1 |
| Frequency hopping interval | subframes | 4 | 4 | 4 | 4 | 4 | 4 | - |
| Cell ID |  | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |  |  |  |

#### A.3.1.4.3	TDD in CEModeA

Table A.3.1.4.3-1: PDSCH Reference Channel for Cat-M1 TDD in CEModeA

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| PDSCH Reference channel |  | R.16 TDD | R.17 TDD |  |
| Carrier bandwidth | MHz | 10 | 10 |  |
| Number of transmitter antennas |  | 1 | 2 |  |
| Allocated resource blocksNote1 | PRBs | 2 | 2 |  |
| Allocated subframes per Radio Frame |  | 4 | 4 |  |
| TDD Uplink-Downlink Configuration |  | 0 | 0 |  |
| TDD Special Subframe Configuration |  | 6 | 6 |  |
| Modulation |  | QPSK | QPSK |  |
| Target coding rate |  | 1/3 | 1/3 |  |
| Information Bit Payload |  |  |  |  |
| All Sub-Frames except 1, 6 (TDD) | Bits | 32 | 32 |  |
| All Sub-Frames except 1, 6 (TDD DwPTS) | - | 0 | 0 |  |
| Number of Code Blocks per Sub-Frame |  | 1 | 1 |  |
| All Sub-Frames except 1, 6 (TDD) |  | 1 | 1 |  |
| All Sub-Frames except 1, 6 (TDD DwPTS) |  | 0 | 0 |  |
| Maximum number of repetitions |  | 16 | 16 |  |
| Frequency hopping |  | ON | ON |  |
| Number of narrowbands for frequency hopping |  | 2 | 2 |  |
| PDSCH Narrowband |  | 2nd | 2nd |  |
| Frequency HoppingOffset | narrowbands | 7 | 7 |  |
| PDSCH start symbol | symbols | 2 | 2 |  |
| Frequency hopping interval | subframes | 10 | 10 |  |
| Cell ID |  | Note 2 | Note 2 |  |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Cell ID shall depend upon the test configuration. |  |  |  |

#### A.3.1.4.4	FDD in CEModeB

Table A.3.1.4.4-1: PDSCH Reference Channel for Cat-M1 FDD in CEModeB

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PDSCH Reference channel |  | R.22 FDD | R.23 FDD | R.30 FDD | R.31 FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 2 |
| Allocated resource blocksNote1 |  | 2 | 2 | 2 | 2 |
| Allocated subframes per Radio Frame | - | 10 | 10 | 10 | 10 |
| Modulation | - | QPSK | QPSK | QPSK | QPSK |
| Target coding rate | - | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 | 32 | 32 | 32 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |
| Sub-Frames 0 ~ 9 | - | 1 | 1 | 1 | 1 |
| Maximum number of repetitions | - | 192 | 192 | 192 | 192 |
| Frequency hopping | - | ON | ON | ON | ON |
| Number of narrowbands for frequency hopping | narrowbands | 2 | 2 | 2 | 2 |
| PDSCH Narrowband | - | 2nd | 2nd | 2nd | 2nd |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 |
| PDSCH start symbol | symbols | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 8 | 8 | 8 | 8 |
| Cell ID |  | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |

#### A.3.1.4.5	 HD-FDD in CEModeB

Table A.3.1.4.5-1: PDSCH Reference Channel for Cat-M1 HD-FDD in CEModeB

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PDSCH Reference channel |  | R.12 HD-FDD | R.13 HD-FDD | R.20 HD-FDD | R.21 HD-FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 2 |
| Allocated resource blocksNote1 | PRBs | 2 | 2 | 2 | 2 |
| Allocated subframes per Radio Frame | subframes | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 | 32 | 32 | 32 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |
| Sub-Frames 0 ~ 9 |  | 1 | 1 | 1 | 1 |
| Maximum number of repetitions |  | 192 | 192 | 192 | 192 |
| Frequency hopping |  | ON | ON | ON | ON |
| Number of narrowbands for frequency hopping | narrowbands | 2 | 2 | 2 | 2 |
| PDSCH Narrowband |  | 1st | 1st | 1st | 1st |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 |
| PDSCH start symbol | symbols | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 8 | 8 | 8 | 8 |
| Cell ID |  | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |

#### A.3.1.4.6	TDD in CEModeB

Table A.3.1.4.6-1: PDSCH Reference Channel for Cat-M1 TDD in CEModeB

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| PDSCH Reference channel |  | R.18 TDD | R.19 TDD |  |
| Carrier bandwidth | MHz | 10 | 10 |  |
| Number of transmitter antennas |  | 1 | 2 |  |
| Allocated resource blocksNote1 | PRBs | 2 | 2 |  |
| Allocated subframes per Radio Frame |  | 4 | 4 |  |
| TDD Uplink-Downlink Configuration |  | 0 | 0 |  |
| TDD Special Subframe Configuration |  | 6 | 6 |  |
| Modulation |  | QPSK | QPSK |  |
| Target coding rate |  | 1/3 | 1/3 |  |
| Information Bit Payload |  |  |  |  |
| All Sub-Frames except 1, 6 (TDD) | Bits | 32 | 32 |  |
| All Sub-Frames except 1, 6 (TDD DwPTS) | - | 0 | 0 |  |
| Number of Code Blocks per Sub-Frame |  | 1 | 1 |  |
| All Sub-Frames except 1, 6 (TDD) |  | 1 | 1 |  |
| All Sub-Frames except 1, 6 (TDD DwPTS) |  | 0 | 0 |  |
| Maximum number of repetitions |  | 192 | 192 |  |
| Frequency hopping |  | ON | ON |  |
| Number of narrowbands for frequency hopping |  | 2 | 2 |  |
| PDSCH Narrowband |  | 2nd | 2nd |  |
| Frequency HoppingOffset | narrowbands | 7 | 7 |  |
| PDSCH start symbol | symbols | 2 | 2 |  |
| Frequency hopping interval | subframes | 20 | 20 |  |
| Cell ID |  | Note 2 | Note 2 |  |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Cell ID shall depend upon the test configuration. |  |  |  |

<End Of Changes>