3GPP TSG-RAN4 Meeting #107	R4-2308362

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on identification of LTE aerial UEs

Agenda item:	9.9.2

Document for:	Discussion

Introduction

The LTE UAV WI, as presented in [1], includes the following objectives with RAN4 impact highlighted:

- Specify the support for UAV identification broadcast (BRID) in LTE PC5. Support of DAA using the same framework as BRID without DAA specific enhancements can be considered [RAN2]. Note: UAV use of LTE PC5 is to be used only in designated bands as defined in regulation for BRID/DAA use. - Based on the technical conditions defined for aerial UE usage in ECC Decision (22)07, study and specify the necessary UE types and additional OOBE requirements for aerial UEs in 1710-1785 MHz, 2500-2570 MHz and 2570-2620 MHz [RAN4]. Note: In other frequency bands, OOBE limits applicable to terrestrial UE remain unchanged for aerial UE Note2: Applicability of power classes for aerial UE may need to be addressed in RAN4 Note3: RAN4 to identify the supported bands for aerial UE impacted by above information

It was identified that the issue above does impact LTE specifications, as they also cover the same bands impacted by ECC decision and also support UAV deployments and aerial features, including aerial subscription. This when defining the NR WI on UAV Enh [7] which is progressed in parallel to this WI.

Discussion

The discussion for supporting the additional ECC requirements for out of band emissions is split into two WI, one covering NR [7] and the other covering LTE [1] specifications. However, most of the work might be reused, as the list of bands impacted is the same and a similar approach is to be used.

The workplan is to be concentrated in the NR WI, the present WI is to be used for LTE related CRs and to cover the points where LTE requires a different approach due to the specificities of the TS 36.101 compared to TS 38.101.

Defining aerial UE

As we discuss in our T-Doc R4-2308349, the identification of the aerial UE should cover a UE that has an aerial subscription type and has performed authentication of the aerial subscription to access aerial services. This UE must be capable to access aerial features. The same definition shall apply for LTE UEs.

Adopt same definition for aerial UEs in LTE and NR.

NS Signalling

The framework for NS signalling was updated in NR compared to LTE specifications for bit-efficiency. In LTE, the original framework was more bit-consuming, and the original 32 NS values available were soon occupied for operations in the different bands. This led LTE to further extend the range of NS signaling:

#### TS 36.331: #### – AdditionalSpectrumEmission If an extension is signalled using the extended value range (as defined by IE AdditionalSpectrumEmission-v10l0), the corresponding original field, using the value range as defined by IE AdditionalSpectrumEmission i.e. without suffix) shall be set to value 32, if signalled. UE supporting an LTE band assigned NS values larger than 32 as defined in TS 36.101 [42], clause 6.2.4, needs to support extension signaling (as defined by IE AdditionalSpectrumEmission-v10l0). AdditionalSpectrumEmission information element -- ASN1START AdditionalSpectrumEmission ::= INTEGER (1..32) AdditionalSpectrumEmission-v10l0 ::= INTEGER (33..288) -- ASN1STOP

Currently the maximum value for NS signalling is 63. Therefore, if NS signalling is introduced in LTE, UEs supporting aerial feature AND bands 3, 7 and 38 shall support the extended range of signalling.

For aerial UEs supporting band 3, 7 and 38, it is required to support the parameter AdditionalSpectrumEmission-v10l0 in TS 36.331

This won’t be a problem for Band 38, as it is already included in the range of extended signalling with the value NS_44. However, NS_44 cannot be reused for this purpose, as the signalling for further restriction on OOBE will only be applied on the ECC region, not necessarily overlapping with other regions broadcasting currently NS_44.

Add the requirements for OOBE under the specific subclause using the aerial suffix, introducing NS_64 for bands 7 and 38 and introducing NS_65 for band 3.

Impact on V2X.

- The only band specified for V2X usage in TS 36.101 is  band 47, which operates in the 5GHz region. Therefore, no V2X impact is identified for operation in the PC5 interface for BRID and DAA in LTE.

Differently from NR, no V2X impact needs to be addressed for UAV operation in LTE.

Conclusion

This paper has discussed the differences expected in specification for implementing the further OOBE requirements for LTE and NR aerial UEs. The following proposals and observations were presented:

Observation 1: The workplan is to be concentrated in the NR WI, the present WI is to be used for LTE related CRs and to cover the points where LTE requires a different approach due to the specificities of the TS 36.101 compared to TS 38.101.

Proposal 1: Adopt same definition for aerial UEs in LTE and NR.

Proposal 2: For aerial UEs supporting band 3, 7 and 38, it is required to support the parameter AdditionalSpectrumEmission-v10l0 in TS 36.331

Proposal 3: Add the requirements for OOBE under the specific subclause using the aerial suffix, introducing NS_64 for bands 7 and 38 and introducing NS_65 for band 3.

The only band specified for V2X usage in TS 36.101 is  band 47, which operates in the 5GHz region. Therefore, no V2X impact is identified for operation in the PC5 interface for BRID and DAA in LTE.

Observation 2: Differently from NR, no V2X impact needs to be addressed for UAV operation in LTE.

- References

- RP-230783, WI on Enhanced LTE Support for UAV

- RP-230025/R4-2304033, Reply to LS to 3GPP on ECC request for standardisation support related to ECC Decision (22)07 on “harmonised framework on aerial UE usage in MFCN harmonised bands”, ETSI TC MSG/TFES

- S2-2303302, “LS Reply on ECC request for standardisation support related to ECC Decision (22)07 on “harmonised framework on aerial UE usage in MFCN harmonised bands”, SA2 Meeting #155, SA2 Meeting #155

- TS 23.356, “Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2”, 3GPP, Release 18.0.0

- TS 36.306, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities”, 3GPP, Release 17.3.0.

- R4-2306641, “WF on NR Support for UAV”, Nokia, Electronic Meeting, 17 April –26 April, 2023

- RP-230782, Revised WID: NR Support for UAV (Uncrewed Aerial Vehicles)