3GPP TSG-RAN WG4 Meeting #107	R4-2308382

Incheon, South Korea, May 22nd – 26th, 2023

Agenda Item:	8.15.3

Source:	MediaTek Inc.

Title: 	On UE RF requirements for FR1 sub-5MHz channel bandwidth

Document for:	Discussion

# Introduction

Regarding UE RF requirements for FR1 sub-5MHz channel bandwidth operation, RAN4#106bis-e has almost finalized all of the impacted requirements [1]. And for ACS with 15 PRBs for 3MHz channel bandwidth, the following agreement has been reached [1]:

However, there is one potential open issue on ACS where only 12 PRBs are available in a special use case for band n100 when it is on its way to migrate gradually from GSM-R to FRMCS [3], where ACS could potentially be impacted for this particular case.

# Discussion

As stated in [3], the following table indicates the maximum number of RBs for NR with different required GSM-R channels:

For the case of 12 RBs, the actual deployment can be illustrated as the following figure:

Fig. 1, Deployment use case in band n100 with 12 RBs available

Fig. 1 shows apparently that GSM-R channel #1 and half of GSM-R channel #2 will leak into the 3MHz channel filter. In this case, ACS requirement is impacted. However, according to the latest WID [2], it is not clear whether or not the use case shown above is within the scope:

- Specify necessary RAN4 requirements to support deploying NR in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN4], including in bands n100, n106, n26, n28 and n85: - Specify system parameters (including channel and sync rasters) for the associated dedicated spectrum. - Minimize impact on RF requirements: - Reuse 5 MHz channel bandwidth at least for FRMCS use case (assuming co-located NR and GSM-R with same operator). - Specify the required RF requirements for optional 3 MHz channel bandwidth in bands n100, n106, n26, n28 and n85.

If it is confirmed within the scope (RAN plenary intervention is required), then RAN4 needs to further study and specify the applicable ACS for the use case, but before that, RAN4 does not need to consider specifying RF requirements for the special use case for band n100.

Proposal: RAN4 not to consider specifying RF requirements for the special use case in band n100 before RAN plenary confirms it within the WID scope.

# Conclusion

In this contribution we have the following proposal on UE RF requirements for FR1 sub-5MHz channel bandwidth:

Proposal: RAN4 not to consider specifying RF requirements for the special use case in band n100 before RAN plenary confirms it within the WID scope.

# References

- R4-2306616, WF on UE RF requirements, Nokia

- RP-230186, Revised WID: NR support for dedicated spectrum less than 5MHz for FR1, Nokia, Nokia Shanghai Bell, Anterix, T-Mobile USA

- RP-230189, UIC response to LS from RAN1 on transmission bandwidths for NR on dedicated spectrum less than 5 MHz, Union International Chemins de Fer