3GPP TSG-RAN WG4 Meeting #107	R4-2308383

Incheon, South Korea, May 22nd – 26th, 2023

Agenda Item:	8.23.3.4

Source:	MediaTek Inc.

Title: 	On switching time for DL PRS or UL SRS frequency hopping for RedCap UEs

Document for:	Discussion

# Introduction

In RAN4#106bis-e, there is some progress on RedCap UEs positioning [1]:

## Sub-topic 2-3: RedCap UE positioning ### Issue 2-3a: UL SRS Tx frequency hopping Agreement: - For UL SRS Tx frequency hopping, use the switching time of {70us, 140us} for FR1 as the starting point - SRS Tx frequency hopping range can be up to 100MHz - Which specific value for frequency hopping is applied depends on UE capability, if multiple values are agreed Agreement: - For UL SRS Tx frequency hopping, use the switching time of {35us, 70us, 140us} for FR2 as the starting point - SRS Tx frequency hopping range can be up to 400MHz - Which specific value for frequency hopping is applied depends on UE capability, if multiple values are agreed ### Issue 2-3b: DL PRS Rx frequency hopping Agreement: - For DL PRS Rx frequency hopping, use the switching time of {70us, 140us} for FR1 as the starting point - PRS Rx frequency hopping range can be up to 100MHz - Which specific value for frequency hopping is applied depends on UE capability, if multiple values are agreed Agreement: - For DL PRS Rx frequency hopping, use the switching time of {35us, 70us, 140us} for FR2 as the starting point - PRS Rx frequency hopping range can be up to 400MHz - Which specific value for frequency hopping is applied depends on UE capability, if multiple values are agreed

And an LS was also sent to RAN1 with the above agreements [2]. However, to facilitate implementation flexibility, for FR1 we propose to add another value of 210us to the switching time sets for both UL SRS and DL PRS frequency hopping.

# Discussion

The maximum channel bandwidth operating at a time in a RedCap UEs is 20MHz. In order to achieve a higher positioning accuracy, a wider bandwidth for positioning can be configured by frequency hopping, as illustrated in Fig. 1 as one example.

Fig. 1, RedCap positioning by frequency hopping

For a frequency hopping, a RedCap UE may need to perform:

- RF retuning to the new frequency

- Hardware preparation

Usually the current set {70us, 140us} may already cover the need of RF retuning as discussed in RAN4, however, in some implementation, hardware preparation time may be more than 140us due to the restriction or design target for the cost and power consumption, thus a higher value, e.g., 210us, could be added into the set to accommodate such implementation flexibility demands from standard perspective, and an LS should be sent to RAN1 to inform the group on the update.

Proposal 1: RAN4 to add 210us to the switching time sets for both UL SRS and DL PRS frequency hopping to accommodate implementation flexibility demands from standard perspective.

Proposal 2: RAN4 to send an LS to RAN1 on the value addition to the switching time sets.

# Conclusion

In this contribution we have the following proposals for RedCap positioning by frequency hopping:

Proposal 1: RAN4 to add 210us to the switching time sets for both UL SRS and DL PRS frequency hopping to accommodate implementation flexibility demands from standard perspective.

Proposal 2: RAN4 to send an LS to RAN1 on the value addition to the switching time sets.

# References

- R4-2306620,	WF on UE RF aspects for Rel-18 NR positioning, Intel.

- R4-2306659,	LS reply for the switching time for RedCap UE positioning, Ericsson.