3GPP TSG-RAN WG4 Meeting #107	R4-2308393

Incheon, South Korea, May 22nd – 26th, 2023

Agenda Item:	9.6.3

Source:	MediaTek Inc.

Title: 	Further discussion on UE RF requirements for L+S IoT NTN band

Document for:	Discussion

# Introduction

RAN4#106bis-e started to discuss UE RF requirements for L+S IoT NTN band, and the following agreements have been reached [1]:

Issue 3-1-1: UE maximum output power Option 1: For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the UE maximum output power for category M1 and category NB1 and NB2 can be specified in Tables below, respectively. Table: UE Power Class for category M1 EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB); Class 5 (dBm); Tolerance (dB). [254]; 23; +/-2. NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance.. Table: UE Power Class for category NB1 and NB2 EUTRA band; Class 3 (dBm); Tolerance (dB); Class 5 (dBm); Tolerance (dB). [254]; 23; +/-2. WF: - Agree Option 1. The inclusion of PC5 is under study. Issue 3-1-2: UE maximum output power reduction - Option 1: Reuse the 36.102 MPR requirements for category M1 and NB1/NB2 as the starting point WF: - Agree Option 1. Issue 3-2-1: In-band blocking - Proposals: - Option 1: Reuse the 36.102 in-band blocking requirements Agreement in the 1st round Agree Option1. Issue 3-2-2: Narrow-band blocking - Proposals: - Option 1: Reuse the 36.102 narrow-band blocking requirements Agreement in the 1st round Agree Option1. Issue 3-2-3: RX spurious emission - Proposals: - Option 1: Reuse the 36.102 spurious emission requirements Agreement in the 1st round Agree Option1.

And in this contribution, we further discuss other UE RF requirements, in particular, several Tx and Rx requirements which do not depend on any specific band.

# Discussion

## 2.1 Transmitter characteristics

The table below illustrates an overview on the Tx RF requirements and the progress until now.

| (sub) Section in TS 36.102 | Discussed or not for L+S IoT NTN band |
| --- | --- |
| 6.2A.1 Maximum output power for Cat. M1 | Yes, agreed |
| 6.2A2 MPR for Cat. M1 | Yes, re-using 36.102 as a starting point |
| 6.2A.3 A-MPR for Cat. M1 | No |
| 6.2A.4 Configured transmitted power for Cat. M1 | No |
| 6.2B.1 Maximum output power for Cat. NB1/NB2 | Yes, agreed |
| 6.2B2 MPR for Cat. NB1/NB2 | Yes, re-using 36.102 as a starting point |
| 6.2B.3 A-MPR for Cat. NB1/NB2 | No |
| 6.2B.4 Configured transmitted power for Cat. NB1/NB2 | No |
| 6.3A.1 Minimum output power for Cat. M1 | No |
| 6.3A2 Transmit OFF power for Cat. M1 | No |
| 6.3A.3 ON/OFF time mask for Cat. M1 | No |
| 6.3A.4 Power control for Cat. M1 | No |
| 6.3B.1 Minimum output power for Cat. NB1/NB2 | No |
| 6.3B2 Transmit OFF power for Cat. NB1/NB2 | No |
| 6.3B.3 ON/OFF time mask for Cat. NB1/NB2 | No |
| 6.3B.4 Power control for Cat. NB1/NB2 | No |
| 6.4A.1 Frequency error for UE category M1 | No |
| 6.4A.2 Transmit modulation quality for category M1 | No |
| 6.4B.1 Frequency error for UE category NB1 and NB2 | No |
| 6.4B.2 Transmit modulation quality for Category NB1 and NB2 | No |
| 6.5A.1 General | No |
| 6.5A.2 Occupied bandwidth for category M1 | No |
| 6.5A.3 Out of band emission for category M1 | No |
| 6.5A.4 Spurious emission for category M1 | No |
| 6.5B.1 General | No |
| 6.5B.2 Occupied bandwidth for category NB1 and NB2 | No |
| 6.5B.3 Out of band emission for category NB1 and NB2 | No |
| 6.5B.4 Spurious emission for category NB1 and NB2 | No |
| 6.6A Transmit intermodulation for category M1 | No |
| 6.6B Transmit intermodulation for category NB1 and NB2 | No |

Minimum output power

For minimum output power, current TS 36.102 reuses values specified in Table 6.3.2.1-1 of TS 36.101 for category M1, and the requirements in clause 6.3.2F of TS 36.101 for category NB1 and NB2. These values can be applicable for L+S IoT NTN bands as well.

Proposal 1: For minimum output power, reuse the current TS 36.102 requirements for L+S IoT NTN bands.

Transmit OFF power

For transmit OFF power, the current TS 36.102 reuses values specified in Table 6.3.3.1-1 of TS 36.101 for categoary M1, and the requirements in clause 6.3.3F of TS 36.101 for category NB1 and NB2. These values can also be applicable for L+S IoT NTN bands.

Proposal 2: For transmit OFF power, reuse the current TS 36.102 requirements for L+S IoT NTN bands.

ON/OFF time mask

For ON/OFF time mask, the current TS 36.102 reuses time mask defined in clause 6.3.4 of TS 36.101 for category M1, and the requirements in clause 6.3.4F of TS 36.101 for category NB1 and NB2, which can be applicable for L+S IoT NTN bands.

Proposal 3: For ON/OFF time mask, reuse the current TS 36.102 requirements for L+S IoT NTN bands.

## 2.2 Receiver characteristics

The table below illustrates an overview on the Rx RF requirements and the progress until now.

| Section in TS 36.102 | Discussed or not for L+S IoT NTN band |
| --- | --- |
| 7.3A Reference sensitivity power level for UE category M1 | No |
| 7.3B Reference sensitivity power level for UE category NB1 and NB2 | No |
| 7.4A Maximum input level for category M1 | No |
| 7.4B Maximum input level for category NB1 and NB2 | No |
| 7.5A Adjacent Channel Selectivity for category M1 | No |
| 7.5B Adjacent Channel Selectivity for category NB1 and NB2 | No |
| 7.6A Blocking characteristics for category M1 | Yes, agree to re-use 36.102 requirements except out-of-band blocking |
| 7.6B Blocking characteristics for category NB1 and NB2 | Yes, agree to re-use 36.102 requirements except out-of-band blocking |
| 7.7A Spurious response for category M1 | Yes, agree to re-use 36.102 requirements |
| 7.7B Spurious response for category NB1 and NB2 | Yes, agree to re-use 36.102 requirements |
| 7.8A Intermodulation characteristics for category M1 | No |
| 7.8B Intermodulation characteristics for category NB1 and NB2 | No |

Maximum input level

For maximum input level, -40dBm is specified for both category M1 and NB1/NB2 under 95% maximum throughput principle with different conditions. And it can be applicable for IoT NTN band as well.

Proposal 4: For maximum input level, reuse the current TS 36.102 requirements for L+S IoT NTN bands.

# Conclusion

In this contribution we have the following proposals for UE RF requirements for L+S IoT NTN band:

Proposal 1: For minimum output power, reuse the current TS 36.102 requirements for L+S IoT NTN bands.

Proposal 2: For transmit OFF power, reuse the current TS 36.102 requirements for L+S IoT NTN bands.

Proposal 3: For ON/OFF time mask, reuse the current TS 36.102 requirements for L+S IoT NTN bands.

Proposal 4: For maximum input level, reuse the current TS 36.102 requirements for L+S IoT NTN bands.

# References

- R4-2306579, WF on IoT_NTN_FDD_LS_band, MediaTek Inc.