# 3GPP TSG-RAN WG4 Meeting # 107                               R4-2308402

# Incheon, KR, May 22 – May 26, 2023

# Title: 	Discussion on demodulation for 4Tx of NR_ENDC_RF_FR1_enh2

# Source: 	ZTE Corporation

# Agenda item:	8.5.3.2

# Document for:	Approval

# 1 Introduction

The objectives for 4Tx of further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1) are listed in the following [1]:

Enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices [RAN4]

- Investigate framework and architecture Example bands:

  - TDD bands: n41, n77/n78

  - FDD bands: n1

    - Note 1: the total number of example bands should be limited to 3. n77/n78 are considered as one band during the study.

    - Note 2: other bands to be introduced in the release independent way later on from Rel-18

    - Note 3: specifying requirements for TDD bands has first priority

- Specify the UE RF requirements to support 4Tx

  - First priority: 4x4 UL MIMO

  - Second priority: investigate and if necessary specify TxD requirement to support the same power class in UL MIMO and single antenna port

  - PA configuration assumption:

    - First priority: 4x23dBm

    - Second priority: 2x23dBm + 2x26dBm, 4x26dBm

  - UE power class

    - First priority: PC 1.5

    - Second priority: PC2/PC3, and/or new power class if needed

    - Note 1: PC1.5 is only applicable for TDD bands

  - Note: detailed combinations for 2nd priority PA/UE power class assumptions are to be revisited in RAN#97

- NOTE1: Requirements are specified with phase approach. Objectives with 1st priority are considered first.

- NOTE2: It is assumed the devices are equipped with 4Rx antennas for band n1 requirement definition.

In this contribution, we share our simulation results 4Tx.

# 2	Discussion

## 2.1 BW

The BW for different SCS was reached in last meeting as below:

Issue 1-1-1: Bandwidth selection between 20MHz and 50MHz Agreement: - For 15kHz: - Introduce requirements for 5MHz and 50MHz, FFS on 20MHz - For 30kHz: - Introduce requirements for 10MHz and 100MHz, FFS on 40MHz

Base on the simulation results in our accompany contribution, we can find that the operating SNR gap between 5MHz and 50MHz for 15kHz is not more than 2dB. And the operating SNR gap between 10MHz and 100MHz for 30kHz is not more than 1dB. Therefore the BW of 20MHz for 15kHz SCS and the BW of 40MHz for 30kHz SCS is not necessary.

Proposal 1: To use only 5MHz and 50MHz BW for requirements definition of 15kHz SCS, and only 10MHz and 100MHz for requirements definition of 30kHz SCS.

## 2.2 Applicability rule

It was agreed to consider the following options for 4Tx applicability rule.

Issue 1-1-6: Applicability rule Way forward: - Option 1: Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the 4 Tx antenna tests with low MIMO correlation level shall apply only for the 4 connectors and the highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration. - Option 2: Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the 4 Tx antenna tests with low MIMO correlation level shall apply only for the highest numbers of supported connectors which is larger or equal to 4, and the specific connectors used for testing are based on manufacturer declaration.

For the above two options, option 1 is more preferred as there may be two antenna configurations to be tested, one is 4 connectors, and the other is a number of connectors larger than 4, such as 8 connectors. This description is closer to the application rule of TS 38.141.

Proposal 2: To consider option 1 as the applicability rule of 4Tx.

# 3 Conclusion

In this contribution, we give some conclusion for 4Tx of NR_ENDC_RF_FR1_enh2 :

Proposal 1: To use only 5MHz and 50MHz BW for requirements definition of 15kHz SCS, and only 10MHz and 100MHz for requirements definition of 30kHz SCS.

Proposal 2: To consider option 1 as the applicability rule of 4Tx.

# 4	Reference

[1]  RP-223478 Revised WID Rel-18 NR_ENDC_ RF_FR1_enh2, Huawei, HiSilicon, NTT DOCOMO

[2]  R4-2305890 WF on FR1 4Tx demodulation requirement, Ericsson