# 3GPP TSG-RAN WG4 Meeting # 107                                    R4-xxxxxx

# Incheon, KR, May 22 – May 26, 2023

# Title: 	Discussion on Test parameters and simulation results

# Source: 	ZTE Corporation

# Agenda item:	8.19.1.2

# Document for:	Approval

# 1 Introduction

During the last RAN4 meeting, some conclusions has been reached for simulation assumptions for MU-MIMO. In this contribution, we want to share further considerations on simulation assumptions and results.

# 2 Discussion

## 2.1 Simulation assumptions

Issue 2-1-4: MCS for the target UE

Way forward - Cover MCS 13 for rank 1 and rank 2 for initial simulation - Cover MCS 17 for rank 2 for initial simulation - Further discuss whether to cover MCS 4 for rank 1 in the next meeting

In Rel-17, we considered MCS 4 for rank1 in initial stage. Since we can consider MCS 4 for rank 1.

Proposal 1. To consider MCS4 for rank 1.

## 2.2 Simulation results

2.2.1 Rank2+2(QPSK)

Figure 2-1 Simulation results of Rank2+2 4T4R TDLA30-10 ULA Low MCS 17

| Propagation conditions | PMI selection | Antenna configuration | MMSE-IRC | E-MMSE-IRC |  | R-ML |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SNR (dB) | SNR (dB) | Gain (dB) | SNR (dB) | Gain (dB) |
| TDLA30-10 ULA Low | Orthogonal | 4T4R | 17.6 | 17.3 | 0.3 | 15.2 | 2.4 |

Observation 1. R-ML receiver with QPSK interference shows 2.4dB performance gain.

2.2.2 Rank2+2(64QAM)

Figure 2-2 Simulation results of Rank2+2 4T4R TDLA30-10 ULA Low MCS 17

| Propagation conditions | PMI selection | Antenna configuration | MMSE-IRC | E-MMSE-IRC |  | R-ML |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SNR (dB) | SNR (dB) | Gain (dB) | SNR (dB) | Gain (dB) |
| TDLA30-10 ULA Low | Orthogonal | 4T4R | 17.6 | 17.3 | 0.3 | 16.6 | 1.0 |

Observation 2. R-ML receiver with 64QAM interference shows 1.0 dB performance gain.

2.2.3 Blind detection

(1) Rank 1+1

Figure 2-3 Simulation results of Rank1+1 2T2R TDLC300-100 ULA Medium

| Propagation conditions | PMI selection | Antenna configuration | MMSE-IRC | E-MMSE-IRC |  | R-ML |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SNR (dB) | SNR (dB) | Gain (dB) | SNR (dB) | Gain (dB) |
| TDLC300-100 ULA Medium | Random | 2T2R | 22.4 | 19.8 | 2.6 | 15.2 | 7.2 |

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for MU-MIMO, The conclusions are:

Proposal 1. To consider MCS4 for rank 1.

Observation 1. R-ML receiver with QPSK interference shows 2.4dB performance gain.

Observation 2. R-ML receiver with 64QAM interference shows 1.0 dB performance gain.

# 4	Reference

[1]  R4-2302929, WF on advanced receiver for MU-MIMO scenario, China Telecom.

[2]  R4-2304670, Discussion on MU-MIMO test parameters and simulation results. ZTE Corporation.