# 3GPP TSG-RAN WG4 Meeting # 107                               R4-2308419

# Incheon, KR, May 22 – May 26, 2023

# Title: 	Discussion on ATG UE demodulation

# Source: 	ZTE Corporation

# Agenda item:	8.14.5.2

# Document for:	Approval

# 1 Introduction

In RAN#96e meeting[1], the revised work item on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package.

During the last RAN4 meeting, some conclusions which need further for ATG DL demodulation are listed below[2].

Issue 2-1: Test scope - FFS For PDSCH: - Option 1: Introduce PDSCH requirements with AWGN+Doppler channel model - Option 2: FFS - Decide whether to introduce new PDSCH requirements after channel model and other parameters (e.g., bandwidth, TDD pattern and so on) are agreed. - If no new PDSCH requirements for ATG, further decide which legacy PDSCH requirements shall be applied for ATG UE. - FFS For PDCCH: - Option 1: Introduce PDCCH requirements with AWGN+Doppler channel model - Option 2: No need to introduce additional PDCCH requirements. But the legacy PDCCH mandatory requirements shall be met. - Option 3: FFS - Decide whether to introduce new PDCCH requirements after channel model is agreed. - If no new PDCCH requirements for ATG, whether and which legacy PDCCH requirements shall be applied for ATG UE - For CSI reporting: - Do not define new CSI reporting requirements for ATG UE. - FFS - Option 1: legacy CSI reporting requirements shall be met. - Option 2: FFS whether and which legacy UE CSI reporting requirements shall be applied for ATG UE. Tentative agreement: - Come back in next meeting. Issue 2-2: TDD pattern - Option 1: Define a new TDD pattern 30D4S6U which only applied for ATG scenario. - Option 2: Proceed with typical TDD pattern for requirement, further check the performance if the new TDD pattern is agreed to be introduced - RF, RRM and Demod sessions should keep the consistency on the agreement for the necessary of new TDD pattern for ATG scenario. - Start with FDD requirement configuration if companies cannot get consensus on TDD pattern issue easily - Option 3: If new dedicated TDD pattern is considered, note should be added in the specification that this is pattern is for ATG scenario used only. Issue 2-3: Bandwidth & SCS - For TDD: - Option 1: 40MHz for TDD 30kHz. - Option 2: 100MHz for TDD 30kHz. - For FDD: - Option 1: 20Mhz for FDD 15kHz - Option 2: 10MHz for FDD 15kHz Issue 2-4: Antenna Configuration - 2T2R - FFS: - 2T4R (for UE supporting 4R) - 4T4R (for UE supporting 4R) Issue 2-5: MCS&Rank - Cover rank 1, 16QAM and 64QAM - FFS: Whether to cover 256 QAM - Companies are encouraged to bring simulation results in next meeting. Issue 2-7: koffset Agreements: - koffset value 2ms, the unit and value can be revised after RAN2 confirm whether SIB19 can be reused or not. Issue 2-8: Applicability rule - Option 1: RAN4 to further evaluate applicability rules between ATG and NTN UEs only after the scope of ATG UE demodulation is agreed (QC) - Option 2: Applicability rule can be defined that if UE supporting both NTN and ATG feature has passed the NTN performance requirements, then it can skip ATG cases with the same MCS and rank configurations. (HW) - Option 3: Needed to have the applicability rule. Introducing a new UE feature is preferable so that we can easily define the applicability rule if we are going to define new ATG requirements into the TS38.101-4. (Ericsson) Tentative agreements: - Further discuss the applicability rule after channel model, test scope and test parameters are converged. Issue 2-11: PDCCH test parameters - Option 1: For simulation assumption, take aggregation level 4 and 8, CORESET duration 1 and 2 symbols as the starting point, and do the further selection according to SNR simulation results. - Option 2: FFS

This contribution shares our views for the issues above.

# 2	Discussion

With regard to TDD pattern, 30D4S6U was proposed by companies in previous meeting and one concern about this TDD pattern is the number of HARQ process as 30 consecutive DL slot in one period.

The following is copied from TS 38.331[3] which shows that 32 HARQ process can be supported in Rel-17.

PDSCH-ServingCellConfig ::= SEQUENCE { codeBlockGroupTransmission SetupRelease { PDSCH-CodeBlockGroupTransmission } OPTIONAL, -- Need M xOverhead ENUMERATED { xOh6, xOh12, xOh18 } OPTIONAL, -- Need S nrofHARQ-ProcessesForPDSCH ENUMERATED {n2, n4, n6, n10, n12, n16} OPTIONAL, -- Need S pucch-Cell ServCellIndex OPTIONAL, -- Cond SCellAddOnly ..., [[ maxMIMO-Layers INTEGER (1..8) OPTIONAL, -- Need M processingType2Enabled BOOLEAN OPTIONAL -- Need M ]], [[ pdsch-CodeBlockGroupTransmissionList-r16 SetupRelease { PDSCH-CodeBlockGroupTransmissionList-r16 } OPTIONAL -- Need M ]], [[ downlinkHARQ-FeedbackDisabled-r17 SetupRelease { DownlinkHARQ-FeedbackDisabled-r17 } OPTIONAL, -- Need M nrofHARQ-ProcessesForPDSCH-v1700 ENUMERATED {n32} OPTIONAL -- Need R ]] }

The following is copied from TS 38.212 v17.5.0[4] which describe the field of HARQ process number for DCI 1-1.

HARQ process number – 5 bits if higher layer parameter harq-ProcessNumberSizeDCI-1-1 is configured; otherwise 4 bits

From the above we can find the dynamic scheduling can support 32 HARQ process number.

The following is copied from TS 38.306[5] from which we can find that the maximum number of 32 harq processes is related to UE capability, which is defined on specific band.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| max-HARQ-ProcessNumber-r17 Indicates the maximal supported HARQ process numbers for UL and for DL respectively. For each value of max-HARQ-ProcessNumber-r17, value u16d32 indicates the maximal supported HARQ process number is 16 for UL and 32 for DL, value u32d16 indicates the maximal supported HARQ process number is 32 for UL and 16 for DL, value u32d32 indicates the maximal supported HARQ process number is 32 for UL and 32 for DL. This field is only applicable for bands in Table 5.2.2-1 in TS 38.101-5 [34] and HAPS operation bands in clause 5.2 of TS 38.104 [35]. | Band | No | N/A | N/A |

From the above description we can find that the max-HARQ-ProcessNumber-r17 is supported by NTN and HAPS, if 32 HARQ process is to be used for ATG, the UE capability needs to be introduced. As the UE capability of NTN or HAPS does not consider compatibility with ATG scenarios during standardization, UE capability of ATG needs to be considered, such as updating the content of the above table to enable ATG to support up to 32 HARQ process numbers.

Proposal 1. To consider UE capability of max HARQ number of 32 for ATG CPE to support new TDD pattern.

With regard to MCS, 256QAM is still FFS, and we give the initial simulation results with throughput comparison between MCSs of 64QAM and 256QAM.

Figure 2-1 Throughput comparison between MCSs of 64QAM and 256QAM

From figure 2-1 we can find 256QAM can provide throughput gain when the SNR is equal to or higher than 18dB and the AWGN channel matrix is a 2X2 unit matrix.

Observation 1. Throughput gain can be expected when MCS of 256QAM is considered.

With regard to the BW selection, different options are to be discussed. Table 2-1 compares the operating SNR for 30kHz SCS for 40MHz BW and 100MHz BW.

Table 2-1 comparison between different BW

| MCS | BW(MHz) | Operating SNR(dB) |
| --- | --- | --- |
| 64QAM MCS23 | 40 | 11.3 |
|  | 100 | 11.4 |
| 64QAM MCS24 | 40 | 12.7 |
|  | 100 | 13.3 |
| 256QAM MCS21 | 40 | 16.7 |
|  | 100 | 16.7 |
| 256QAM MCS22 | 40 | 17.3 |
|  | 100 | 17.3 |

From table 2-1 we can find that the operating SNR for 40MHz BW and 100MHz BW is close .

Observation 2. Based on initial simulation results, there is no big gap in operating SNR between 40MHz and 100MHz.

Proposal 2. To consider the following BW for ATG demodulation requirements.

- For TDD:

  - 100MHz for TDD 30kHz.

- For FDD:

  - 20MHz for FDD 15kHz

# 3 Conclusion

In this contribution, we give some conclusions for ATG UE demodulation .

The conclusions are:

Proposal 1. To consider UE capability of max HARQ number of 32 for ATG CPE to support new TDD pattern.

Observation 1. Throughput gain can be expected when MCS of 256QAM is considered.

Observation 2. Based on initial simulation results, there is no big gap in operating SNR between 40MHz and 100MHz.

Proposal 2. To consider the following BW for ATG demodulation requirements.

- For TDD:

  - 100MHz for TDD 30kHz.

- For FDD:

  - 20MHz for FDD 15kHz

# 4	Reference

[1]  RP-221369, Revised WID on Air-to-ground network for NR, CMCC

[2]  R4-2305899, WF for ATG demodulation requirements, CMCC

[3]  3GPP TS 38.331 v17.4.0

[4]  3GPP TS 38.212 v17.5.0

[5]  3GPP TS 38.306 v17.4.0