3GPP TSG-RAN WG4 Meeting #107	R4-2308425

Incheon, Korea (Republic of), 22 – 26 May 2023

Agenda Item:	9.7.7.2

Source:	Ericsson

Title:	Simulation results of SAN demodulation requirements for IoT-NTN

Document for:	Information

# 1	Introduction

RAN4#106bis-e agreed with the way forward on SAN demodulation requirements for IoT over NTN [1], and updated the simulation assumption. This contribution provides our simulation results according to the updated simulation assumption in the way forward.

# 2	SAN demodulation requirements for eMTC

## 2.1	PUSCH demodulation requirements

Table 1	Simulation parameters for PUSCH.

| Parameters | CE Mode A | CE Mode B |
| --- | --- | --- |
| Max number of HARQ transmissions | 4 | 2 |
| RV sequences | 0, 2, 3, 1, 0, 2, 3, 1 | 0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1 |
| Number of PUSCH repetitions | 8 | 256 |
| Frequency hopping | OFF | OFF |
| Duplex mode | FDD only | FDD only |
| Number of Tx antennas | 1 | 1 |
| Number of Rx antennas | 1 and 2 | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLC5-5 Option 2: NTN-TDLA100-5 | Option 1: NTN-TDLA100-5 Option 2: NTN-TDLC5-5 |
| FRC | TS36.104 A3-2 | TS36.104 A3-1 |
| System BW | 1.4MHz only | 1.4MHz only |
| Fraction of maximum throughput | 70% | 70% |

Table 2	Simulation results for PUSCH CE Mode A.

| Rx antenna | Repetitions | Propagation condition | SNR for 70% of peak rate |
| --- | --- | --- | --- |
| 1 | 8 | NTN-TDLA100-5 | -6.1 |
| 1 | 8 | NTN-TDLC5-5 | -6.1 |
| 2 | 8 | NTN-TDLA100-5 | -10.5 |
| 2 | 8 | NTN-TDLC5-5 | -9.2 |

Table 3	Simulation results for PUSCH CE Mode B.

| Rx antenna | Repetitions | Propagation condition | SNR for 70% of peak rate |
| --- | --- | --- | --- |
| 1 | 256 | NTN-TDLA100-5 | -15.1 |
| 1 | 256 | NTN-TDLC5-5 | -16.3 |
| 2 | 256 | NTN-TDLA100-5 | -17.4 |
| 2 | 256 | NTN-TDLC5-5 | -18.1 |

## 2.2	PUCCH demodulation requirements

### 2.2.1	ACK missed detection requirements for single user PUCCH format 1a

Test metric:

- The ACK missed detection probability shall not exceed 1%.

Table 4	Simulation parameters for PUCCH format 1a.

| Parameters | Values |
| --- | --- |
| Number of PUCCH repetitions | 8 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | NTN-TDLA100-5 |
| System BW | 1.4MHz |

Table 5	Simulation results for PUCCH format 1a.

| Rx antenna | Repetitions | Propagation condition | SNR for 1% of ACK missed detection |
| --- | --- | --- | --- |
| 1 | 8 | NTN-TDLA100-5 | -5.4 |
| 2 | 8 | NTN-TDLA100-5 | -11.1 |

## 2.3	PRACH demodulation requirements

### 2.3.1	PRACH detection requirements

Test metric:

- The false alarm probability shall be less than or equal to 0.1%.

- The probability of detection shall be equal to or exceed 99%.

Table 6	Simulation parameters for PRACH preamble detection requirements.

| Number of Tx antennas | Number of Rx antennas | Propagation channel | Frequency offset | Number of repetitions | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | NTN-TDLA100-5 | 270Hz | 8 | YES | YES |  |  |
|  |  |  |  | 16 |  |  | YES | YES |

Table 7	Simulation results for PRACH preamble detection requirements.

| Rx antenna | Repetitions | Propagation condition | Burst format | SNR for 99% of detection success rate |
| --- | --- | --- | --- | --- |
| 1 | 8 | NTN-TDLA100-5 | BF0 | -13.3 |
| 1 | 8 | NTN-TDLA100-5 | BF1 | -13.3 |
| 1 | 16 | NTN-TDLA100-5 | BF2 | -19.6 |
| 1 | 16 | NTN-TDLA100-5 | BF3 | -19.6 |
| 2 | 8 | NTN-TDLA100-5 | BF0 | -18.8 |
| 2 | 8 | NTN-TDLA100-5 | BF1 | -18.9 |
| 2 | 16 | NTN-TDLA100-5 | BF2 | -24.0 |
| 2 | 16 | NTN-TDLA100-5 | BF3 | -24.0 |

# 3	SAN demodulation requirements for NB-IoT

## 3.1	NPUSCH format 1 demodulation requirements

Table 8	Simulation parameters for NPUSCH format 1.

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions | FRC | Repetition number | Fraction of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | 3.75KHz | 1 | Option 1: NTN-TDLA100-1 Option 2: NTN-TDLC5-1 | TS36.104 A16-1 | 4 | 70% |
| 1 | 1 and 2 | 15KHz | 12 | Option 1: NTN-TDLC5-1 Option 2: NTN-TDLA100-1 | TS36.104 A16-5 | 16 | 70% |

Table 9	Simulation results for NPUSCH format 1.

| Rx antenna | Subcarrier spacing | Number of tones | Repetitions | Propagation condition | SNR to achieve 70% of peak rate |
| --- | --- | --- | --- | --- | --- |
| 1 | 3.75kHz | 1 | 4 | NTN-TDLA100-1 | -3.5 |
| 1 | 3.75kHz | 1 | 4 | NTN-TDLC5-1 | -6.1 |
| 1 | 15kHz | 12 | 16 | NTN-TDLA100-1 | -7.5 |
| 1 | 15kHz | 12 | 16 | NTN-TDLC5-1 | -10.0 |
| 2 | 3.75kHz | 1 | 4 | NTN-TDLA100-1 | -7.1 |
| 2 | 3.75kHz | 1 | 4 | NTN-TDLC5-1 | -8.7 |
| 2 | 15kHz | 12 | 16 | NTN-TDLA100-1 | -11.7 |
| 2 | 15kHz | 12 | 16 | NTN-TDLC5-1 | -13.1 |

## 3.2	NPUSCH format 2 demodulation requirements

### 3.2.1	ACK missed detection requirements

Test metric:

- The DTX to ACK probability, i.e., the probability that ACK is detected when nothing is sent per NPUSCH format 2 transmission, shall not exceed 1% per NPUSCH format 2 transmission.

- The ACK missed detection probability shall not exceed 1%.

Table 10	Simulation parameters for NPUCCH format 2.

| Number of TX antennas | Number of RX antennas | Propagation conditions | Number of allocated subcarriers | Subcarrier spacing | Repetition number |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | NTN-TDLA100-1 | 1 | 3.75KHz | 16 |
| 1 | 1 and 2 | NTN-TDLA100-1 | 1 | 15KHz | 16 |

Table 11	Simulation results for NPUSCH format 2.

| Rx antenna | Subcarrier spacing | Repetitions | Propagation condition | SNR for 1% of ACK missed detection rate |
| --- | --- | --- | --- | --- |
| 1 | 3.75kHz | 16 | NTN-TDLA100-1 |  |
| 1 | 15kHz | 16 | NTN-TDLA100-1 | 3.8 |
| 2 | 3.75kHz | 16 | NTN-TDLA100-1 |  |
| 2 | 15kHz | 16 | NTN-TDLA100-1 | -5.1 |

## 3.3	NPRACH demodulation requirements

### 3.3.1	NPRACH detection requirements

Test metric:

- The false alarm probability shall be less than or equal to 0.1%.

- The probability of detection shall be equal to or exceed 99%.

Table 12	Simulation parameters for NPRACH missed detection.

| Number of Tx | Number of Rx | Repetition number | Propagation channel and correlation | Frequency offset | PF0 | PF1 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | 8 | NTN-TDLA100-1 | 200Hz | YES | YES |
| 1 | 1 and 2 | 16 | NTN-TDLA100-1 | 200Hz | YES | YES |

Table 13	Simulation results for NPRACH missed detection.

| Rx antenna | Repetitions | Propagation condition | Preamble format | SNR for 99% of detection success rate |
| --- | --- | --- | --- | --- |
| 1 | 8 | NTN-TDLA100-1 | PF0 | 10.5 |
| 1 | 8 | NTN-TDLA100-1 | PF1 | 10.5 |
| 1 | 16 | NTN-TDLA100-1 | PF0 | 5.9 |
| 1 | 16 | NTN-TDLA100-1 | PF1 | 5.8 |
| 2 | 8 | NTN-TDLA100-1 | PF0 | 0.7 |
| 2 | 8 | NTN-TDLA100-1 | PF1 | 1.1 |
| 2 | 16 | NTN-TDLA100-1 | PF0 | -1.9 |
| 2 | 16 | NTN-TDLA100-1 | PF1 | -1.7 |

# References

- R4-2305965, “WF for SAN demodulation requirements for IoT over NTN”, Ericsson.