3GPP TSG-RAN WG4 Meeting #107	R4-2308431

Incheon, Korea (Republic of), 22 – 26 May 2023

Agenda Item:	8.13.5.2

Source:	Ericsson

Title:	Simulation results of CA PDSCH demodulation requirements for FR2 HST

Document for:	Discussion

# 1	Introduction

RAN4#106bis-e agreed with the way forward on demodulation requirements for Rel-18 FR2 HST [1], and agreed with the initial simulation assumption for PDSCH CA demodulation requirements [2]. This contribution provides our initial simulation results for PDSCH CA demodulation requirements according to the simulation assumption and discusses the remaining open issues.

# 2	Simulation results

Table 1 shows our initial simulation results for FR2 HST-DPS according to the simulation assumption [2]. The detailed FRC tables are shown in Appendix.

Table 1	PDSCH CA simulation results for alignment.

| Test number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | FRC 1-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10.5 |
| 1-2 | FRC 1-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10.5 |
| 1-3 | FRC 1-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10.5 |
| 1-4 | R.PDSCH.5-12.2 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10.5 |
| 2-1 | FRC 2-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10.5 |
| 2-2 | FRC 2-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10.5 |
| 2-3 | FRC 2-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10.5 |
| 2-4 | R.PDSCH.5-12.1 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10.5 |

# 3	Discussion

## 3.1	Carrier combinations

Agreement - Specify CA PDSCH requirements for FR2 HST with component carrier configuration as - {50, 100, 200, 400} Hz for 120KHz SCS - Companies are encouraged to check whether existing SNR requirement for 200MHz CBW defined in single carrier can be reused for CA case with 200MHz in the next meeting pending on the collected impairment results - FFS on the carrier combinations considered for requirement

In RAN4#106bis-e, RAN4 agreed to specify the FR2 HST CA PDSCH demodulation requirements by combining multiple single carrier PDSCH demodulation requirements, as same as the CA demodulation requirements defined in Rel-16. However it is FFS on the CA CC configurations. The existing CA demodulation requirements in FR2 is specified for intra-band contiguous CA (CA_C), intra-band non-contiguous CA (CA_N), and inter-band CA (CA_AX with X bands), according to the RF core requirements specified in TS36.101-2. On the other hand, this objective of this WI [3] is to specify the demodulation performance requirements for intra-band carrier aggregation only. We therefore propose the HST CA demodulation requirements are only applicable for intra-band contiguous CA (CA_C) and intra-band non-contiguous CA (CA_N). Other than that, we think the existing applicability and test rules for FR2 CA PDSCH demodulation can be used for HST CA scenarios, that is, UE is tested only for the largest aggregated CA bandwidth combination.

Proposal 1: Rel-18 FR2 CA PDSCH demodulation requirements for HST are only applicable for intra-band contiguous CA and intra-band non-contiguous CA combinations.

Proposal 2: Apply the existing applicability and test rules for FR2 CA PDSCH demodulation (TS38.101-4 clause 7.1.1.5) to Rel-18 CA PDSCH demodulation requirements for HST.

## 3.2	Additional margin

Way forward: - FFS on the necessity of considering additional margin for CA requirement based on single carrier simulation with considering imperfection model - FFS on how to consider the imperfection model if considered

RAN4 PDSCH demodulation requirements are usually set from the average of companies’ impairment results with additional margin if the span of alignment results is small (e.g. 2.5dB). RAN4 have applied this approach for Rel-16 FR2 CA PDSCH demodulation requirements, where RAN4 set the additional margin to 0.5dB because of 16QAM [4]. We think RAN4 can apply the same approach for HST CA demodulation requirements as far as the span of alignment results is small enough, e.g., 2.5dB. Since the FRC uses 64QAM for HST-DPS, however, we propose to set the additional margin as 0.7dB.

Proposal 3: RAN4 should consider additional margin of 0.7dB for Rel-18 CA PDSCH demodulation requirements for HST as far as the span of alignment results is less than 2.5dB.

# 4	Summary

Proposal 1: Rel-18 FR2 CA PDSCH demodulation requirements for HST are only applicable for intra-band contiguous CA and intra-band non-contiguous CA combinations.

Proposal 2: Apply the existing applicability and test rules for FR2 CA PDSCH demodulation (TS38.101-4 clause 7.1.1.5) to Rel-18 CA PDSCH demodulation requirements for HST.

Proposal 3: RAN4 should consider additional margin of 0.7dB for Rel-18 CA PDSCH demodulation requirements for HST as far as the span of alignment results is less than 2.5dB.

# References

- R4-2305892, “WF on demodulation requirements for Rel-18 FR2 HST”, Samsung.

- R4-2305893, “Simulation assumption for PDSCH requirements with CA”, Ericsson.

- RP-222272, “Revised WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung.

- R4-2006531, “Summary of Normal CA simulation results (NR FR2 120 kHz TDD)”, Intel Corporation.

# Appendix

Table 2	FRC for test 1 (HST-DPS-FR2-BI-B, single TCI states)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-12.2 TDD (For information) | FRC 1-1 | FRC 1-2 | FRC 1-3 |
| Channel bandwidth | MHz | 200 | 50 | 100 | 400 |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| Allocated resource blocks | PRBs | 132 | 32 | 66 | 264 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 9 | 9 | 9 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 13 | 13 | 13 | 13 |
| Allocated slots per 2 frames |  | 126 | 126 | 126 | 126 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 17 | 17 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 12 | 12 | 12 | 12 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 18 | 18 | 18 | 18 |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Overhead for TBS determination |  | 6 | 6 | 6 | 6 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 61480 | [14856] | [30728] | [122976] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 90176 | [21504] | [45096] | [180376] |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i = 1 |  | N/A (Note 4) |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 8 | [2] | [4] | [15] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs | 11 | [3] | [6] | [22] |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 2 and 82 (Note 3) | Bits | 180720 | [32640] | [75384] | [391392] |
| For Slots i = 3 and 83 (Note 3) | Bits | 116568 | [17088] | [43308] | [263088] |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 146520 | [35520] | [73260] | [293040] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,79,82,…,159} | Bits | 210672 | [51072] | [105336] | [421344] |
| For Slot i = 1 | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Max. Throughput averaged over 2 frames | Mbps | 522.195 | [124.838] | [261.116] | [1044.529] |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1 with periodicity 20ms |  |  |  |  |  |

Table 3	FRC for test 2 (HST-DPS-FR2-UNI-A, two TCI state)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-12.1 TDD (For information) | FRC 2-1 | FRC 2-2 | FRC 2-3 |
| Channel bandwidth | MHz | 200 | 50 | 100 | 400 |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| Allocated resource blocks | PRBs | 132 | 32 | 66 | 264 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 9 | 9 | 9 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 13 | 13 | 13 | 13 |
| Allocated slots per 2 frames |  | 127 | 127 | 127 | 127 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 17 | 17 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 12 | 12 | 12 | 12 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 18 | 18 | 18 | 18 |
| For Slot i = 1 |  | 18 | 18 | 18 | 18 |
| Overhead for TBS determination |  | 6 | 6 | 6 | 6 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 61480 | [14856] | [30728] | [122976] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 90176 | [21504] | [45096] | [180376] |
| For Slot i = 1 |  | 90176 | [21504] | [45096] | [180376] |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i = 1 |  | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 8 | [2] | [4] | [15] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs | 11 | [3] | [6] | [22] |
| For Slot i = 1 |  | 11 | [3] | [6] | [22] |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 2 and 82 (Note 3) | Bits | 195696 | [41856] | [90360] | [406368] |
| For Slots i = 3 and 83 (Note 3) | Bits | 131544 | [26304] | [58284] | [278064] |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 146520 | [35520] | [73260] | [293040] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {2,…,79,82,…,159} | Bits | 210672 | [51072] | [105336] | [421344] |
| For Slot i = 1 | Bits | 210672 | [51072] | [105336] | [421344] |
| Max. Throughput averaged over 2 frames | Mbps | 526.704 | [125.914] | [263.371] | [1053.548] |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1 with periodicity 20ms |  |  |  |  |  |