3GPP TSG-RAN WG4 Meeting #107	R4-2308433

Incheon, Korea (Republic of), 22 – 26 May 2023

Agenda Item:	8.13.5.4

Source:	Ericsson

Title:	Tunnel deployment scenario for FR2 HST enhancements

Document for:	Discussion

# 1	Introduction

RAN4#106bis-e agreed with WF on demodulation requirements for Rel-18 FR2 HST [1]. This contribution discusses the tunnel deployment scenario for FR2 HST.

For UE demod: - Reuse the channel model in RAN4 spec 38.101-4, i.e., single path with LOS propagation, for performance requirements study of FR2 HST-DPS in tunnel deployment as baseline - FFS on new channel model is needed for Bi-directional deployment in tunnel scenario - FFS on new channel model (multi-tap) is needed pending on UE location with faraway or close around RRH - FFS on consider multipath fading channel for demodulation performance study For BS demod: FFS with below candidate options - Extend existing FR1 channel model (geometry-Ds and Dmin and speed) scenario 3 (tunnel space) in RAN4 spec 38.104 - Reuse FR2 Rel-17 38.104 single path with LOS propagation channel model with necessary update if identified - Multipath fading channel

# 2	Discussion

RAN4 demodulation performance requirements for NR are specified with several propagation channel models, and they are derived from the outcome of RAN1 technical report (e.g., [2]) or RAN4 technical report (e.g., [3]). We should point out RAN4 rely on the RAN1 TR to define the multi-path fading channel models such as TDLA or TDLD. The channel models RAN4 has developed is the single-tap LOS based channel models such as HST single tap or HST-DPS/HST-SFN. These channel models basically changes the Doppler shift according to the modeled UE speed and signal direction to/from TRP(s).

According to the TR on the channel model for NR up to 100GHz [2], RAN1 have not studied the tunnel scenario. We are reluctant to define new multi-path fading channel models for the tunnel deployment scenario without RAN1 study. Therefore we propose to consider simple LOS-based single-tap channel models to evaluate the demodulation performance for tunnel deployment scenario.

Proposal 1: RAN4 should consider the LOS-based single-tap model(s) to study UE/BS demodulation performance for tunnel deployment scenario.

In Rel-13, RAN4 studied the channel model for high-speed train for LTE, and this study also includes the tunnel scenario [3].

The high speed condition of scenario 2e could reuse channel model for the Scenario 3: Tunnel for multi-antennas specified in TS 36.104 B.3 [2]. The high speed condition of scenario 2g could reuse the channel model of Scenario 2: Tunnel with leaky cable specified in TS 25.104 B.4A [3]. The high speed condition of scenario 4 could reuse the channel model of Scenario 1: Open space specified in TS 36.104 B.3[2]. Since the parameters of scenario 2e, 2g and 4 are different of the parameters defined in TS 36.104 [2]and TS 25.104 [3], so the parameters for the three scenarios are listed in Table 5.1-1. In order not to introduce misunderstandings, the UTRA scenarios numbering will be kept here. ■ Scenario 1 (corresponding to scenario 4 in this SI): Open space ■ Scenario 2(corresponding to scenario 2g in this SI): Tunnel with leaky cable ■ Scenario 3(corresponding to scenario 2e in this SI): Tunnel for multi-antennas Table 5.1-1: Parameters for high speed train conditions Parameter; Value. Scenario 1; Scenario2; Scenario 3. 3000 m; Infinity; 500 m. 300 m; -; 5 m. K; -; 10 dB; -. 350 km/h; 350 km/h; 350 km/h. 875 Hz; 875 Hz; 875 Hz.

Figure 1	RAN4 Rel-13 HST study item assumption.

Since Rel-13 LTE is assumed to be operated below 6GHz, RAN4 studies the channel modes with 1) leaky cable and 2) multi-antenna scenarios. However we don’t think the leaky cable is feasible for carrier frequency of 30GHz assumed in this WI, because of the large the transmission loss. Moreover, we cannot apply Scenario 3 in Rel-13 HST or Scenario 3 specified in TS38.104 directly to 30GHz, because these models are, in our understanding, based on the bi-directional deployment and UE is supposed to have the omni-directional antenna.

In RAN4#106bis-e, RRM session agreed to consider uni-directional and bi-directional deployment for tunnel scenario with single Rx panel reception as follows [4]. Although RAN4 agreed to study the tunnel deployment scenario regardless of RRM core part conclusion, we should respect the agreements in RRM discussion.

Issue 2-1-1: Deployment assumptions for Scenario#1 Agreement: Consider both uni-directional and bi-directional deployments in the tunnel for Sceanrio#1 (single-panel reception UE and DPS transition scheme).

Figure 3 compares the Doppler trajectories between TS38.104 Scenario 3 and uni-directional scenario (Figure 2) with the parameters in Table 1. The detailed Doppler trajectory models are given in Appendix A. It is observed from Figure 3 is that the uni-directional deployment scenario does not show the fast Doppler shift changes at the TCI state switching, and it could be suitable for FR2 scenario.

Proposal 2: RAN4 reuse the uni-directional HST-DPS channel model to study the tunnel deployment scenario with parameters:

- Ds=700m

- Dmin=2m

- Ds_offset=10m

Proposal 3: For the tunnel deployment scenario, RAN4 should assume PC6 UE receives signals with single Rx panel.

Figure 2	Example of tunnel scenario using uni-directional deployment.

| (a) Scenario 3 | (b) Uni-directional |
| --- | --- |

Figure 3	Doppler trajectories for Tunnel scenario

Table 1	Example of parameters for FR2 HST tunnel scenario

| Parameters | Values |
| --- | --- |
| Ds | 700m |
| Dmin | 2m |
| v | 350km/h |
| fd | 9,722Hz for UE 19,444Hz for BS |

# 3	Summary

Proposal 1: RAN4 should consider the LOS-based single-tap model(s) to study UE/BS demodulation performance for tunnel deployment scenario.

Proposal 2: RAN4 reuse the uni-directional HST-DPS channel model to study the tunnel deployment scenario with parameters:

- Ds=700m

- Dmin=2m

- Ds_offset=10m

Proposal 3: For the tunnel deployment scenario, RAN4 should assume PC6 UE receives signals with single Rx panel.

# References

- R4-2305892, “WF on demodulation requirements for Rel-18 FR2 HST”, Samsung.

- TR38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”, V17.0.0, 3GPP.

- TR36.878, “Study on performance enhancements for high speed scenario in LTE”, V12.0.0, 3GPP.

- R4-2306398, “WF on NR FR2 HST UL Timing Adjustment Solutions and Tunnel Deployment”, Nokia, Nokia Shanghai Bell.

# Appendix

## A.1	Scenario 3 (cf. TS38.104 G.3)

fst=fdcosθ(t)

cosθt=DS2-vtDmin2+Ds2-vt2,  0≤t<Dsv -1.5Ds+vtDmin2+-1.5Ds+vt2,  Dsv≤t<2Dsvcosθt mod 2Dsv,  t<2Dsv

## A.2	Uni-directional scenario (cf.: TS38.101-4 B.3.4)

fst=fdcosθ(t)

cosθt=Ds_offset+Ds-vtDmin2+Ds_offset+Ds-vt2,  0≤t<Dsv cosθt mod Dsv,  t<Dsv