3GPP TSG-RAN WG4 Meeting #107                              R4-2308436

Incheon, KR, May 22 – May 26, 2023

Source: 	Murata Manufacturing Co Ltd.

Title: 	n8 DL 25MHz REFSENS and RSD

Agenda Item:	7.22.2	High power UE for FR1 for FDD single band(s) with PC2	[HPUE_NR_FR1_FDD_R18]

Document for:	Approval

Introduction

In the RAN4#106-bis-e meeting, the asymmetrical UL/DL 25MHz for n8 has been discussed in WID [1]. The UL CBW is restricted to 20MHz same as n5. Also, the RSD of PC2 n8 is being discussed in other WF [2]. In this paper, we propose the REFSENS of n8 25MHz for PC3 and the RSD for PC2.

Discussion

WF review

WF R4-2306545 [2]

2	Band n8

Agreement:

PC2 considerations for band n8 and 25 MHz CBW will be handled in the PC2 FDD WI.

WF R4-2306544 [3]

<Sub-topic 1-3 Handling of PC2 for new CBW proposed in WI“NR_BW_Bands_R18”>

Agreement:

- On n8 25MHz: Cover PC2 for n8 25MHz in FDD PC2 HPUE WI, but this does not prevent/delay completion for other potential completed CBWs.

REFSENS for PC3 and RSD for PC2

FE architecture

The two kinds of FE architecture shown in the Figure 2 are used to analyse REFSENS and RSD. For 2Tx/2Rx architecture, the mutual influence between primary Tx and diversity Tx was considered.

Figure 2. FE architecture

Calculation conditions

Table 1. Calculation conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Tx power | 23 dBm | For 1Tx PC3, at ANT1 |
|  | 26 dBm | For 1Tx PC2, at ANT1 |
|  | 23 dBm | For 2Tx PC2, at ANT1&ANT2 |
| Post PA loss | 4 dB |  |
| ANT_ Iso. | 10 dB | ANT1⇔ANT2 |
| Duplexer rejection | 55 | Tx to Rx, at Tx band |
|  | 55 | Tx to Rx, at Rx band |
|  | 40 | Tx to ANT, at Rx band |
| BPF | 40 | ANT to Rx, at Tx band |
| MRC | ANT1&ANT2 | Correlated |

Results

The calculation results are shown in Table 2.

Table 2. REFSENS for PC3 and RSD for PC2

|  | Parameter | Unit | UL/DL = 20/25MHz |
| --- | --- | --- | --- |
| PC3 | Scaling REFSENS | dBm | -89.7 |
|  | Desense | dB | 5.4 |
|  | MRC REFSENS | dBm | -84.4 |
| PC2 | RSD 1Tx | dB | 2.8 |
|  | RSD 2Tx | dB | 6.1 |

Proposal:

The REFSENS of PC3 and RSD of PC2 are proposed in bold in Table 2.

Conclusion

In this contribution, we proposed the REFSENS of PC3 and the RSD of PC2 for n8 25MHz asymmetrical UL/DL.

Proposal:

The REFSENS of PC3 and RSD of PC2 are proposed in bold in Table 2.

Table 2. REFSENS for PC3 and RSD for PC2

|  | Parameter | Unit | UL/DL = 20/25MHz |
| --- | --- | --- | --- |
| PC3 | Scaling REFSENS | dBm | -89.7 |
|  | Desense | dB | 5.4 |
|  | MRC REFSENS | dBm | -84.4 |
| PC2 | RSD 1Tx | dB | 2.8 |
|  | RSD 2Tx | dB | 6.1 |

References

[1] RP-230756 “Basket WID: adding new channel bandwidth(s) support to existing NR bands”, Ericsson, 3GPP TSG RAN Meeting #99, March 20th – 23rd, 2023

[2] R4-2306545 “WF on adding new channel bandwidths support to existing NR bands”, Ericsson, 3GPP TSG-RAN4 Meeting #106-bis-e, Online, 17th – 26th April 2023