3GPP TSG-RAN WG4 Meeting #107	R4-2308438

Incheon, Korea, May 22- May 26, 2023

Agenda Item:	8.26.2.2

Source: 	Ericsson

Title:	Discussions on collision between MUSIM gaps

Document for:	Discussion

# Introduction

In last meeting, RAN4 had the discussion on MUSIM gaps requirements[1].  The main issues for MUSIM gaps are how to handle the collision scenarios as follow:

- Collisions between MUSIM gap and legacy measurement gap

- Collisions between MUSIM gap and SMTC and other L3/L1 measurement resources

- Collisions between different MUSIM gaps

We had shared our views about the general pricinples to define the requirements. In this contribution, we will continue to discuss how to handle the collision for MUSIM gaps.

# MUSIM gaps priority

Priority of MUSIM gaps

In last meeting, RAN4 agrees to introduce the priority for MUSIM gaps in NW’s configuration. Two open issues for the priority configuration are shown as follow.

Issue 2-1-4-1: Constraints on MUSIM gap priority configuration from NW A - Proposals - P1: NW A maintains the same relative priorities requested by the UE; The configured priority level may or may not be the same as that requested by UE. (vivo Apple MTK Xiaomi Huawei Qualcomm Nokia Charter ZTE) - P1-a: Based on P1, NW A is not required to keep the relative priority order for a particular MUSIM gap when the MGRP of that particular MUSIM gap is less than a threshold, in this scenario NW A will still keep the same relative order of the other MUSIM gaps except for that particular MUSIM gap (vivo) - P2: When MUSIM gaps with equal priority is allowed, if UE requests two MUSIM gaps with the same priority X and if the network configures both gaps, then both gaps must be assigned a common priority X’. X’ may or may not be equal to X. (vivo Apple Qualcomm oppo) - P4: If the network cannot fulfill the UE priority requests the network may chose not to assign the requested MUSIM gaps (Nokia Qualcomm) - P6: In the special case when both one MUSIM gap and one other MG gap has set the highest priority level in gapPriority-r17 IE, then we propose that MUSIM gap has the ability to signal with an extra 1-bit to indicate higher priority than the highest level in gapPriority-r17 IE (Charter)

The first remaining issue is whether to introduce any constraints for NW-A’s configuration. However, in RAN4 #106 meeting, it was already agreed as follow.

RAN4 #106 meeting Issue 2-1-2: Priority/usage indication on MUSIM gaps from UE side - Agreements - UE can optionally indicate its preferred priority for all or a subset MUSIM gaps - It is up to NW A on how to use this information

Thus, RAN4 should follow the agreement and no further discussion on the constraints for NW-A’s configuration is needed.

Proposal 1: RAN4 not to further discuss any constraints for NW-A’s configuration.

Another remaining issue is whether to configure the priority to aperiodic gap. As we know, aperiodic MUSIM gap is a one-shot gap. It should be prioritized when UE requests the aperiodic MUSIM gap. Otherwise, UE doesn’t need to request such aperiodic gap.

Issue 2-1-5: Priority setting for aperiodic MUSIM gaps Agreements from GTW - Option 1 (QC, Nokia, vivo, Charter, Xiaomi) - The priority level of aperiodic MUSIM gap can be configured by NW A - If the priority level is not configured by NW A then the aperiodic MUSIM gap by default has the highest priority level - The aperiodic MUSIM gap priority level can be optionally requested by UE from NW A - Option 2 (MTK, Huawei, Apple, Ericsson, OPPO): - Aperiodic MUSIM gap by default has the highest priority level. - The gap priority level is not explicitly configured by the NW

Thus, it’s unnecessary to further requested a priority to the aperiodic gap. To move forward, we proposed to merge option 1 and option 2 as follow.

Proposal 2: The aperiodic MUSIM gap by default has the highest priority level. The priority level of aperiodic MUSIM gap can be configured by NW A.

UAI

In last meeting, RAN4 also agreed the detail of UE assistant information as follow. The remaining open issues for UAI are additional restriction.

Issue 2-1-4-2: Constraints on MUSIM gap priority indication from UE side - Proposals - P1: When UE requests the MUSIM gaps, the MGRP of highest priority gap should be larger than 160ms; When UE requests only one MUSIM gap, the MGRP should be larger than 80ms (Ericsson ZTE)

The MUSIM gaps are mainly to monitor UE’s behaviour in NW-B’s IDLE mode. In IDLE mode, UE is required to perform serving cell evaluation, neighbour cell measurements, and paging monitoring etc. In legacy requirement, the minimum space of the measurement samples for serving cell evaluation is DRX cycle and the measurement interval for intra-frequency/inter-frequency/inter-RAT measurement will be at least 1.28s. Thus, it’s unnecessary to request a short MGRP to monitor these measurement behaviours in IDLE mode. The general UE’s behaviours for measurement with the shorest DRX configuration in IDLE mode is shown in the figure below.

Figure 1: Example of UE measurement behaviours in IDLE mode(DRX=320ms, serving SMTC=160ms)

In our understanding, the highest priority MUSIM gap will be used to at least monitor paging which means the MGRP of the gap should be based on configured DRX cycle. When this highest MUSIM gap is also used for other behaviours, such as serving cell evaluation, the MGRP of this gap will be shorten. However, the number of requested MUSIM gaps will be also reduced. Thus, to trade off the effectivity of the MUSIM gaps and the flexibility of the MUSIM gap design, UE should report the highest MUSIM gap with MGRP larger than 160ms.

Proposal 3: When UE requests multiple MUSIM gaps, the MGRP of highest priority gap should be larger than 160ms. When UE requests only one MUSIM gap, the MGRP should be larger than 80ms.

# Collision within MUSIM gaps

Solution for collision between different MUSIM gaps

In last meeting, another important issue is how to handle the collision within MUSIM gaps.

Issue 2-2-2: Solutions for collision between different MUSIM gaps - Option 1: Priority based solution is used for collision handling between different MUSIM gaps - Option 2: Keep solution (keep all collided MUSIM gap) is used for collision handling between different MUSIM gaps - Option 3a: Use both Priority based solution and Keep solution for collision handling between different MUSIM gaps - Priority based solution is used when collided MUSIM gaps have different priority levels - Conditions when Keep solution is used are FFS. - Option 3b: Use both Priority based solution and Keep solution for collision handling between different MUSIM gaps - Conditions when priority based solution is used and conditions when Keep solution is used are FFS.

In our understanding, paging monitoring is the most important thing for NW-B’s Idle mode in MUSIM. The SSB before paging for AGC retuning also cannot be dropped. The time proximity between SSB and related PO is uncertain which depends on different SSB and PO multiplexing pattern, default/non-default association between SSB and PO, and SSB index indication(ssb-position-in-burst). Both MUSIM gap occasions should not be dropped once UE requests two periodic gaps which meets the collision proximity.

Figure 2: Example of MG for PO monitoring in NW-B Idle mode

On the other hand, if two MUSIM gaps’ occasions are used for NW-B’s measurement, UE will face the same issue as Con-MGs to perform consecutive measurements within 4ms. Thus, the gap dropping rule in Con-MGs should be applied. Considering UE needs to handle both aspects of MUSIM gap collision, a hybrid solution should be used.

Proposal 4: MUSIM gap ‘keep rule’ will be applied in some certain scenarios, such as Paging monitoring and AGC.

The next issue is the condition to apply the gap keep rule. In our understanding, the typical case is for AGC and paging monitoring, or these two MUSIM gaps will be used for the same frequency layer. However, the issue is NW doesn’t know which MUSIM gap will be used for paging or which two MUSIM gaps will be used for the same frequency layer. Thus, RAN4 needs to further discuss the solution to indicate to the NW when gap keep rule will be applied within two MUSIM gaps.

Proposal 5: RAN4 needs to further discuss how to indicate to the NW when ‘keep rule’ will be applied within two MUSIM gaps.

# Collision between MUSIM gaps with NW-A’s gaps

Collision between MUSIM gaps and Type-1 MG

In last meeting, one of collision issues is MUSIM gaps colliding with Type-1 gap. Some companies proposed to not define any requirement in this scenario. However, we think this should be a real scenario. Both NW and UE may only support MUSIM gaps but not support Rel-17 concurrent gaps. Thus, we believe a clear UE behaviour should be defined.

Issue 2-3-2: Solutions for collision between MUSIM gap and Type-1 MG or gap configured without priority - P1: When a MUSIM gap collides with a legacy MG, requirements shall not apply if any one of the collided gaps is not assigned a priority. (vivo Apple xiaomi oppo) - Against: (Nokia) - P2: No requirements apply if the two gaps are configured with same MGRP (Huawei Ericsson) - Against: (Nokia) - P3: RAN4 to prioritize the gap with longer MGRP for the following MUSIM collision scenarios: Any of the collision gaps is Type-1 MG; NW-A doesn’t configure a priority associated with any of the collision gaps. (Ericsson Huawei ZTE MTK) - P4: MUSIM gaps are assumed to have higher priority than a Type-1 MG when either MUSIM gaps or Type-1 MG (or both) are not assigned priorities by the network. (Qualcomm) - Against: (Huawei xiaomi) - P5: Introduce priority for Type-1 MG when MUSIM gaps are configured when also having Type-1 measurement gaps allocated. (Nokia) - Against: (Qualcomm Huawei xiaomi) - P6: The sharing rule solution could be considered. (xiaomi) - Against: (Qualcomm Huawei)

Some companies proposed to always prioritize the MUSIM gaps once no priority is configured. We don’t think such solution can work. For example, when UE is moving to the cell edge of the serving cell in NW-A and the NW-A configures the Type-1 MG to perform inter-frequency measurement for handover, the Type-1 MG cannot be dropped when colliding with MUSIM gaps. Especially, when the MGRP of Type-1 MG is larger than MGRP of MUSIM gap, always prioritizing the MUSIM gap means L3 mobility measurement cannot be performed for NW-A. Thus, a reasonable solution is to prioritize the gap with longer MGRP to avoid no measurement opportunity for one configured gap.

Proposal 6: RAN4 to prioritize the gap with longer MGRP for the following MUSIM collision scenarios:

- Any of the collision gaps is Type-1 MG;

- NW-A doesn’t configure a priority associated with any of the collision gaps.

# Collision between MUSIM gaps with other signals in NW-A

In last meeting, companies discussed how to define the collision and the priority between MUSIM gap with NW-A’s DL RS and uplink signals.

Issue 2-4-3: Collision between SMTC and MUSIM gaps for handover and Scell activation - Proposals - P1: Collisions between other RRM procedures and MUSIM gaps are handled in the same way as collisions between RRM procedures and legacy MG, i.e., no special handling solution is defined. (Qualcomm MTK xiaomi Huawei Nokia vivo oppo) - P2: RAN4 to define requirements for the collision between MUSIM gaps with Handover, Scell activation and SI update. When NW-A’s RS resources for one-shot RRM procedure (Handover, Scell activation, SI update) collide with MUSIM gaps, MUSIM gaps should have lower priority; Periodic paging monitoring or one-shot procedure in NW-B Idle mode, such as On-demand SI reading have higher priority than Measurements procedures for both NW-A and NW-B (Ericsson) - P3: When NW-A’s uplink signals for one-shot RRM procedure(Handover, Scell activation) collide with MUSIM gaps, MUSIM gaps should have lower priority, such as NW-A’s PRACH and CSI-RS reporting for Scell activation should be prioritized. (Ericsson) - P4: Add a high-level clarification in RAN4 spec that during one-shot procedure such as Scell activation, SI update and so on, UE is not expected to enable MUSIM gaps unless existing RRM requirement for the corresponding one-shot procedure can be met. (Apple Ericsson)

As mentioned in the WID, the main intention to define the MUSIM gaps requirement is to guarantee minimized impact on NW-A’s performance. We noticed that the requested MUSIM gaps are basically periodical gaps for measurement, paging monitoring. However, some mobility procedures in NW-A are one-shot procedures, such as Handover, SCell activation, SI update. Compared with periodic procedures, these one-shot procedures are more important from NW-A’s perspective. If the proceudre’s delay is extended, it will have severe impact to NW-A. Thus, NW-A’s SSB/SMTC or uplink signalas for one-shot mobility procedure should have higher priority than MUSIM gaps. On the contrary, when NW-A’s L1/L3 measurement resources collide with MUSIM gaps, MUSIM gaps should have higher priority.

Thus, when RAN4 discusses the MUSIM gaps colliding with DL RS or UL signals, it’s necessary to differentiate the usage of the DL RSs and UL signals.

Proposal 7: RAN4 to add a high level clarification for the collision between MUSIM gaps with Handover, SCell activation and SI update.

- When NW-A’s L1/L3 measurement resources collide with MUSIM gaps, MUSIM gaps should have higher priority.

- When NW-A’s RS resources for one-shot RRM procedure(Handover, SCell activation, SI update) collide with MUSIM gaps, MUSIM gaps should have lower priority.

- When NW-A’s uplink signals for one-shot RRM procedure(Handover, SCell activation) collide with MUSIM gaps, MUSIM gaps should have lower priority, such as NW-A’s PRACH and CSI-RS reporting for SCell activation should be prioritized.

# Multiple gaps collision

In Rel-17 Con-MGs, RAN4 only defined the requirement to handle two gap collision since only two Type-2 MGs are allowed. When MUSIM gaps are introduced, the total number gaps will be 5 or 6. Thus, multiple gap collision scenario may happen.

Issue 5-1-2: Order for applying the priority when number of colliding MGs is larger than 2 - Proposals: - P1: Collisions between gaps are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority (vivo oppo Huawei) - P2: RAN4 to postpone multiple gap collision issue until RAN4 has a clear understanding on MUSIM gaps’ priority. (Ericsson) - P3: If multiple gaps collide it will be the gap with the highest priority that is used by the UE and other lower priority gaps are dropped. (Nokia)

From our understanding, at least RAN4 needs to consider the following scenarios:

- two Type-2 MGs with one MUSIM gap

- one NW-A gap with two MUSIM gaps

However, there is no clear solution to handle the gap collisions for MUSIM gaps. It’s hardly to decide how to handle the multiple gaps collision issue. For example, when MUSIM gap keep rule is introduced, NW needs to decide whether to drop the gaps or keep them depends on different scenarios. When UE requests MUSIM gap 1 and gap 2 and needs to keep both of them, only keep the highest priority gap cannot work. In this case, UE may need to keep both MUSIM gap 1 and MUSIM gap 2 and apply the gap priority rule to both of MUSIM gaps. Thus, we proposed to postpone the discussion until RAN4 achieves the agreements on MUSIM gaps’ collision rules.

Figure 3: Example of multiple MG collision

Proposal 8: RAN4 to postpone multiple gap collision issue until RAN4 has a clear understanding on MUSIM gaps’ priority.

- Summary

In this contribution, we have discussed the MUSIM gaps requirements. Based on the discussions, we have made following proposals and observations:

Proposal 1: RAN4 not to further discuss any constraints for NW-A’s configuration.

Proposal 2: The aperiodic MUSIM gap by default has the highest priority level. The priority level of aperiodic MUSIM gap can be configured by NW A.

Proposal 3: When UE requests multiple MUSIM gaps, the MGRP of highest priority gap should be larger than 160ms.

Proposal 4: MUSIM gap ‘keep rule’ will be applied in some certain scenarios, such as Paging monitoring and AGC.

Proposal 5: RAN4 needs to further discuss how to indicate to the NW when ‘keep rule’ will be applied within two MUSIM gaps.

Proposal 6: RAN4 to prioritize the gap with longer MGRP for the following MUSIM collision scenarios:

- Any of the collision gaps is Type-1 MG;

- NW-A doesn’t configure a priority associated with any of the collision gaps.

Proposal 7: RAN4 to add a high level clarification for the collision between MUSIM gaps with Handover, SCell activation and SI update.

- When NW-A’s L1/L3 measurement resources collide with MUSIM gaps, MUSIM gaps should have higher priority.

- When NW-A’s RS resources for one-shot RRM procedure(Handover, SCell activation, SI update) collide with MUSIM gaps, MUSIM gaps should have lower priority.

- When NW-A’s uplink signals for one-shot RRM procedure(Handover, SCell activation) collide with MUSIM gaps, MUSIM gaps should have lower priority, such as NW-A’s PRACH and CSI-RS reporting for SCell activation should be prioritized.

Proposal 8: RAN4 to postpone multiple gap collision issue until RAN4 has a clear understanding on MUSIM gaps’ priority.

- References

- R4-2220443, “WF on RRM requirements for Rel-17 MUSIM gaps”, Vivo