3GPP TSG-RAN WG4 Meeting #107	R4-2308442

Incheon, Korea, May 22- May 26, 2023

Agenda Item:	8.10.2.3

Source:	Ericsson

Title:	Discussion on NCSG and Con-MGs

Document for:	Discussion

# Introduction

In last meetings, RAN4 discussed the combination of NCSG and Con-MGs [1]. Some terminology for gap were used as follow.

Issue 2-2: Definitions: legacy, concurrent, baseline and component gaps < Agreement >: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17

In this contribution, we will continue to discuss the related issues for NCSG and ConMGs with the agreed terminology.

# Parallel measurements

Capability

In last meeting, one of the open issues is whether additional capability is needed for NCSG+NCSG.

Issue 4-2-4: [Case 2] Whether the same RF chain is assumed for the two NCSG (i.e. NCSG + NCSG) patterns < Agreement >: - No need to discuss whether the same RF chain is assumed for the two NCSG patterns (not necessary). Issue 4-2-5: [Case 2] Whether to consider a new capability for NCSG + NCSG in an FR < Way forward >: - FFS the following options - Option 1a: - No, without UE capability if two NCSG patterns are supported based on single RF chain. - Option 2: - Yes, with UE capability - Option 2a: - Yes, with UE capability if two RF chains are in use to support NCSG+NCSG, we are fine to introduce new UE capability - Option 3: - Postpone the discussion on new capability for NCSG + NCSG until RAN4 has a consensus on parallel measurement.

In our understanding, whether a new capability is introduced depends on whether new hardware or software updates needed from UE side. In Rel-17, RAN4 introduced NCSG which implies UE has a spare RF chain to perform measurement without gap. In last meeting, RAN4 agrees to support NCSG+NCSG in Rel-18. However, it’s unclear about the meaning behind such NCSG+NCSG capability. One possible understanding is that UE can only performs the measurements within two different SSB occasions with offset by two NCSG patterns. In this case, no further enhancement is needed from UE once UE supports NCSG and ConMGs. Another possible understanding is UE will use more than one spare RF chains when UE supports NCSG+NCSG. It implies UE needs to have additional hardware update compared with NCSG and ConMGs capability.

Proposal 1: No new capability is needed if only one spare RF chain is assumed for NCSG+NCSG.

Proposal 2: RAN4 can discuss the new capability if two spare RF chains are assumed for NCSG+NCSG.

Parallel measurement

In last meeting, RAN4 further discuss the parallel measurements issues. Some open issues related to parallel measurements are listed below.

Issue 4-2-1: [Case 2] Whether to consider parallel measurements upon gap collision < Agreements from GTW >: - Option 1: Support of parallel measurements upon NCSGs collision is up to UE capability - Option 2: Do not support parallel measurements upon NCSGs collision Issue 4-2-2: [Case 2] Whether to support parallel measurements in the following scenarios for two NCSG < Way forward >: - FFS the following options Scenario 1: NW only configures deactivated SCells’ measurement Scenario 2: NW only configures the MOs in intra-bands in which UE reports to support ‘NCSG’ Scenario 3: NW configures MOs in intra-band associated with NCSG1 and MOs in inter-band associated with NCSG2 if UE reports ‘NCSG’ for these bands

In our understanding, if RAN4 wants to answer the 1st issue whether to consider parallel measurement upon collision, RAN4 firstly needs to have further technical analysis for all the scenarios proposed in 2nd issue.

Some companies mentioned that when UE supports NCSG for band X and band Y, it implies UE has a spare RF chain which can be used to perform measurement for band X or band Y in parallel with the serving cells’ transmission and reception. It doesn’t mean the UE can perform two measurements from band X and band Y in parallel. We agree this observation is valid.

However, in Rel-15, RAN4 defines intra-frequency measurement outside gap as parallel measurements. In Rel-16, RAN4 further introduces inter-frequency without gap as parallel measurements with intra-frequency outside gap.

Thus, if UE reports to support NCSG for an intra-band, and the NW only configures the MOs in the intra-band associated with the NCSG gaps, such as intra-frequencies, and/or deactivated SCells, in our understanding, UE can perform the parallel measurement among the two NCSG patterns.

Observation 1: UE can perform transmission/reception for two CA cells simultaneously.

Observation 2: In Rel-15/16, RAN4 defines UE to perform parallel measurements between intra-frequency without gap and inter-frequency without gap.

Furthermore, the inter-band’s MO can also be measured in parallel with the MOs in the intra-band if UE reports to support NCSG for the intra-band and inter-band when NW configures MOs in intra-band associated with NCSG1 and MOs in inter-band associated with NCSG2.

For example, when UE reports to support NCSG capabilities and there are 3 potential MOs to be measured as follow. In our understanding, the MO f2 can be measured in parallel with MO f0, f1 since UE reports to support NCSG for the inter-band B3 which means UE has a spare RF chain to perform measurement for band B3 in parallel with serving cells’ measurement in intra-band B1. Thus, when UE supports NCSG and ConMGs, with suitable configuration, UE can perform parallel measurements for intra-band and inter-band if UE reports ‘NCSG’ in these bands.

Table 1. The UE reporting band status and NW configured MOs

|  | Supported Bands |  |  |
| --- | --- | --- | --- |
|  | B1(Intra-) | B2(Intra-) | B3(Inter-) |
| PCell(B1)+SCell(B2) | 1 | 1 | 1 |
| Configured MOs | f0 | f1 | f2 |
| Note: 0: no-gap-no-ncsg, 1: NCSG, 2: gap |  |  |  |

In last meeting, some companies think UE can perform parallel measurement in the above scenarios, but the use case is only for fully overlapping between two NCSGs. However, we think another important use case is for proximity collision. In Rel-17, if two gaps are close to each other, the gap collision rule shall be also applied. We think it’s unreasonable to further apply the proximity in NCSG gap collision.

Proposal 3: RAN4 to support parallel measurement on NCSG and ConMGs.

Proposal 4: When considering gap dropping rule of NCSG, the proximity distance shall be 0 for NCSG collision.

Proposal 5: When UE supports NCSG and ConMGs, UE can perform parallel measurements in the following scenarios:

- Case 1: NW only configures deactivated SCells’ MOs

- Case 2: NW only configures the MOs in intra-bands in which UE reports to support ‘NCSG’

- Case 3: NW configures MOs in intra-band associated with NCSG1 and MOs in inter-band associated with NCSG2 if UE reports ‘NCSG’ for these bands

Generical parallel measurement

When we further consider the possible parallel measurement, we believe that UE has capability to perform parallel measurement based on the spare RF chain for some candidate band combinations, especially when UE supports more than one spare RF chains. However, currently, NW doesn’t know whether the parallel measurement is supported under current MOs configuration and association. For example, the configured MO associated with MG and the MO associated with the NCSG may not be measured in parallel as explained by some companies. Thus, we suggest RAN4 to further study the solution to allow NW knowing the parallel measurement MO combination before NW configuring the MOs. After that, NW can configure the MOs with a reasonable association to allow the parallel measurement between NCSG patterns.

Observation 3: When UE reports the supported NCSG band status, NW doesn’t know whether parallel measurement can be performed among these bands.

Proposal 6: RAN4 to study a general solution to allow both NW and UE to know the parallel measurements combination when UE supports NCSG parallel measurement capability.

# Rel-17 MG rule enhancement

Gap association rule

When UE capable of NCSG reports the NCSG indication for each band, the NW can configure a NCSG and a Type-2 MG. The MOs in the bands which UE indicates ‘no gap with interruption’ can be configured with the association of NCSG and the MOs in the bands which UE indicates ‘gap’ can be configured with the association of Type-2 MG. The other MOs in the bands which UE indicates ‘no gap no interruption’ and the MOs related to gapless measurement capability (inter-frequency without gap in Rel-16) will be measured outside the gap. Thus, when NW configures a NCSG and a Type-2 MG, RAN4 also needs to consider each frequency layers’ association among these three sets: NCSG, Type-2 MG and outside gap.

Issue 4-4-1: [Case 2] Potential changes for NCSG upon SCell activation < Agreements from GTW >: - UE behavior for deactivated SCell measurements with NCSG in Case 2 is FFS - Option 1: Legacy UE behavior (i.e. UE measures the deactivated SCell outside of MG) - Option 2: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped.

In our understanding, this issue is about further clarify UE’s association other than an optimization. In Rel-17, deactivated SCell is agreed to be measured within NCSG instead of outside gap if NW configures NCSG. However, when NW activates the SCell, the related SSB may not be measured if the SSB is outside active BWP when UE reports ‘gap’ for the band.

In Rel-18, when UE supports both MG and NCSG, deactivated SCell can be measured within NCSG. After SCell activation, the MO may further decide to be measured within gap or NCSG or outside gap depends on UE’s band reporting and BWP status.

- If UE reports ‘ncsg’ for the band, and

  - if the SSB is within the active BWP, the MO will be measured outside MG and NCSG;

  - if the SSB is outside the active BWP, the MO will be measured within NCSG.

  - in this case, the MO can be associated within the NCSG.

- If UE reports ‘gap’ for the band, and

  - if the SSB is within the active BWP, the MO will be measured outside MG and NCSG;

  - if the SSB is outside the active BWP, the MO will be measured within MG.

  - in this case, the MO can be associated within the Type-2 MG.

Figure 1. The transition from NCSG to MG for a MO with deactivated SCell

Observation 4: In Rel-17, when UE supports NCSG, deactivated SCell measurement will be performed within NCSG.

Proposal 7: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped regardless of gap association.

# Conclusion

In the contribution, we discuss the open issues for NCSG+Con-MGs. We have the following proposals:

Observation 1: UE can perform transmission/reception for two CA cells simultaneously.

Observation 2: In Rel-15/16, RAN4 defines UE to perform parallel measurements between intra-frequency without gap and inter-frequency without gap.

Observation 3: When UE reports the supported NCSG band status, NW doesn’t know whether parallel measurement can be performed among these bands.

Observation 4: In Rel-17, when UE supports NCSG, deactivated SCell measurement will be performed within NCSG.

Proposal 1: No new capability is needed if only one spare RF chain is assumed for NCSG+NCSG.

Proposal 2: RAN4 can discuss the new capability if two spare RF chains are assumed for NCSG+NCSG.

Proposal 3: RAN4 to support parallel measurement on NCSG and ConMGs.

Proposal 4: When considering gap dropping rule of NCSG, the proximity distance shall be 0 for NCSG collision.

Proposal 5: When UE supports NCSG and ConMGs, UE can perform parallel measurements in the following scenarios:

- Case 1: NW only configures deactivated SCells’ MOs

- Case 2: NW only configures the MOs in intra-bands in which UE reports to support ‘NCSG’

- Case 3: NW configures MOs in intra-band associated with NCSG1 and MOs in inter-band associated with NCSG2 if UE reports ‘NCSG’ for these bands

Proposal 6: RAN4 to study a general solution to allow both NW and UE to know the parallel measurements combination when UE supports NCSG parallel measurement capability.

Proposal 7: Proposal 7: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped regardless of gap association.

# Reference

- R4-2306330, “WF on further enhancements on measurement gaps and measurements without gaps”, MTK