3GPP TSG-RAN WG4 Meeting #107                              R4-2308456

Incheon, KR, May 22 – May 26, 2023

Source: 	Murata Manufacturing Co Ltd.

Title: 	REFSENS and RSD of n71 symmetrical UL/DL 25MHz and 30MHz

Agenda Item:	7.26.2.1	Adding new channel bandwidth(s) support to existing NR bands [NR_bands_R18_BWs]

Document for:	Approval

Introduction

In the RAN4#106-bis-e meeting, the symmetrical UL/DL 25MHz and 30MHz CBW of n71 has been discussed in WID [1]. On another hand, in WF [3], it was agreed to keep the relevant PC2 work of new bandwidth of n71 within the “Adding new channel BWs support to existing NR bands” WI. In this paper, we analyzed the impact of UL CBW extending on Rx noise and proposed REFSENS of PC3 operation and RSD for PC2 operation.

Discussion

WF review

WF R4-2306545 [2]

1.1	How to handle new UL BW for existing DL BW

Agreements:

Existing 25MHz and 30MHz REFSENS with 20MHz UL is maintained as the baseline requirement in the REFSENS Table and applies for legacy UEs

The symmetrical UL/DL 25MHz and 30MHz CBW is optional in R18 (and earlier) and mandatory from Rel-19.

The symmetrical UL/DL 25MHz and 30MHz CBW REFSENS values are captured in a way that makes it clear it is the optional requirement (using notes for example).

20 MHz UL CBE will still be used for DL CBW=35 MHz.

1.6	REFSENS (PC3)

Agreement:

UL RB allocation: 20 RBs at the bottom of the channel for both 25 and 30 MHz CBW

WF R4-2306544 [3]

<Sub-topic 1-3 Handling of PC2 for new CBW proposed in WI“NR_BW_Bands_R18”>

Agreement:

- On n8 25MHz: Cover PC2 for n8 25MHz in FDD PC2 HPUE WI, but this does not prevent/delay completion for other potential completed CBWs.

- On n71 25/30MHz and other potential new CBWs of other FDD bands: Keep the relevant PC2 work within the “Adding new channel BWs support to existing NR bands” WI.

The existing REFSENS of asymmetrical UL/DL are baseline of analysis. And the UL configuration of symmetrical UL/DL 25MHz and 30MHz are same as asymmetrical UL/DL. The RSD for PC2 also shall be considered in this WI.

Rx band noise analysis

The Figure 2 indicate the location of the IMD products generated by wanted Tx signal, LO leak, Tx Image, C-IM3 and C-IM5.

Figure 1. Tx output spectral

It is seen that only C-IM5 and the IM3 generated from C-IM3 and Tx signal fall into DL band when UL CBW is restricted to 20MHz. As their power level are lower relatively, the impact on Rx noise is not significant. However, when the UL CBW is extended to 25MHz or 30MHz, the IM3 generated from Tx signal and Tx image also fall into DL band. It causes the large increase of Rx band noise. Especially for CBW 30MHz, the IM3 completely fall into DL CBW.

Using the traditional Rx band noise measurement method used to REFSENS analysis in the past, we found that:

- Compared with asymmetrical UL/DL 25MHz, the Rx noise of symmetrical UL/DL 25MHz has increased by 13.8dB.

- Compared with asymmetrical UL/DL 30MHz, the Rx noise of symmetrical UL/DL 30MHz has increased by 20.8dB.

Observation:

- When UL CBW is extended to 25MHz and 30MHz, the IM3 of Tx signal and Tx image fall into DL CBW. it causes the large increase of Rx noise.

- Compared with asymmetrical UL/DL 25MHz, the Rx noise of symmetrical UL/DL 25MHz has increased by 13.8dB

- Compared with asymmetrical UL/DL 30MHz, the Rx noise of symmetrical UL/DL 30MHz has increased by 20.8dB.

REFSENS for PC3 and RSD for PC2

FE architecture

The two kinds of FE architecture shown in the Figure 2 are used to analyse REFSENS and RSD. For 2Tx/2Rx architecture, the mutual influence between primary Tx and diversity Tx was considered.

Figure 2. FE architecture

Calculation conditions

Table 1. Calculation conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Tx power | 23 dBm | For 1Tx PC3, at ANT1 |
|  | 26 dBm | For 1Tx PC2, at ANT1 |
|  | 23 dBm | For 2Tx PC2, at ANT1&ANT2 |
| Post PA loss | 4 dB |  |
| ANT_ Iso. | 10 dB | ANT1 - ANT2 |
| Duplexer rejection | 55 | Tx to Rx, at Tx band |
|  | 55 | Tx to Rx, at Rx band |
|  | 45 | Tx to ANT, at Rx band |
| BPF | 45 | ANT to Rx, at Tx band |
| MRC | ANT1&ANT2 | Correlated |

Results

The calculation results are shown in Table 2.

Compared with existing REFSENS of asymmetrical UL/DL, the degradation of REFSENS is around 12.2dB and 19.4dB for symmetrical UL/DL 25MHz and 30MHz respectively.

Table 2. REFSENS for PC3 and RSD for PC2

|  |  |  | Asymmetrical |  | Symmetrical |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DL CBW | MHz | 25 | 30 | 25 | 30 |
| PC3 | Scaling REFSENS | dBm | -89.9 | -89.1 | -89.9 | -89.1 |
|  | Desense | dB | 5.9 | 6.6 | 18.0 | 25.9 |
|  | REFSENS | dBm | -84.1 | -82.6 | -71.9 | -63.2 |
| PC2 | RSD 1Tx | dB | 2.2 | 2.0 | 3.0 | 3.1 |
|  | RSD 2Tx | dB | 6.0 | 6.5 | 7.4 | 7.8 |

Proposal:

The REFSENS for PC3 and RSD for PC2 are proposed in bold in Table 2.

Conclusion

In this contribution, we presented the Rx band noise analysis of symmetrical UL/DL 25MHz and 30MHz for n71 and proposed the calculation results of the REFSENS for PC3 and the RSD for PC2.

Observation:

- When UL CBW is extended to 25MHz and 30MHz, the IM3 of Tx signal and Tx image fall into DL CBW. it causes the large increase of Rx noise.

- Compared with asymmetrical UL/DL 25MHz, the Rx noise of symmetrical UL/DL 25MHz has increased by 13.8dB

- Compared with asymmetrical UL/DL 30MHz, the Rx noise of symmetrical UL/DL 30MHz has increased by 20.8dB.

Proposal:

The REFSENS for PC3 and RSD for PC2 are proposed in bold in Table 2.

Table 2. REFSENS for PC3 and RSD for PC2

|  |  |  | Asymmetrical |  | Symmetrical |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DL CBW | MHz | 25 | 30 | 25 | 30 |
| PC3 | Scaling REFSENS | dBm | -89.9 | -89.1 | -89.9 | -89.1 |
|  | Desense | dB | 5.9 | 6.6 | 18.0 | 25.9 |
|  | REFSENS | dBm | -84.1 | -82.6 | -71.9 | -63.2 |
| PC2 | RSD 1Tx | dB | 2.2 | 2.0 | 3.0 | 3.1 |
|  | RSD 2Tx | dB | 6.0 | 6.5 | 7.4 | 7.8 |

References

[1] RP-230756 “Basket WID: adding new channel bandwidth(s) support to existing NR bands”, Ericsson, 3GPP TSG RAN Meeting #99, March 20th – 23rd, 2023

[2] R4-2306545 “WF on adding new channel bandwidths support to existing NR bands”, Ericsson, 3GPP TSG-RAN4 Meeting #106-bis-e, Online, 17th – 26th April 2023

[3] R4-2306544 “WF on FDD HPUE Basket WIs”, China Unicom, 3GPP TSG-RAN4 Meeting #106-bis-e, Online, 17th – 26th April 2023