3GPP TSG-RAN WG4 Meeting # 107	R4-2308460

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	5.2.10.3

Source: 	OPPO

Title: 	Discussion on maintenance Rel-17 positioning

Document for:	Approval

1	Introduction

This contribution will discuss some maintenance issues for Rel-17 positioning measurements.

2	Discussion

In Rel-16, two new gap patterns #24 and #25 with longer MGL are introduced for PRS measurement, and these gap pattern can used only when UE is configured with PRS measurement. Considering that PRS measurement is only allowed with per UE gap, the gap patterns #24 and #25 are defined as per UE type accordingly. However, PRS measurement within per FR gap is supported in Rel-17 subjected to UE capability independentGapConfigPRS-r17, then we think gap patterns #24 and #25 of per FR type should be supported as well. Otherwise, the PRS resources with longer duration can only be measured with gap pattern #24 or #25 of per-UE type, and the benefits of per FR type gap will not be achieved.

Proposal 1: Gap patterns #24 and #25 could be configured as per FR type provided UE supports independentGapConfigPRS-r17.

If the UE is configured via LPP [34] to measure PRS for any RSTD, PRS-RSRP, and UE Rx-Tx time difference measurement defined in TS 38.215 [4], in order for the requirements in clauses 9.9.2, 9.9.3, and 9.9.4 to apply, the network must provide - a single per-UE measurement gap pattern for concurrent monitoring of all positioning frequency layers and intra-frequency, inter-frequency and/or inter-RAT frequency layers of all frequency ranges, or - for measurement gap patterns other than #24 and #25, if UE supports independent measurement gap patterns for different frequency ranges for PRS measurement, per-FR measurement gap pattern for the frequency range for concurrent monitoring of all positioning frequency layers and intra-frequency, inter-frequency cells and/or inter-RAT frequency layers in the corresponding frequency range.

For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis. For UEs which support and are configured with per FR gaps, the CSSF requirements do not apply when NR PRS measurement in one FR gap collides with SSB/CSI-RS/PRS measurements in the other FR gap in time domain.

Based on the current spec as shown above, the per FR gap if configured, will be used for concurrent monitoring all positioning frequency layers and intra-frequency, inter-frequency cells and/or inter-RAT frequency layers in the corresponding frequency range. And the CSSF calculation is done on a per FR basis in this case. In our understanding, it means the positioning frequency layers in FR1 should be measured within FR1 gap, and the same rule applies to FR2.

Proposal 2: Positioning frequency layers in FR1 should be measured within FR1 gap, and positioning frequency layers in FR2 should be measured within FR2 gap.

Based on discussions in RAN4#106-bis-e, RAN4 would like to confirm that PRS to be associated with per FR measurement gap in case of concurrent gaps can be supported from RAN4 perspective, provided that UE supports independentGapConfigPRS-r17. RAN4 respectfully asks RAN2 to take the above information into account.

During the previous meetings, per FR PRS gap in case of concurrent gaps were discussed and RAN4 agreed to support such the configuration provided that UE support independentGapConfigPRS-r17 [1]. However, all the positioning frequency layers should be associated with only one of the measurement gaps if concurrent gaps are configured. If proposal 2 is agreed, we think this restriction should be updated accordingly.

Proposal 3: If concurrent gaps are configured,

- All positioning frequency layers are associated with only one of the measurement gaps, or

- All positioning frequency layers in each FR are associated with only one of the per FR measurement gaps in the same FR.

3	Conclusion

This contribution gave our considerations on Rel-17 positioning and the following proposals:

Proposal 1: Gap patterns #24 and #25 could be configured as per FR type provided UE supports independentGapConfigPRS-r17.

Proposal 2: Positioning frequency layers in FR1 should be measured within FR1 gap, and positioning frequency layers in FR2 should be measured within FR2 gap.

Proposal 3: If concurrent gaps are configured,

- All positioning frequency layers are associated with only one of the measurement gaps, or

- All positioning frequency layers in each FR are associated with only one of the per FR measurement gaps in the same FR.

4	Reference

- R4-2306388, Reply LS on support of per FR PRS gap, Huawei, HiSilicon