3GPP TSG-RAN WG4 Meeting # 107	R4-2308462

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.10.2.3

Source: 	OPPO

Title: 	Discussion on case 2 requirements NCSG and conMG

Document for:	Approval

1	Introduction

RAN4 #106-bis e-meeting discussed the joint configuration between NCSG and concurrent MG, and reached some conclusions [1]. This contribution will give our further considerations.

2	Discussion

- Issue 4-1-1: [Case 2] Measurement gaps combinations principles < Agreement >: - The previous agreements from meeting #105 and #106 on the maximum number of configured MGs still apply: - Previous agreement 1: Do not increase the max number of gaps for Case 2 (NCSG and multiple concurrent MGs) comparing to Rel-17 concurrent MG design. - Previous agreement 2: Gap combination configuration of case 2 can use gap combination of concurrent gaps defined in TS38.133 table 9.1.8-1 as formatting baseline with the clarification that each configured gap can be NCSG or Type-2 MG. - New agreement: - For UE capable of [Concurrent NCSG], one FR can be configured with up to 2 NCSGs, regardless they are per-UE or per-FR configured and the configuration in the other FR. - UE capabilities for [Concurrent NCSG] are FFS. - For gap combination configuration of case 2, the per-UE gap in gap combination #3, #4 and #5 is associated with PRS measurement and cannot be NCSG. - FFS the wording of the notes to be added in the specification

For gap combinations in case 2 NCSG and conMG, the per-UE gap in gap combination #3, #4 and #5 is associated with PRS measurement and cannot be NCSG. During the previous meetings, RAN4 agreed to support per FR PRS gap in case of concurrent gaps provided that UE support independentGapConfigPRS-r17 [2]. Whether per FR PRS gap should also be considered in case 2 should be clarified. If so, any gap associated with PRS measurements cannot be NCSG.

Proposal 1: Discuss whether per FR PRS gap should be considered in case 2 NCSG and conMG.

For per FR capability, two capabilities are introduced for MG and NCSG respectively. The legacy capability, i.e. independentGapConfig is to indicate whether the UE supports two independent measurement gap configurations for FR1 and FR2, and this capability is reused for concurrent gaps. And a new capability ncsg-MeasGapPerFR-r17, is introduced to indicates whether the UE supports per-FR NCSG. The support of gap combinations in TS 38.133 table 9.1.8-1 with NCSG should be subject to the latter UE capability ncsg-MeasGapPerFR-r17.

Proposal 2: In case 2 NCSG and conMG, the support of gap combinations in TS 38.133 table 9.1.8-1 should be subject to UE capability ncsg-MeasGapPerFR-r17.

- Issue 4-2-1: [Case 2] Whether to consider parallel measurements upon gap collision < Agreements from GTW >: - Option 1: Support of parallel measurements upon NCSGs collision is up to UE capability - Option 2: Do not support parallel measurements upon NCSGs collision

As per the current definition of collision, at least two types of NCSG collision are identified as shown in figure 1. The ML boundary of two NCSG are not aligned in type 1. In this case, the VIL of NCSG 1 will be overlapped with the ML of NCSG 2, then the measurement of NCSG 2 will be impacted due to the radio retuning of NCSG 1 even if separate RF chains are assumed. Even for NCSG collision type 2 with aligned ML boundary, whether parallel measurement is feasible needs further discussion. With NCSG capability, UE could receive data on the serving band and measure SSB on the target band simultaneously. But it does not mean that simultaneous measurements on two different target bands can also supported. In this case, only one MO can be measured in the colliding occasion and the measurements on the two RF chains cannot be performed in parallel.

Proposal 3: Not consider parallel measurements upon NCSG collision.

Figure 1: illustration of NCSG collision

- Issue 4-2-5: [Case 2] Whether to consider a new capability for NCSG + NCSG in an FR < Way forward >: - FFS the following options - Option 1a: - No, without UE capability if two NCSG patterns are supported based on single RF chain. - Option 2: - Yes, with UE capability - Option 2a: - Yes, with UE capability if two RF chains are in use to support NCSG+NCSG, we are fine to introduce new UE capability - Option 3: - Postpone the discussion on new capability for NCSG + NCSG until RAN4 has a consensus on parallel measurement.

Besides the RF architecture, UE implementations with single NCSG and double NCSG may also be different. Scheduling availability will be considered for measurement within NCSG and sometimes transmission or reception is allowed on some symbols within ML. However within MG, only measurement is allowed and UE cannot transmit or receive data in the entire MG occasion. Considering such the difference between MG + NCSG and NCSG + NCSG, additional UE capability is needed for NCSG + NCSG scenario.

Proposal 4: Support NCSG + NCSG in an FR with additional UE capability.

- Issue 4-4-1: [Case 2] Potential changes for NCSG upon SCell activation < Agreements from GTW >: - UE behavior for deactivated SCell measurements with NCSG in Case 2 is FFS - Option 1: Legacy UE behavior (i.e. UE measures the deactivated SCell outside of MG) - Option 2: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped.

For UE supporting nr-NeedForGapNCSG-reporting-r17 and indicating NeedForGapNCSG-InfoNR for intra-frequency measurement, - An intra-frequency SSB measurement is defined as measurement without gap if - the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoNR for the intra-frequency measurement, and - the SSB is not completely contained in the active BWP of the UE, and - the active downlink BWP is not an initial BWP [3] The delay requirements are specified in clause 9.2.5 An intra-frequency SSB measurement is defined as measurement with NCSG if - the UE indicates ‘ncsg’ via NeedForGapNCSG-InfoNR for the intra-frequency measurement, and - the SSB is not completely contained in the active BWP of the UE, and - the active downlink BWP is not an initial BWP [3] When network configures NCSG, the delay requirements are specified in clause 9.2.7 When network configures measurement gap, the delay requirements are specified in clause 9.2.6 - An intra-frequency SSB measurement is defined as measurement with gap if - the UE indicates ‘gap’ via NeedForGapNCSG-InfoNR for the intra-frequency measurement, and - the SSB is not completely contained in the active BWP of the UE, and - the active downlink BWP is not an initial BWP [3] When network configures measurement gap, the delay requirements are specified in clause 9.2.6 - The UE can perform intra-frequency SSB based measurement corresponding to a deactivated SCell or dormant SCell with NCSG. - For intra-frequency SSB based measurements with NCSG, UE may cause scheduling restriction as specified in clause 9.2.7.3.

For deactivated SCell measurement, two sets of measurement requirements, e.g. outside MG/NCSG and within NCSG, are specified. Based on the current spec, it seems that intra-frequency SSB for a deactivated SCell or dormant SCell can be measured with NCSG by default. Then the RRM requirements for deactivated SCell measurement outside MG/NCSG is useless anymore. To distinguish these two cases, additional UE capability for deactivated SCell measurement should be considered.

Proposal 5: Introduce UE capability to distinguish deactivated SCell measurements outside MG/NCSG and within NCSG.

3	Conclusion

This contribution discussed the potential issues for case 2 requirements, and gave the following proposals.

Proposal 1: Discuss whether per FR PRS gap should be considered in case 2 NCSG and conMG.

Proposal 2: In case 2 NCSG and conMG, the support of gap combinations in TS 38.133 table 9.1.8-1 should be subject to UE capability ncsg-MeasGapPerFR-r17.

Proposal 3: Not consider parallel measurements upon NCSG collision.

Proposal 4: Support NCSG + NCSG in an FR with additional UE capability.

Proposal 5: Introduce UE capability to distinguish deactivated SCell measurement outside MG/NCSG and within NCSG.

4	Reference

- R4-2306330, WF on Rel-18 NR MG enhancements, MediaTek Inc

- R4-2306388, Reply LS on support of per FR PRS gap, Huawei, HiSilicon