3GPP TSG-RAN WG4 Meeting # 107	R4-2308470

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.26.2.1

Source: 	OPPO

Title: 	Discussion on general RRM requirements for Rel-17 MUSIM gaps

Document for:	Approval

1	Introduction

According to the revised WID, the corresponding requirements for MUSIM gaps will be discussed in Rel-18. This contribution will discuss the maintenance issues as captured in [2].

2	Discussion

Issue 1-1-2: MUSIM overhead - Proposals: - Option 1: Do not define overhead cap for MUSIM gaps (Qualcomm vivo CMCC Ericsson Huawei Nokia Apple) - Option 2: Define overhead cap for MUSIM gaps. (xiaomi oppo) - Option 2a: Measurement requirement does not apply when more than one MUSIM gap is configured with MGRP = [20] ms (xiaomi) Issue 1-1-3: Total number of gaps when MUSIM gaps are configured - Proposals: - P1: Consider only one Rel-17 legacy gap when MUSIM gaps are configured. (vivo) - P2: (Qualcomm vivo CMCC xiaomi Huawei Ericsson Apple) - When MUSIM gaps are configured and Rel-17 Con-MGs is not configured, the number of legacy MGs can be - Up to 1 per-UE MG, or - Up to 1 per-FR MG in each FR - When MUSIM gaps are configured, when Rel-17 con-MG is configured, the number of legacy MGs can optionally be - Up to 2 per-UE MGs - Up to 2 per-FR MGs in each FR and up to 3 per-FR MGs across FRs - Up to 1 per-UE MG and up to 1 per-FR MG in each FR - P3: Allocation of MUSIM gaps does not impact the non-MUSIM gap allocation capability. (Qualcomm MTK Nokia Apple) - P4: UE shall not request more MUSIM gaps than it is capable of handling with the current measurement gap allocation (Nokia)

In Rel-17, overhead cap rule on multiple concurrent gaps was agreed in the RAN4 #104-e meeting. The main reason is to avoid too much throughput loss and is also suitable for MUSIM gaps. In our understanding, the intention behind limiting the number of legacy gaps in issue 1-1-3 is the same as overhead cap rule. But it is less generic and is mainly from the perspective of restrictions for legacy gaps. When different numbers or combinations of MUSIM gaps are configured, it should be allowed to configure different number of legacy gaps accordingly. Therefore, defining total overhead cap rules considering both MUSIM gaps and legacy gaps is preferred.

Proposal 1: Define total overhead cap rules considering both MUSIM gaps and legacy gaps.

However, different from concurrent gaps where at most 2 gaps could be configured in each FR, UE could be configured with up to 3 MUSIM gaps and one aperiodic gap, not to mention the gaps for NW-A measurements. Excluding the case when more than one MGP with MGRP=20ms may also lead to a higher throughput loss in NW-A. For example, 2 MUSIM gaps with MGRP=40ms and 1 MUSIM gap with MGRP=20ms will cause the same throughput loss with 2 gaps with MGRP=20ms. For simplicity, we propose to preclude the case when more than 2 gaps are configured with MGRP<=40ms in an FR. And we are also open to overhead cap rules considering other cases such as when MUSIM gap with longer MGRP is configured or when 3 MUSIM gaps + 1 legacy gap are configured.

Proposal 2: Further study the following rules besides the existing overhead cap rule in Rel-17:

- measurement requirement does not apply when more than 2 gaps are configured with MGRP<=40ms in an FR.

- FFS other overhead cap rules.

As for the total number of gaps when MUSIM gaps are configured, both P2 and P3 are acceptable for us, which are agreed for concurrent gaps in Rel-17. Besides, the existing overhead should also be reused to avoid throughput degradation, especially if the aforementioned MUSIM overhead is not agreed.

Proposal 3: For the total number of legacy gap when MUSIM gaps are configured, support P2 or P3 with the existing overhead cap restriction in Rel-17.

Issue 1-1-4: Mandatory MUSIM gap patterns - Proposals - P1: No need to discuss further whether to introduce mandatory MUSIM gap patterns (Qualcomm vivo oppo Apple MTK Huawei) - P2: RAN4 to define the mandatory MUSIM gap patterns (Ericsson Nokia Chapter CMCC)

Different from legacy gaps configured by network, MUSIM gap is up to UE request and UE will only request the gap supported patterns based on its capability. It is not necessary to introduce mandatory MUSIM gap patterns.

Proposal 4: Not introduce mandatory MUSIM gap pattern.

3	Conclusion

This contribution gave our views on general aspect for Rel-17 MUSIM gaps:

Proposal 1: Define total overhead cap rules considering both MUSIM gaps and legacy gaps.

Proposal 2: Further study the following rules besides the existing overhead cap rule in Rel-17:

- measurement requirement does not apply when more than 2 gaps are configured with MGRP<=40ms in an FR.

- FFS other overhead cap rules.

Proposal 3: For the total number of legacy gap when MUSIM gaps are configured, support P2 or P3 with the existing overhead cap restriction in Rel-17.

Proposal 4: Not introduce mandatory MUSIM gap pattern.

4	Reference

- R4-2306357, WF on NR Dual TxRx Multi-SIM, vivo