3GPP TSG-RAN WG4 Meeting # 107	R4-2308475

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.31.3.3

Source: 	OPPO

Title: 	On RRM requirements for co-channel coexistence for LTE SL and NR SL

Document for:	Approval

1	Introduction

As shown below, RAN4 is expected to continue discussion on co-channel coexistence between LTE sidelink and NR sidelink based on RAN1 progress [1]. This contribution will our views.

#### Issue 2-1-2: Whether to reuse existing RRM requirements for Co-channel coexistence for LTE SL and NR SL Candidate options: - Option 1: The existing RRM requirements are applicable for the co-channel coexistence solution of TDM-based semi-static resource pool partitioning - Option 2: Wait for RAN1’s conclusion on co-channel coexistence solutions, e.g., for dynamic resource pool. #### Issue 2-1-3: Others Candidate options: RAN4 can discuss the following proposals when RAN1 has completed to SL-CoEx work if needed. - P1: RAN4 to determine a set of evaluation parameters for NR SL and LTE SL coexistence. This can be with two perfectly aligned resource pools (in time and frequency) as well as equal subchannel sizes. - P2: No enhancements for the IDC framework from Rel-16 is needed to support Type A devices for LTE and NR co-channel coexistence.

2	Discussion

When TDM-based semi-static resource pool partitioning is used for co-channel coexistence between LTE sidelink and NR sidelink, UE behaviour is the same as that for IDC case on a dedicated carrier. In this case, UE will switch between LTE sidelink and NR sidelink, and interruptions to WAN and scheduling restrictions are required during the switch procedure. The existing RRM requirements can be applicable. The main enhancement for dynamic resource pool solution is resource selection procedure in RAN1. For example, NR SL module will exclude candidate resources overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission. Since no dedicated RRM requirement is specified for resource selection procedure, we can skip the discussion on the RRM impact from dynamic resource pool solution. But we are also fine to postpone the decision until clear conclusions are reached in RAN1.

Proposal-1: For TDM-based semi-static resource pool partitioning solution, the existing RRM requirements can be applicable including interruptions to WAN in section 12.7.3 and scheduling availability in section 12.9.1.

3	Conclusion

This contribution gave our views on core requirements for co-channel coexistence between LTE sidelink and NR sidelink and the following proposals:

Proposal-1: For TDM-based semi-static resource pool partitioning solution, the existing RRM requirements can be applicable including interruptions to WAN in section 12.7.3 and scheduling availability in section 12.9.1.

4	Reference

- R4-2306364, WF on NR_SL_enh2_part2, OPPO