3GPP TSG-RAN WG4 Meeting # 107					        	 R4-2308477

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.8.3.2

Source: 	OPPO

Title: 	On L1 measurement requirements for FR2 multi-RX DL reception

Document for:	Approval

1	Introduction

A WF on FR2 multi-RX DL reception was approved in RAN4#106bis meeting [1].

In this contribution, we further discuss L1 enhancement requirements for L1-RSRP for FR2 multi-Rx reception.

2	Discussion

Issue 1-1: Which reference signals to be considered for multi-rx L1 measurement requirements Agreement from GTW: - Multi-Rx L1 measurement requirements are defined under assumption of the following reference signals availability - CSI-RS + CSI-RS: CSI-RS reference signals are transmitted from the two TRPs - SSB + SSB: SSB signals are transmitted from the two TRPs - FFS - SSB + CSI-RS: SSB and CSI-RS signals are transmitted from different TRPs - FFS if same or different RS combinations would apply for different RRM requirements - FFS whether to consider simultaneous and/or non-simultaneous RS transmission from different TRP to define RRM requirement

Beam sweeping factor reduction

Agreement from GTW: - Introduce enhanced RRM requirements based on faster beam sweeping with multi-Rx chains based on UE capabilities - Option 1: Introduce beam sweeping factor reduction - Other options not precluded - FFS on specific conditions when enhancement apply - FFS if any signalling is needed - FFS on details of UE capabilities Way forward: Please bring further analysis on the FFS parts of the above GTW agreement to the next meeting. Agreements: How to capture the enhanced requirements (based on the agreement of enhanced requirements) in the spec due to faster beam sweeping can be discussed during CR stage. Issue 1-2-3: Whether to consider reducing the sharing factor for L3 and L1 Agreements: NO, do not consider enhancement to sharing factor for L3 and L1 in this WI

For the existing FR2 RRM requirements, UE needs to perform Rx beam sweeping and scaling factor (e.g., N=8) for SSB based L1-RSRP, RLM and BFD/CBD measurement delay is introduced.

The SSB based L1 measurements for FR2 are performed with beam sweeping factor of 8 for all the cases. The L1 measurements include RLM, BFD/CBD, L1-RSRP and L1-SINR.

For CSI-RS based L1 measurement, current scaling factor has considered number of resources in the resource set and maximum number of Rx beam (e.g., N=ceil (maxNumberRxBeam / Nres_per_set)). We can further discuss whether further reduction can apply duo to multi-Rx reception.

Proposal 1: Beam sweeping factor reduction is feasible at least for SSB-based L1-RSRP, RLM and BFD/CBD measurement. FFS beam sweeping factor reduction for CSI-RS based L1 measurement.

As agreed in last meeting, enhanced RRM requirements based on faster beam sweeping with multi-Rx chains will be introduced based on UE capabilities

Due to support of simultaneous multi-panel operation with several independent RX beams/chains, UE has multiple occasions to receive the measurement RS from a single direction in one sweeping cycle. For instance, 2 independent Rx beams are sweeping simultaneously, and each with separate baseband processing unit to enable simultaneous 2 beams sweeping. If the 2 Rx beams sweep each time without overlapping with previous directions that has been swept in one sweeping cycle, then at most 2 times faster beam sweeping can be achieved, and scaling factor can be reduced as N’=4. If the 2 Rx beams sweep each time with one beam overlapped with previous directions that has been swept in one cycle, then at worst beam sweeping is not fastened, and scaling factor is still the same as N=8.

Thus, whether it is possible to reduce the beam sweeping factor reduction in case UE receives the measurement RS from a single direction, depends on UE beam sweeping schemes and, like whether the multi-Rx beams are non-overlapping, partial overlapping, or fully overlapping with previous swept directions.

It is possible for UE to enable simultaneous multiple beams Rx and faster beam sweeping with independent Rx chains. Different Rx Beam sweeping/adjustment schemes (e.g., non-overlapping, partial overlapping, or fully overlapping with previous swept directions) may lead to different beam sweeping factors. Enhanced RRM requirements based on faster beam sweeping with multi-Rx chains will be introduced based on UE capabilities of Rx Beam sweeping/adjustment schemes.

Observation 1: Different Rx Beam sweeping/adjustment schemes (e.g., non-overlapping, partial overlapping, or fully overlapping with previous swept directions) may lead to different beam sweeping factors.

Proposal 2: Enhanced RRM requirements based on faster beam sweeping with multi-Rx chains will be introduced based on UE capabilities of Rx Beam sweeping/adjustment schemes.

Group based beam reporting (GBBR) requirements

According to the discussion in last meeting, if GBBR can be introduced, what requirements to introduced need to be further discussed.

Issue 1-3-2-1: How to perform measurement. That means the measurement delay required for GBBR (i.e., measurement delay for beam pair) - Option 1: Reuse legacy L1-RSRP delay as UE do not know which beams can be paired. - Option 2: Due to simultaneous reception measurement delay for beam pair can be enhanced for L1-RSRP Issue 1-3-2-2: How to choose beam pair that are going to be reported based on measurement results - Option 1: define conditions for selecting beam pair in the report - Please provide conditions that can be considered - Option 2: How to choose beam pair is UE implementation Issue 1-3-2-3: NW configures UE with GBBR, but UE did not find a good beam pair to report. What should be the UE behaviour - Option 1: NO behaviour is needed - Option 2: report NA or single beam - Option 3: Check with RAN1 - Note: if we agree for this, we may need to define what are the conditions for beam pair selection as we need to define what is not finding good beam pair.

We think the measurement delay required for GBBR (i.e., measurement delay for beam pair) can reuse legacy L1-RSRP delay. How to choose beam pair is UE implementation. For the worst case, UE do not know which beams can be paired and the beam pair could not provide any gain than exist procedure.

Proposal 3: How to choose beam pair that are going to be reported based on measurement results is up to UE implementation.

Proposal 4: The measurement delay required for GBBR can reuse legacy L1-RSRP delay.

As for the case UE cannot find any good beam pair, RAN4 can wait for LS reply from RAN1.

3	Conclusion

In this contribution, we propose our analysis and views on scenarios, beam sweeping factor reduction and the impact on measurement/scheduling restriction for L1 enhancements of FR2 multi-RX DL reception.

Proposal 1: Beam sweeping factor reduction is feasible at least for SSB-based L1-RSRP, RLM and BFD/CBD measurement. FFS for CSI-RS based L1 measurement.

Observation 1: Different Rx Beam sweeping/adjustment schemes (e.g., non-overlapping, partial overlapping, or fully overlapping with previous swept directions) may lead to different beam sweeping factors.

Proposal 2: Study the impact of different Rx Beam sweeping/adjustment schemes on Rx sweep factor.

Proposal 3: How to choose beam pair that are going to be reported based on measurement results is up to UE implementation.

Proposal 4: The measurement delay required for GBBR can reuse legacy L1-RSRP delay.

4	Reference

[1] R4-2306319, WF on NR FR2 multi-Rx chain DL reception – Part 2, Qualcomm