3GPP TSG-RAN WG4 Meeting # 107		R4-2308485

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.25.2.1

Source: 	OPPO

Title: 	On general and scenarios of L1/L2 based inter-cell mobility

Document for:	Approval

1	Introduction

A WF on Rel-18 Mobility enhancement [1] was approved in RAN4#106bis meeting.

In this contribution, we discuss the leftover issues on general aspects and scenarios of L1/L2 based inter-cell mobility.

2	Discussion

- Sub-topic 1-2 DL synchronization before cell switch command Issue 1-2-1: Requirements for DL synchronization before cell switch command < Way forward >: - RAN4 do not need to define any new requirements for obtaining symbol boundary and frame boundary of target cell before cell switch command, as legacy requirements for PSS/SSS detection and time index detection apply. - RAN4 do not need to define any new requirements for acquiring SFN of target cell before cell switch command. - Further discuss whether and how to define requirements for SSB based T/F fine tracking on neighbour cell based on further RAN1/2 progress. Issue 1-2-2: DL pre-sync starting point and UE capability requirements for DL pre-sync < Way forward >: FFS - Proposal 1: RAN4 to agree that UE starts performing DL pre-sync with LTM candidates when UE configured with TA establishment or reception of configuration for TA establishment. - Proposal 2: RAN4 to discuss the UE capability aspects of downlink synchronisation to multiple cells so that UE can transmit PRACH to the candidate cell on the first PRACH occasion after the PDCCH order reception.

As agreed in last meeting, it is unnecessary to define sync and async scenarios for LTM requirements. About whether to specify requirements for downlink/uplink synchronization before cell switch, we think cell detection or DL synchronization by L3 procedure needs long duration, at least tens of ms. To support L1/L2 based mobility, DL synchronization should be guaranteed.

RAN1/2 is also considering whether to perform DL synchronization to candidate/target cell before receiving the cell switch command. It is also up to decisions from other working groups.

One possible solution is not to define specific requirements for downlink synchronization before cell switch and reusing existing requirements for L3 measurement. In our view, before cell switch, the coarse DL timing information can be obtained through L3 measurements, e.g., symbol index information and frame boundary or SFN. We think that could be enough to DL synchronization for L1 measurement. If talking about T/F fine tracking, we think RAN4 should wait for RAN1 conclusion. We are fine to further discuss whether and how to define requirements for SSB based T/F fine tracking on neighbor cell based on further RAN1/2 progress.

Observation 1: The requirements of downlink synchronization before cell switch can use the existing requirements for L3 measurement as baseline.

Proposal 1: RAN4 not to define the specific requirement for DL synchronization before cell switch.

Proposal 2: Fine to further discuss requirements for SSB based T/F fine tracking on neighbour cell based on further RAN1/2 progress.

Issue 1-3-1: Transmit timing accuracy of PDCCH ordered RACH before cell switch command < Agreement>: - The legacy transmit timing accuracy requirement in 38.133 cl.7.1.2 is also applicable to PDCCH ordered RACH transmission for candidate cell(s) before cell switch command. - FFS: SSB based T/F fine tracking is needed for UE to meet Te requirements Issue 1-3-2: Delay requirements for PDCCH ordered RACH before cell switch command < Way forward >: FFS the following options - Option 1 (Apple, Ericsson, QC): Better to define similar delay requirements for PDCCH ordered RACH for candidate cell(s) in RAN1 but not RAN4 - Option 2 (MTK, Xiaomi, vivo, Huawei, ZTE, Nokia): RAN4 to study delay requirements for PDCCH ordered PRACH transmission to neighbour cell. - Option 2a (xiaomi): - If the reception of RAR is not configured/indicated, the delay of PDCCH-order based RACH transmission is defined as the time between the time of DCI command reception for PDCCH-order based RACH and the end of preamble transmission to the candidate cell plus RF retuning back time. - If the reception of RAR is configured/indicated, the delay of the PDCCH-order based RACH transmission is defined as the time between the time of DCI command reception for PDCCH-order based RACH and the end of RAR window plus RF retuning back time. - Option 2b (vivo): - RAN4 discuss and clarify whether the UL BWP of target cell is activated during uplink synchronization. Delay requirements need to be discussed after RAN1/2 concludes the corresponding procedure. - There is potential application delay and interruption for a cell on which DL sync is indicated to be performed by UE and/or UE needs to be prepared to transmit PRACH to the target cell. RAN4 will discuss the corresponding delay requirements. - Option 2c (QC): - RAN4 to discuss whether and how to define delay requirements for PDCCH ordered PRACH transmission to LTM cell for which UE needs additional processing to build and load RF scripts. It is also up to decisions from other working groups. - LTM requirements are applicable only when a QCL source reference signal of “PDCCH ordered PRACH to an LTM candidate cell before LTM handover” is the same or one of the reference signals configured and used for LTM L1-RSRP measurements from the cell. Issue 1-3-3: Interruption due to PDCCH ordered RACH before cell switch command < Agreement>: - Further discuss whether and how to define interruption and/or scheduling restriction requirements due to PDCCH ordered RACH before cell switch command once corresponding RAN1 design is stable.

RAN4 agreed to further discuss whether and how to define interruption and/or scheduling restriction requirements due to PDCCH ordered RACH before cell switch command once corresponding RAN1 design is stable.
Some companies also pointed out that UE needs additional processing to build and load RF scripts if agreed to delay and interruption requirements for PDCCH ordered PRACH transmission to LTM cell. In this case, downlink synchronization requirements for UE before receiving cell switch command may need to be revisited, which may also have impact on RAN1. So, We also think it is better to define similar delay requirements for PDCCH ordered RACH for candidate cell(s) in RAN1 but not RAN4.

Proposal 3: It is better to define similar delay requirements for PDCCH ordered RACH for candidate cell(s) in RAN1 but not RAN4.

3	Conclusion

Observation 1: The requirements of downlink synchronization before cell switch can use the existing requirements for L3 measurement as baseline.

Proposal 1: RAN4 not to define the specific requirement for DL synchronization before cell switch.

Proposal 2: Fine to further discuss requirements for SSB based T/F fine tracking on neighbour cell based on further RAN1/2 progress.

Proposal 3: It is better to define similar delay requirements for PDCCH ordered RACH for candidate cell(s) in RAN1 but not RAN4.

4	Reference

[1] R4-2306249, WF on L1/L2 inter-cell mobility, MediaTek Inc.