3GPP TSG-RAN WG4 Meeting #107                                                                R4-2308502

Incheon, KR, May 22 – May 26, 2023

Title: 	ATG co-existence simulation results

Source: 	CATT

Agenda item:	8.14.1.4

Document for:	Discussion

- Background

This contribution provides the ATG co-existence simulation results from our company according to the template table for the final simulation results [1].

- Discussion

In the simulation, non-colocated ATG and TN BS antennas and uniform ATG UE altitude distribution are assumed. ATG UE 8x2 antenna array was assumed for scenarios 1 and 4.

The data related to the users within the cell with largest throughput loss is still not available, we will update in next RAN4 meeting.

The simulation results for scenario 1 is as following,

| ATG/ TN BS antenna model | Performance Metric | Throughput Loss (%) at ATG BS ACLR 45 dB |
| --- | --- | --- |
| Non-Subarray | 5% in the whole network | 0.91 |
|  | Average of all users in the whole network | 0.24 |
|  | 5% of users within the cell with largest throughput loss for the case of TN DL victim |  |
|  | Average of all users within the cell with largest throughput loss for the case of TN DL victim |  |

The simulation results for scenario 2 is as following,

| ATG/ TN BS antenna model | Performance Metric | Throughput Loss (%) at ATG UE ACLR 30 dB |  |
| --- | --- | --- | --- |
|  |  | Maximum distance between ATG BS and ATG UE |  |
|  |  | 100 km | 300km |
| Non-Subarray | 5% in the whole network | 0.00 | 0.00 |
|  | Average of all users in the whole network | 0.00 | 0.00 |
|  | 5% of users within the cell with largest throughput loss for the case of TN UL victim |  |  |
|  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |

The simulation results for scenario 3 is as following,

| ATG/ TN BS antenna model | Performance Metric | Throughput Loss (%) at ATG UE ACS 33 dB |  |
| --- | --- | --- | --- |
|  |  | Maximum distance between ATG BS and ATG UE |  |
|  |  | 100 km | 300km |
| Non-Subarray | 5% in the whole network | 0.00 | 0.37 |
|  | Average of all users in the whole network | 0.00 | 0.16 |

The simulation results for scenario 4 is as following,

| ATG/ TN BS antenna model | Performance Metric | Throughput Loss (%) at ATG BS ACS 46 dB |
| --- | --- | --- |
| Non-Subarray | 5% in the whole network | 0.36 |
|  | Average of all users in the whole network | 0.36 |

The simulation results for scenario 9 is as following,

| ATG/ TN BS antenna model | Performance Metric | Throughput Loss (%) at ATG BS ACLR 45 dB |
| --- | --- | --- |
| Non-Subarray | 5% in the whole network | 5.87 |
|  | Average of all users in the whole network | 0.97 |
|  | 5% of users within the cell with largest throughput loss for the case of TN DL victim |  |
|  | Average of all users within the cell with largest throughput loss for the case of TN DL victim |  |

The simulation results for scenario 10 is as following,

| ATG/ TN BS antenna model | Performance Metric | Throughput Loss (%) at ATG UE ACLR 30 dB |  |
| --- | --- | --- | --- |
|  |  | Maximum distance between ATG BS and ATG UE |  |
|  |  | 100 km | 300km |
| Non-Subarray | 5% in the whole network | 0.00 | 0.00 |
|  | Average of all users in the whole network | 0.00 | 0.00 |
|  | 5% of users within the cell with largest throughput loss for the case of TN UL victim |  |  |
|  | Average of all users within the cell with largest throughput loss for the case of TN UL victim |  |  |

The simulation results for scenario 11 is as following,

| ATG/ TN BS antenna model | Performance Metric | Throughput Loss (%) at ATG UE ACS 33 dB |  |
| --- | --- | --- | --- |
|  |  | Maximum distance between ATG BS and ATG UE |  |
|  |  | 100 km | 300km |
| Non-Subarray | 5% in the whole network | 0.00 | 5.87 |
|  | Average of all users in the whole network | 0.00 | 0.97 |

The simulation results for scenario 12 is as following,

| ATG/ TN BS antenna model | Performance Metric | Throughput Loss (%) at ATG BS ACS 46 dB |
| --- | --- | --- |
| Non-Subarray | 5% in the whole network | 0.0049 |
|  | Average of all users in the whole network | 0.0049 |

Reference

[1] R4-2306607, “WF on ATG co-existence simulation”, RAN4#106b, CMCC