3GPP TSG-RAN WG4 Meeting # 107	R4-2308504

Incheon, KR, May 22 – May 26, 2023

Agenda Item: 8.8.3.2

Source: MediaTek Inc.

Title: Discussion on L1 measurements for multi-Rx UEs

Document for: Discussion

# Introduction

According to WF [1] and discussion summary [2], RAN4 had some discussion but most issues were discussed without conclusion yet. In this meeting, this WI is divided into six agenda items to be discussed: (1) general issues, (2) L1 measurement, (3) RLM/BFD/CBD, (4) scheduling/measurement restriction, (5) dual TCI state switching and (6) receive timing difference. The discussion in this paper focus on the “L1 measurement”, e.g., L1-RSRP/L1-SINR measurement.

# Discussion

The following discussion will cover N factor, P factor and open issue for L1 measurements.

- N factor

- P factor

- L1-SINR

- Rel-17 Group-based beam reporting for L1-RSRP requirement

## 2.1 N factor

Before we discuss whether N factor can be reduced or not, we would like to share our understanding on UE measurement behavior first. In the following, the legacy R15/R16 UE is discussed first, and then R18 multi-Rx UE is further studied.

R15/R16 UE

For a R15/R16 UE, it is assumed that more than one antenna module can be equipped but only one antenna module is active at a time. To meet the measurement delay requirement (N=8), UE may or may not use L3 measurement results to down-select antenna module first. So, there are two possible designs as below.

- Design 1: use L3 measurement to down select antenna modules

- Design 2: not use L3 measurement to down select antenna modules

For design 1, as step 1 in Figure 1, UE takes previous L3 measurement results into account. Therefore, in step 2, UE can further perform L1 measurement only on the antenna module which is down selected by L3 measurement results. In this case, the number of fine beams on each antenna module can be up to 8 to meet the L1 measurement delay requirement (N=8).

Figure 1. one possible UE design: performing L1 measurement based on previous L3 measurement info

For design 2, as Figure 2, UE does not take previous L3 measurement results into account. So, UE uses fine beam to perform L1 measurement on two antenna modules sequentially. In this case, the number of fine beams on each antenna module is 4 and total number of beams is 8. The reason why 4 fine beams is assumed on each antenna module is that UE should complete L1 measurement on both two antenna modules sequentially within a limited time (N=8).

Figure 2. one possible UE design: performing L1 measurement without previous L3 measurement info

R18 UE

In R18, two active antenna modules at a time are allowed. So, we would like to further study what is the difference for the design 1 and design 2 in this R18 WI.

For design 1, as step 1 in Figure 3, UE takes previous L3 measurement results into account. In step 2, the key point is the number of beams on each antenna module is still 8 if UE wants to maintain the same spherical coverage on each antenna module. Therefore, to our understanding, for design 1 UE, N should still be 8. Otherwise, the measurement delay is insufficient for UE to finish beam sweeping.

Figure 3. one possible UE design (two antenna modules can be activated at a time): performing L1 measurement based on previous L3 measurement info

For design 2, as Figure 4, UE does not take previous L3 measurement results into account. UE may activate up to two antenna modules at a time for L1 measurement. In this case, UE can perform L1 measurement on both antenna modules at a time. Therefore, N can be reduced.

Figure 4. one possible UE design (two antenna modules can be activated at a time): performing L1 measurement without previous L3 measurement info

So, based on above analysis, N cannot be reduced for all UEs because it depends on UE implementation.

Observation 1: N cannot be reduced for all UEs because it depends on UE implementation.

Proposal 1: Reducing the value of N in L1 measurement (L1-RSRP/BFD/CBD/RLM) delay requirement should be up to UE’s capability.

In last meeting of RAN4#106b, there’re agreement related to beam sweeping in WF [1].

Agreement from GTW: - Introduce enhanced RRM requirements based on faster beam sweeping with multi-Rx chains based on UE capabilities - Option 1: Introduce beam sweeping factor reduction - Other options not precluded - FFS on specific conditions when enhancement apply - FFS if any signalling is needed - FFS on details of UE capabilities Way forward: Please bring further analysis on the FFS parts of the above GTW agreement to the next meeting. Agreements: How to capture the enhanced requirements (based on the agreement of enhanced requirements) in the spec due to faster beam sweeping can be discussed during CR stage.

According to our previous analysis and proposal, whether UE could reduce beam sweeping(N) factor or not depend on UE implementation and up to UE’s capability. To us, the simple way is to indicate N factor reduction (E.g., N=4 or 8) base on UE capability. But we are open to discuss the specific conditions for the beam sweeping reduction.

## 2.2 P factor

In the last meeting, L1 and L3 measurement cannot be performed simultaneously based on below agreement in RAN4 #106 WF [1] . So, the Psharing_factor cannot be enhanced.

<Agreement >: Issue 1-1-6: Simultaneous L3 measurements and L1 measurements - Simultaneous L3 and L1 measurements is not supported in this release for multi-Rx. - Note: The agreement has no impact on scheduling restriction discussion for L3 measurements. Feasibility and benefit of scheduling restriction relaxation for L3 measurements is FFS.

In the WID [3], there is no consideration for high mobility. So, we do not see the need and gain why UE is required to perform L3 measurement more frequently. Besides, in order to achieve simultaneous L3 and L1 measurement, joint consideration between L3 and L1 measurement in UE implementation is needed. It will hugely increase UE complexity because UE needs to control different measurements on different antenna modules at a time. As a result, the following proposal is suggested.

Proposal 2: In L1 measurement (L1-RSRP/BFD/CBD/RLM) delay requirement, P factor should not be enhanced due to simultaneous L3 and L1 measurement is not supported in R18.

## 2.3 L1-SINR

In last meeting, some companies discuss L1-SINR handling as below issue [1].

Issue 1-5-1: Shall L1-SINR requirements be defined for the multi-RX UE FFS whether enhancements to L1-SINR are supported in this WI.

RAN4 agreed that Rel-17 group-based reporting is used as a prerequisite to define requirement for simultaneous reception. And the details for Rel-17 group-based reporting could refer to the Reporting Quantity configuration defined in TS 38.214[4] as below.

TS 38.214 5.2.1.4.2 Report Quantity Configurations

From above, we could observe Rel-17 group-based reporting is only for L1-RSRP reporting, and L1-SINR is not in the scope of Rel-17 group-based reporting, so the following proposal is suggested.

Proposal 3: L1-SINR is not in the scope of Rel-17 group-based beam reporting.

## 2.4 Rel-17 Group-based beam reporting for L1-RSRP requirement

In last meeting, some companies raised the requirement related to Rel-17 group-based beam reporting as below issue [1].

Issue 1-3-2-1: How to perform measurement. That means the measurement delay required for GBBR (i.e., measurement delay for beam pair) - Option 1: Reuse legacy L1-RSRP delay as UE do not know which beams can be paired. - Option 2: Due to simultaneous reception measurement delay for beam pair can be enhanced for L1-RSRP Issue 1-3-2-2: How to choose beam pair that are going to be reported based on measurement results - Option 1: define conditions for selecting beam pair in the report - Please provide conditions that can be considered - Option 2: How to choose beam pair is UE implementation Issue 1-3-2-3: NW configures UE with GBBR, but UE did not find a good beam pair to report. What should be the UE behaviour - Option 1: NO behaviour is needed - Option 2: report NA or single beam - Option 3: Check with RAN1 - Note: if we agree for this, we may need to define what are the conditions for beam pair selection as we need to define what is not finding good beam pair. Issue 1-3-6: Type of reporting used for GBBR FFS if Periodic, semi-persistent and aperiodic report are supported

For issues 1-3-2-1, we want to divide the UE behavior into two stages for Rel-17 group-based beam reporting (GBBR) to discuss related L1-RSRP requirement.

Case 1: Before UE send Rel-17 GBBR.

Case 2: After UE send Rel-17 GBBR.

For the case 1, even UE itself does not know whether there is any chance to turn on multiple Rx chains. Therefore, by default we can assume UE will only turn on one active antenna module to measure L1-RSRP for two RSs from different QCL type D, so we can reuse legacy L1-RSRP measurement requirement.

Proposal 4: Before UE sends Rel-17 GBBR, reuse legacy L1-RSRP measurement requirement for Rel-17 GBBR.

For the case 2, after UE sent Rel-17 GBBR, it may turn on two active antenna modules to measure L1-RSRP for two RSs from different QCL type D. But whether UE can enhance the L1-RSRP measurement requirement for case 2 should be discussed further on RAN4.

Proposal 5: After UE sends Rel-17 GBBR, whether UE can enhance L1-RSRP measurement requirement should be discussed further in RAN4.

For issue 1-3-2-2, in our understanding, How UE selects beam pair has not specified in RAN1. (refer to below reference from TS 38.214 [4]). It’s up to UE implementation. So, the following proposal is suggested.

Proposal 6: How UE selects beam pair has not specified in RAN1 and it’s up to UE implementation.

Reference TS 38.214 5.2.1.4.3 L1-RSRP Reporting 5.2.1.4.2 Report Quantity Configurations

For issue 1-3-2-3, we can refer to above reference from TS 38.214 [4]. If NW configured UE with Rel-17 GBBR, but UE did not find a good beam pair, it still shall report beam pair with measured L1-RSRP in single report instance. So, the following proposal is suggested.

Proposal 7: No need to specify the behavior if UE cannot find a good beam pair with NW configured Rel-17 GBBR.

In addition, all type of reporting for Rel-17 GBBR could support from RAN1 perspective for issue 1-3-6. We can observe periodic, semi-persistent, aperiodic report has specified below in TS 38.214. So, the following proposal is suggested.

Proposal 8: All reporting type of periodic, semi-persistent, aperiodic report are supported for Rel-17 GBBR.

Reference TS 38.214 5.2.1.4.1 Resource Setting configuration

# Summary

In this paper, the discussion of R18 multi-Rx chains WI is provided. We have the following proposal:

Observation 1: N cannot be reduced for all UEs because it depends on UE implementation.

Proposal 1: Reducing the value of N in L1 measurement (L1-RSRP/BFD/CBD/RLM) delay requirement should be up to UE’s capability.

Proposal 2: In L1 measurement (L1-RSRP/BFD/CBD/RLM) delay requirement, P factor should not be enhanced due to simultaneous L3 and L1 measurement is not supported in R18.

Proposal 3: L1-SINR is not in the scope of Rel-17 group-based beam reporting.

Proposal 4: Before UE sends Rel-17 GBBR, reuse legacy L1-RSRP measurement requirement for Rel-17 GBBR.

Proposal 5: After UE sends Rel-17 GBBR, whether UE can enhance L1-RSRP measurement requirement should be discussed further in RAN4.

Proposal 6: How UE selects beam pair has not specified in RAN1 and it’s up to UE implementation.

Proposal 7: No need to specify the behavior if UE cannot find a good beam pair with NW configured Rel-17 GBBR.

Proposal 8: All reporting type of periodic, semi-persistent, aperiodic report are supported for Rel-17 GBBR

# Reference

- R4-2306319, WF on FR2_multiRx_part2, Ericsson, RAN4 #106-bis-e

- R4-2302768, Topic summary for [106bis-e][203] FR2_multiRx_part2, Ericsson, RAN#106-bis-e

- RP-223527, Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Incorporated, RAN #98e

- NR Physical layer procedures for data (3GPP TS 38.214 version 17.3.0)