3GPP TSG-RAN4 Meeting #107	R4-2308535

# Incheon, Korea, 22-26 May 2023

Source:				Ericsson

Title:					NTN enhancements – System parameters

Agenda item:		8.27.1.1

Document for:		Approval

# Introduction

This document discusses the system parameters and bands definition in the scope of the NTN enhancements WI ([1]), based on the agreed Way Forward [2] from last RAN4#106-bis meeting.

# Discussion

## Channel arrangement

### Channel raster

In last RAN4#106-bis meeting, the way forward XX has captured the following agreement:

We further checked the proposed boundaries and think they should be updated, they don’t look correct as they are not aligned with the raster numbers with step size of 4 and 8.

We would make the following proposal instead:

Proposal1: Specify the following channel raster NREF values for bands n512, n511 and n510.

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 60 | 2070833 – <1> – 2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2112499 | 1553336 – <8> – 1746664 |
| n511 | 60 | 2084999 – <1> –2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2084999 – <2> –2112499 | 1553336 – <8> – 1746664 |
| n510 | 60 | 2070833 – <1> – 2084999 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2084999 | 1553336 – <8> – 1746664 |

### Synchronization raster

In last RAN4#106-bis meeting, the way forward XX has captured the following agreement:

We further checked the proposed intervals 12 and 24 and we would agree with those values.

Nevertheless, we think the boundaries are not correct, they would need further discussion. Indeed, the proposed values are not multiple of 12 or 24.

To our understanding, the band starting at 17.3 GHz, the Nmin value should be 976, and the next GSCN number multiple of 12, which would be the first GSCN value for that band,  should then be 17448 (instead of 17444). Accordingly, the last GSNC value should then be 19428 (instead of 19424).

Following a similar reasoning for a GSCN step size of 24, the GSCN values boundaries should then be 17472 and 19416.

Proposal2: Specify n512, n511 and n510 synchronization raster entries according to following table:

| NTN operating band | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |

# Conclusion

In this contribution, we discussed the system parameters to be used for the NTN satellite bands agreed in last RAN4 meeting. We made the following proposals:

Proposal1: Specify the following channel raster NREF values for bands n512, n511 and n510.

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 60 | 2070833 – <1> – 2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2112499 | 1553336 – <8> – 1746664 |
| n511 | 60 | 2084999 – <1> –2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2084999 – <2> –2112499 | 1553336 – <8> – 1746664 |
| n510 | 60 | 2070833 – <1> – 2084999 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2084999 | 1553336 – <8> – 1746664 |

Proposal2: Specify n512, n511 and n510 synchronization raster entries according to following table:

| NTN operating band | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |

# Reference

- RP-230809, NR NTN (Non-Terrestrial Networks) enhancements, Thales

- R4-2305925, WF for system parameters of above 10 GHz NTN band, Thales