3GPP TSG-RAN WG4 Meeting # 107                               R4-2308570

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on ATG UE Tx requirements

Agenda Item:	8.14.2.2

Document for:	Approval

Introduction

In RAN4#106bis meeting, a way forward [1] was approved with some progress. In this paper, we’d like to provide our views on ATG UE Tx requirements for the other requirements.

Discussion on maximum output power requirements

In last meeting, RAN4 has the following agreements on maximum output power requirements.

Sub-topic 1-2 The maximum output power Agreement: - New capability can be introduced for ATG UE to indicate [the minimum of maximum output power] at maximum modulation order and full PRB configurations. - The capability report granularity is 1dB. Agreement: - Upper limit for new capability for maximum conductive output power or TRP - 40dBm Agreement: - Introduce two UE types to distinguish the antenna types for ATG UE - <Way forward>:FFS on accuracy/ tolerance. - <Way forward>:FFS whether and/or how to improve the term of [the minimum of maximum output power].

For the term [the minimum of maximum output power], it seems very confusing. The lower limit of Pcmax can be specified in configured transmitted power. In addition, the lower limit of Pcmax was also restricted by Pemax which was signalled by network. Generally, RAN4 just specify the output power at specific condition.

Proposal 1: [the minimum of maximum output power] can be replaced by “Rated output power”, i.e. mean power level indicated by UE at maximum modulation order and full PRB configurations and associated with a particular operating band.

In RAN4#105 meeting, we have the following way forward for accuracy/ tolerance [2].

<Way forward >: RAN4 can specify a range of ATG UE MOP, e.g.. 29dBm ~ FFS. ATG UE can indicate its MOP by using UE capability. The tolerance of ATG UE MOP can be ±2dB.

Although the “[the minimum of maximum output power]” can be indicated by ATG UE, RAN4 still need to specify accuracy requirements to guarantee that ATG UE report this value accurately. Thus, it’s necessary to specify the accuracy requirements for ATG UE “conductive output power or TRP”.

Proposal 2: The [the minimum of maximum output power] at maximum modulation order and full PRB configurations shall remain with +2 dB and -2 dB of the new capability as reported by ATG UE.

In addition, The IE P-Max is specified in TS 38.331 as below. The range of this IE is from -30 to 33dBm.

It’s obvious that 33dBm is enough for ATG scenario to limit the UE’s uplink transmission power. RAN4 can further discuss whether we can extend the range of P-Max or a similar new IE with larger range can be introduced for ATG scenario.

Proposal 3: The range of this IE P-Max from -30 to 33dBm is not enough for ATG UE. Whether to extend the range of P-Max or introduce a similar new IE with larger range for ATG can be further discussed.

Discussion on Configured transmitted power

Since RAN4 has agreed not to specify MPR/AMPR requirements and ATG UE need to indicate the maximum output power at maximum modulation order and full PRB configurations, the Pcmax for ATG UE can be expressed as below. Otherwise, if we don’t specify the configured output power of ATG UE, network signalling Pemax can’t restrain the output power of ATG UE for compliance with regulation.

Proposal 4: it’s proposed to define the following Pcmax,f,c. Otherwise, network signalling Pemax can’t restrain the output power of ATG UE for compliance with regulation.

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c, PMaxOutputPowerClass}

PCMAX_H,f,c = PEMAX,c

where

PEMAX,c is the value given by [either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE], whichever is applicable according to TS 38.331[7];

PMaxOutputPowerClass is the maximum UE output power at maximum modulation order and full PRB configurations which is indicated by ATG UE;

Discussion on Minimum output power

In last meeting, we have the following way forward for minimum output power requirements.

Sub-topic 1-3 Minimum output power for ATG Candidate options: - Option 2: Considering some margins, the minimum output power for ATG could be written as below with 25Db relaxation: Channel bandwidth; (MHz); 5,10,15,20; 25,30,35,40,45,50; 60,70,80,90,100. REF_SCS; (kHz); 15; 30. Minimum output power; (dBm); -40+25; -40+25+10log10 (BWChannel /20); -40+25+10log10 (BWChannel /20). Measurement bandwidth; (MHz); MBW=REF_SCS*(12*NRB+1)/1000. NOTE: The minimum output power value is rounded to the nearest number down to one decimal point.. - Option 3: Define UE minimum output power to be -25 dBm for 2 GHz, and -20 dBm for 4 GHz. - Option 5: the minimum output power for ATG UE could be [-3dBm/100MHz] and scale with other BW. <Way forward>: FFS the assumption of minimum distance between ATG BS and ATG UE to derive the minimum output power.

According to the contribution [3, 4], companies provided some technical judgements and analysis about the minimum output power.

Minimum output power for ATG UE + antenna gain for ATG UE + antenna gain for BS – pathloss(2GHz, 3000m) >= RESENS of BS. And the assumptions for each parameter are shown below.

Antenna gain for ATG UE: 10 dB.

Antenna gain for BS: 23dB

Pathloss(2GHz, 3000m) = 32.5 +20log10(2)+20log10(3000)=108dB

REFSENS for BS 5MHz = -101.7 dBm

So Minimum output power for ATG UE is -27dBm.

Since 20km minimum distance between ATG BS and UE is assumed for co-existence study, generally, X/Y can’t be larger than 10dB.

Proposal 5: To specify Minimum output power for ATG UE with following requirements.

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -20 | -20+10log10 (BWChannel /20) | -20+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

- 3 Summary

Based on the discussion, all the observations and proposals are listed below:

Proposal 1: [the minimum of maximum output power] can be replaced by “Rated output power”, i.e. mean power level indicated by UE at maximum modulation order and full PRB configurations and associated with a particular operating band.

Proposal 2: The [the minimum of maximum output power] at maximum modulation order and full PRB configurations shall remain with +2 dB and -2 dB of the new capability as reported by ATG UE.

Proposal 3: The range of this IE P-Max from -30 to 33dBm is not enough for ATG UE. Whether to extend the range of P-Max or introduce a similar new IE with larger range for ATG can be further discussed.

Proposal 4: it’s proposed to define the following Pcmax,f,c. Otherwise, network signalling Pemax can’t restrain the output power of ATG UE for compliance with regulation.

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c, PMaxOutputPowerClass}

PCMAX_H,f,c = PEMAX,c

where

PEMAX,c is the value given by [either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE], whichever is applicable according to TS 38.331[7];

PMaxOutputPowerClass is the maximum UE output power at maximum modulation order and full PRB configurations which is indicated by ATG UE;

Proposal 5: To specify Minimum output power for ATG UE with following requirements.

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -20 | -20+10log10 (BWChannel /20) | -20+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

- References

[1] R4-2306609, WF on ATG UE RF requirements, Huawei, HiSilicon

[2] R4-2220543, WF on ATG terminal RF requirement, Huawei, HiSilicon

[3] R4-2218694, ATG UE Tx RF requirement, Apple

[4] R4-2219612, Discussion on ATG UE Tx requirements, Huawei, HiSilicon