3GPP TSG-RAN WG4 Meeting # 107                               R4-2308579

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on RF impacts for R18 RedCap UE

Agenda Item:	8.32.2

Document for:	Approval

Introduction

In RAN4#106bis meeting, three remaining issues are listed below to be further discussed referring to WF [1].

Firstly, two kinds of Rel-18 eRedCap UE (20MHz + PR1 vs BW3/PR3 + PR1) are designed based on RAN plenary’s agreement [2]. RAN4 may need to discuss how to treat them.

Secondly, for PR3 eRedCap UE, it’s discussed whether PRB location should be restricted for REFSENS or blocking requirements.

Thirdly, for DL FRC, some updates are needed due to DL PRB limitation and peak rate limitation.

In this paper, we’d like to share our views about these issues.

Discussion on Rel-18 eRedCap UE (20MHz + PR1 vs BW3/PR3 + PR1)

In RAN#99 meeting, two kinds of Rel-18 eRedCap UE (20MHz + PR1 vs BW3/PR3 + PR1) are designed based on RAN plenary’s agreement [2].

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively. Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

Based on TR 38.865 [3], RAN1 has defined Peak rate reduction (PR1/PR3) as below.

- Option PR1: Relaxation of the constraint for peak data rate reduction. - The relaxed constraint is 1 (instead of 4). - The parameters (, , ) can be as in Rel-17 RedCap.

Option PR3: Restriction of maximum number of PRBs for PDSCH and PUSCH. - For 15 kHz SCS, the maximum number of RBs is 25. - For 30 kHz SCS, the maximum number of RBs is 12. (Updated based on latest RAN1’s agreement) - The restricted number of PRBs in Option PR3 is a hardcoded limit.

Thus, we have the following observations on Rel-18 eRedCap UE (20MHz + PR1 vs BW3/PR3 + PR1)

Observation 1: the characteristics are listed below for Rel-17 RedCap UE and Rel-18 eRedCap UE (20MHz + PR1 vs BW3/PR3 + PR1)

|  | Rel-17 RedCap UE | Rel-18 eRedCap UE (20MHz + PR1) | Rel-18 eRedCap UE (BW3/PR3 + PR1) |
| --- | --- | --- | --- |
| UE RF channel BW | Up to 20MHz | Up to 20MHz | Up to 20MHz |
| PRB restriction | No PRB restriction | No PRB restriction | For 15 kHz SCS, the maximum number of RBs is 25. For 30 kHz SCS, the maximum number of RBs is 12 |
| Peak data rate reduction | No peak data rate restriction | Targeted to same peak data rate, i.e. probably 10.67Mbps (wait for RAN1’s final decision to get the precise value.) |  |

Proposal 1: for Rel-18 eRedCap UE (20MHz + PR1), the requirements for Rel-17 RedCap UE can be reused, but to restrict the peak data rate, e.g. 64QAM DL FRC for maximum input level can be updated to meet the maximum peak data rate.

Proposal 2: for Rel-18 eRedCap UE (BW3/PR3 + PR1), the REFSENS can be scaled compared to Rel-17 RedCap UE because of maximum 25 RBs for 15kHz SCS and 12 RBs for 30kHz SCS.

Discussion on PRB location for REFSENS and blocking requirements

PRB location for blocking requirements

In previous meetings, companies raised that PRB location will have an impact on the related blocking requirements. The in-band blocking requirements are used to verify the linearity performance of Rx chain, i.e. IP2. In fact, some unwanted products from IP2 are not very related to the PRB location as it is caused by interference signal itself depicted as below. In addition, RAN4 assume the IM2 distortion is flat with some margin when blocking requirements were specified. That means the Interference unevenness of some nonlinear products have been considered.

Figure 1 2nd order non-linearity in direct-onversion receivers

Proposal 3: for blocking requirements, there is no need to restrict the PRB location since Interference unevenness of some nonlinear products have been considered with some margin when blocking requirements were specified.

PRB location for REFSENS requirements

In previous meetings, companies proposed that not only the REFSENS for Rel-18 PR3 eRedCap UE should be scaled, but also PRB location should be restricted in the middle of the channel for some FDD bands. For the scaling factor for 10MHz, 15MHz and 20MHz, the following values can be considered based on the contribution [3].

Table 1 Scaling factor for 10MHz, 15MHz, 20MHz RF bandwidths

|  | Scaling factor |  |  |  |
| --- | --- | --- | --- | --- |
| RF bandwidth | 5MHz | 10MHz | 15MHz | 20MHz |
| 15kHz | -10log25/25=0 | -10log52/25=-3.2 | -10log79/25=-5 | -10log106/25=-6.3 |
| 30KHz |  | -10log24/12=-3 | -10log38/12=-5 | -10log51/12=-6.3 |

Since the concerns from companies are only for FDD bands, we’d like to check the current REFSENS requirements for FDD bands. The highlighted REFSENS may have some degradation due to the interference from Tx signal and smaller duplex frequency gap. Generally, RAN4 use the following equation to derive the REFSENS requirements considering the noise figure and others which were captured in the implementation margin.

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain

For the thermal noise, the same/flat PSD interference is assumed spanning the whole channel.

Observation 2: for the REFSENS without any degradation from Tx signal, the same/flat PSD thermal noise/interference is assumed as spanning the whole channel.

For the REFSENS degradation evaluation, RAN4 always assume the interference PSD from Tx 2nd adjacent channel (Currently, there is no 1st adjacent channel interference for maximum 20MHz) is flat/same with some margin and UL RB limitation, even if the real interference is not flat.

Observation 3: for the REFSENS with some degradation from Tx 2nd adjacent channel, companies are encouraged to investigate whether the degradation is enough when assuming the interference PSD from Tx 2nd adjacent channel is flat/same with some margin and UL RB limitation.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |  |  |  |  |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
| n2 | 15 | -98 | -94.8 | -93 | -91.8 |  |  |  |  |  |  |
|  | 30 |  | -95.1 | -93.1 | -92 |  |  |  |  |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 |  |  |  |  |  |  |
| n3 | 15 | -97.0 | -93.8 | -92.0 | -90.8 |  |  |  |  |  |  |
|  | 30 |  | -94.1 | -92.1 | -91.0 |  |  |  |  |  |  |
|  | 60 |  | -94.5 | -92.4 | -91.2 |  |  |  |  |  |  |
| n5 | 15 | -98.0 | -94.8 | -93.0 | -86.8 |  |  |  |  |  |  |
|  | 30 |  | -95.1 | -93.1 | -88.6 |  |  |  |  |  |  |
| n71 | 15 | -98.0 | -94.8 | -93.0 | -91.8 |  |  |  |  |  |  |
|  | 30 |  | -95.1 | -93.1 | -92.0 |  |  |  |  |  |  |
|  | 60 |  | -95.5 | -93.4 | -92.2 |  |  |  |  |  |  |
| n8 | 15 | -97.0 | -93.8 | -91.4 | -85.8 |  |  |  |  |  |  |
|  | 30 |  | -94.1 | -91.7 | -87.2 |  |  |  |  |  |  |
| n12 | 15 | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 | -84.1 |  |  |  |  |  |  |  |
| n13 | 15 | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 |  |  |  |  |  |  |  |  |
| n14 | 15 | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 |  |  |  |  |  |  |  |  |
| n18 | 15 | -100.0 | -96.8 | -95.0 |  |  |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 |  |  |  |  |  |  |  |
| n20 | 15 | -97.0 | -93.8 | -91.0 | -89.8 |  |  |  |  |  |  |
|  | 30 |  | -94.1 | -91.1 | -90.0 |  |  |  |  |  |  |
| n24 | 15 | -100.0 | -96.8 |  |  |  |  |  |  |  |  |
|  | 30 |  | -97.1 |  |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 |  |  |  |  |  |  |  |  |
| n25 | 15 | -96.5 | -93.3 | -91.5 | -90.3 |  |  |  |  |  |  |
|  | 30 |  | -93.6 | -91.6 | -90.5 |  |  |  |  |  |  |
|  | 60 |  | -94.0 | -91.9 | -90.7 |  |  |  |  |  |  |
| n26 | 15 | -97.56 | -94.56 | -92.76 | -87.6 |  |  |  |  |  |  |
|  | 30 |  | -94.86 | -92.76 | -87.7 |  |  |  |  |  |  |
| n28 | 15 | -98.5 | -95.5 | -93.5 | -90.8 |  |  |  |  |  |  |
|  | 30 |  | -95.6 | -93.6 | -91.0 |  |  |  |  |  |  |
| n30 | 15 | -99.0 | -95.8 |  |  |  |  |  |  |  |  |
|  | 30 |  | -96.1 |  |  |  |  |  |  |  |  |
| n65 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
| n66 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n70 | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
| n71 | 15 | -97.2 | -94.0 | -91.6 | -86.0 |  |  |  |  |  |  |
|  | 30 |  | -94.3 | -91.9 | -87.4 |  |  |  |  |  |  |
| n74 | 15 | -99.53 | -96.33 | -94.53 | -89.33 |  |  |  |  |  |  |
|  | 30 |  | -96.63 | -94.63 | -89.53 |  |  |  |  |  |  |
|  | 60 |  | -97.03 | -94.93 | -89.63 |  |  |  |  |  |  |
| n85 | 15 | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  | 30 |  | -94.1 | -84.1 |  |  |  |  |  |  |  |
| n100 | 15 | -100 |  |  |  |  |  |  |  |  |  |
| n105 | 15 | -97.28 | -94.0 | -91.6 | -86.9 |  |  |  |  |  |  |
|  | 30 |  | -94.3 | -91.9 | -87.9 |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: Void. NOTE 8: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS |  |  |  |  |  |  |  |  |  |  |  |

Proposal 4: To use the following scaling factor as baseline to derive the REFSENS for Rel-18 PR3 eRedCap UE. FFS whether to restrict PRB location for the REFSENS with some degradation from Tx 2nd adjacent channel interference.

|  | Scaling factor |  |  |  |
| --- | --- | --- | --- | --- |
| RF bandwidth | 5MHz | 10MHz | 15MHz | 20MHz |
| 15kHz | -10log25/25=0 | -10log52/25=-3.2 | -10log79/25=-5 | -10log106/25=-6.3 |
| 30KHz |  | -10log24/12=-3 | -10log38/12=-5 | -10log51/12=-6.3 |

Discussion on FRC updates.

In last meeting, RAN4 agreed to modify FRC table for QPSK and 64QAM. In this meeting, we’d like to discuss the details.

FRC for 64QAM

For 64QAM, since peak data rate may exceed the 10Mbps, the MCS index can be updated to meet this requirements. Referring to TS 38.214, RAN1 has the following MCS index table. MCS index 22 can be used to meet the peak data rate.

Table A.3.2.3-1 Fixed reference channel for maximum input level receiver requirements (SCS 15 kHz, FDD, 64QAM)

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 5, 10, 15, 20 (NOTE 5) |
| Subcarrier spacing | kHz | 15 |
| Subcarrier spacing configuration |  | 0 |
| Allocated resource blocks |  | 25 |
| Subcarriers per resource block |  | 12 |
| Allocated slots per Frame |  | 8 |
| MCS Index |  | 22 |
| MCS Table for TBS determination |  | 64QAM |
| Modulation |  | 64 QAM |
| Target Coding Rate |  | 0.6504 |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload per Slot |  |  |
| For Slots 0,1 | Bits | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 10504 |
| Transport block CRC | Bits | 24 |
| LDPC base graph |  | 1 |
| Number of Code Blocks per Slot |  |  |
| For Slot 0,1 | CBs | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 2 |
| Binary Channel Bits per Slot |  |  |
| For Slot 0,1 | Bits | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 16200 |
| Max. Throughput averaged over 1 frame | Mbps | 8.4032 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: The FRC in this column is only used for [eRedCap PR3 UE] introduced in Rel-18. |  |  |

Table A.3.2.3-2 Fixed reference channel for maximum input level receiver requirements (SCS 30 kHz, FDD, 64QAM)

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 10, 15, 20 (NOTE 5) |
| Subcarrier spacing | kHz | 30 |
| Subcarrier spacing configuration |  | 1 |
| Allocated resource blocks |  | 1112 |
| Subcarriers per resource block |  | 12 |
| Allocated slots per Frame |  | 17 |
| MCS Index |  | 2422 |
| MCS Table for TBS determination |  | 64QAM |
| Modulation |  | 64 QAM |
| Target Coding Rate |  | 0.6504 |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload per Slot |  |  |
| For Slots 0,1 | Bits | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 4992 |
| Transport block CRC | Bits | 24 |
| LDPC base graph |  | 1 |
| Number of Code Blocks per Slot |  |  |
| For Slot 0,1 | CBs | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 1 |
| Binary Channel Bits per Slot |  |  |
| For Slot 0,1 | Bits | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 7776 |
| Max. Throughput averaged over 1 frame | Mbps | 8.486 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: The FRC in this column is only used for [eRedCap PR3 UE] introduced in Rel-18. |  |  |

Proposal 5: For Rel18 eRedCap PR3 UE, MCS index 22 in 64QAM table can be used for maximum input level.

FRC for QPSK

Since 12 RB is used for 30kHz SCS, so new FRC for 30kHz QPSK 10/15/20MHz should be added.

Table A.3.2.2-2 Fixed reference channel for receiver requirements (SCS 30 kHz, FDD, QPSK 1/3)

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 10, 15, 20 (NOTE 5) |
| Subcarrier spacing | kHz | 30 |
| Subcarrier spacing configuration |  | 1 |
| Allocated resource blocks |  | 12 |
| Subcarriers per resource block |  | 12 |
| Allocated slots per Frame |  | 17 |
| MCS Index |  | 4 |
| MCS Table for TBS determination |  | 64QAM |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload per Slot |  |  |
| For Slots 0,1 | Bits | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 808 |
| Transport block CRC | Bits | 16 |
| LDPC base graph |  | 2 |
| Number of Code Blocks per Slot |  |  |
| For Slot 0,1 | CBs | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 1 |
| Binary Channel Bits per Slot |  |  |
| For Slot 0,1 | Bits | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 2592 |
| Max. Throughput averaged over 1 frame | Mbps | 1.374 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: The FRC in this column is only used for [eRedCap PR3 UE] introduced in Rel-18. |  |  |

Proposal 6: New DL FRC (25RB, 15 kHz SCS) for 10, 15, 20MHz and (12RB, 30 kHz SCS) for 10, 15, 20MHz can be introduced in A3.2.2, A3.2.3 and A3.2.4 for FDD and in A3.3.2, A3.3.3 and A3.3.4 for TDD to support Rx requirements test for Rel18 eRedCap UE.

Summary

Observation 1: the characteristics are listed below for Rel-17 RedCap UE and Rel-18 eRedCap UE (20MHz + PR1 vs BW3/PR3 + PR1)

|  | Rel-17 RedCap UE | Rel-18 eRedCap UE (20MHz + PR1) | Rel-18 eRedCap UE (BW3/PR3 + PR1) |
| --- | --- | --- | --- |
| UE RF channel BW | Up to 20MHz | Up to 20MHz | Up to 20MHz |
| PRB restriction | No PRB restriction | No PRB restriction | For 15 kHz SCS, the maximum number of RBs is 25. For 30 kHz SCS, the maximum number of RBs is 12 |
| Peak data rate reduction | No peak data rate restriction | Targeted to same peak data rate, i.e. probably 10.67Mbps (wait for RAN1’s final decision to get the precise value.) |  |

Proposal 1: for Rel-18 eRedCap UE (20MHz + PR1), the requirements for Rel-17 RedCap UE can be reused, but to restrict the peak data rate, e.g. 64QAM DL FRC for maximum input level can be updated to meet the maximum peak data rate.

Proposal 2: for Rel-18 eRedCap UE (BW3/PR3 + PR1), the REFSENS can be scaled compared to Rel-17 RedCap UE because of maximum 25 RBs for 15kHz SCS and 12 RBs for 30kHz SCS.

Proposal 3: for blocking requirements, there is no need to restrict the PRB location since Interference unevenness of some nonlinear products have been considered with some margin when blocking requirements were specified.

Observation 2: for the REFSENS without any degradation from Tx signal, the same/flat PSD thermal noise/interference is assumed as spanning the whole channel.

Observation 3: for the REFSENS with some degradation from Tx 2nd adjacent channel, companies are encouraged to investigate whether the degradation is enough when assuming the interference PSD from Tx 2nd adjacent channel is flat/same with some margin and UL RB limitation.

Proposal 4: To use the following scaling factor as baseline to derive the REFSENS for Rel-18 PR3 eRedCap UE. FFS whether to restrict PRB location for the REFSENS with some degradation from Tx 2nd adjacent channel interference.

|  | Scaling factor |  |  |  |
| --- | --- | --- | --- | --- |
| RF bandwidth | 5MHz | 10MHz | 15MHz | 20MHz |
| 15kHz | -10log25/25=0 | -10log52/25=-3.2 | -10log79/25=-5 | -10log106/25=-6.3 |
| 30KHz |  | -10log24/12=-3 | -10log38/12=-5 | -10log51/12=-6.3 |

Proposal 5: For Rel18 eRedCap PR3 UE, MCS index 22 in 64QAM table can be used for maximum input level.

Proposal 6: New DL FRC (25RB, 15 kHz SCS) for 5, 10, 15, 20MHz and (12RB, 30 kHz SCS) for 10, 15, 20MHz can be introduced in A3.2.2, A3.2.3 for FDD and in A3.3.2, A3.3.3 for TDD to support Rx requirements test for Rel18 eRedCap PR3 UE.

- References

[1] R4-2306638, WF on eRedCap UE RF requirements, Ericsson

[2] RP-230778, Proposal for PR1 in eRedCap, Moderator (CMCC)

[3] R4-2305072, Discussion on UE RF requirements for eRedcap, Xiaomi

RP-223544, Revised WID on Enhanced support of reduced capability NR devices, Ericsson