3GPP TSG-RAN WG4 Meeting # 107                               R4-2308582

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon, Spark NZ

Title: 	Discussion on RF architecture and potential MSD issues for CA_n28-n105

Agenda Item:	8.4.2.2

Document for:	Approval

Introduction

In RAN4#106bis meeting, we have the following WF [1] on RF architecture and critical issues for CA_n28-n105. Since 2UL IMD3/IM5 may fall into either n105 DL or n28 DL or both DL bands simultaneously to cause REFSENS degradation (MSD), it’s agreed to only consider single UL configuration in the study item where UL can be in either n28 or n105. Thus, the remaining issues are related to delta Tib / Rib and MSD study due to cross band isolation. In this paper, we’d like to discuss these parts.

Discussion on Delta Tib and Rib

Based on the discussion in last meeting, it seems that two-antenna solutions with triplexers below can be assumed to derive Tib and Rib for CA_n28-n105 as the feasibility with quadplexers is questioned by companies.

Figure 1 2-antenna implementation with dual triplexers in main path and a triplexer in diversity path

As the frequency gap between n105 UL and n28 DL is about 45MHz, the difficulty of implementing triplexer n105 duplxer + n28 DL filter is not much more than the triplexer n5 duplxer + n28 DL filter.

As the frequency gap between n28 UL and n105 DL is about 51MHz, the difficulty of implementing triplexer n28 duplxer + n105 DL filter is not much more than the triplexer n28A UL + (n20/n28 DL) + n20 UL.

Comparing to the Tib and Rib of CA_n5-n28 and CA_n20-n28, 0.7dB Tib for both n28&n105 and 0.2dB Rib for both n28&n105 can be used.

Proposal 1: The following Delta Tib and Rib values for CA_n28-n105 can be considered.

Table 1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28A-n105A | n28 | 0.7 |
|  | n105 | 0.7 |

Table 2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n28A-n105A | n28 | 0.2 |
|  | n105 | 0.2 |

Discussion on MSD due to cross band isolation

n28 UL => n105 DL

Referring to contribution [2], the 2nd adjacent channel interference from 30MHz band n28 UL may fall into the frequency range 643~652MHz. n28 Tx filter rejection at 612~652MHz is assumed as 30~40 dB. Three-antenna RF architecture is assumed for MSD evaluation.

We have the following evaluation for MSD due to cross band isolation for CA_n28-n105.

Table 3 MSD due to cross band isolation n28 => n105

|  | parameters | n105 main path | n105 diversity path |
| --- | --- | --- | --- |
| transmit power for n28, dBm |  | 23 | 23 |
| Tx band n28 BW(MHz, Lcrb) | 30 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n28 uplink freq，dB | 10 |  |  |
| antenna isolation, dB | 10 |  |  |
| n105 receival singal at ANT port, dBm |  | 13 | 13 |
| n105 filter rejection at 703~733MHz, dB | 40 |  |  |
| signal After n105 filter, dBm |  | -31 | -31 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -112 | -112 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -108 | -108 |
| noise figure dB | 11.8 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -162.2 |  |  |
| Rx band n105 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -95.67 | -95.67 |
| ACLR2, dB | 43 |  |  |
| n28 PA leakage PSD at PA output port at 2nd adjacent channel, dBm/MHz |  | -34.77 |  |
| n28 PA noise PSD at PA output port, dBm/RxBW |  | -28.24 |  |
| n28 Tx filter rejection at 612~652MHz, dB | 40 |  |  |
| n28 PA noise power at Rx n105 ant port at 612~652MHz, dBm |  | -78.24 | -78.24 |
| Total noise level at ANT port |  | -78.16 | -78.16 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -79.16 | -79.16 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(5MHz BW), dBm | -80.17 |  |  |
| MSD | 17.03 |  |  |

Proposal 2: Based on the evaluation, the REFSENS degradation due to cross band isolation from 30MHz n28 UL to n105 DL can be considered below.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n28 | n105 | 718 | 30 | 15 | 25 (RBstart=0) | 649.5 | 5 | [17.0] | >ACLR |

n105 UL => n28 DL

Referring to contribution [2], the 3rd and 4th adjacent channel interference from 20MHz band n105 UL may fall into the frequency range 758~803MHz. -120dBm/Hz PA noise PSD at PA output port can be assumed. n105 Tx filter rejection at 758~803 MHz is assumed as 40 dB. Three-antenna RF architecture is assumed for MSD evaluation.

Table 4 MSD due to cross band isolation n105 => n28

|  | parameters | n28 main path | n28 diversity path |
| --- | --- | --- | --- |
| transmit power for n105, dBm |  | 23 | 23 |
| Tx band n105 BW(MHz, Lcrb) | 20 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n105 uplink freq，dB | 10 |  |  |
| antenna isolation, dB | 10 |  |  |
| n28 receival singal at ANT port, dBm |  | 13 | 13 |
| n28 filter rejection at 663~703MHz, dB | 40 |  |  |
| signal After n28 filter, dBm |  | -31 | -31 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -112 | -112 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -108 | -108 |
| noise figure dB | 10.5 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -163.5 |  |  |
| Rx band n5 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -96.97 | -96.97 |
| n105 PA noise PSD at PA output port, dBm/Hz | -120 |  |  |
| n105 PA noise PSD at PA output port, dBm/RxBW |  | -53.47 |  |
| n105 Tx filter rejection at 758~803MHz, dB | 40 |  |  |
| n105 PA noise power at Rx n105 ant port at 758~803MHz, dBm |  | -103.47 | -103.47 |
| Total noise level at ANT port |  | -95.82 | -95.82 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -96.82 | -96.82 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(5MHz BW), dBm | -97.83 |  |  |
| MSD | 0.67 |  |  |

Proposal 3: Based on the evaluation, the REFSENS degradation is neglect for MSD from n105 UL => n28 DL with 40dB Tx filter rejection. Thus, there is no need to specify MSD due to cross band isolation from n105 UL => n28 DL.

- Summary

Proposal 1: The following Delta Tib and Rib values for CA_n28-n105 can be considered.

Table 1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28A-n105A | n28 | 0.7 |
|  | n105 | 0.7 |

Table 2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n28A-n105A | n28 | 0.2 |
|  | n105 | 0.2 |

Proposal 2: Based on the evaluation, the REFSENS degradation due to cross band isolation from 30MHz n28 UL to n105 DL can be considered below.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n28 | n105 | 718 | 30 | 15 | 25 (RBstart=0) | 649.5 | 5 | [17.0] | >ACLR |

Proposal 3: Based on the evaluation, the REFSENS degradation is neglect for MSD from n105 UL => n28 DL with 40dB Tx filter rejection. Thus, there is no need to specify MSD due to cross band isolation from n105 UL => n28 DL.

- References

[1] R4-2306469, WF on RF architecture and critical issues for CA_n28-n105, Apple

[2] R4-2304561, Input on new CA_n28A-n105A band combinations study, Skyworks Solutions, Inc.