3GPP TSG-RAN WG4 Meeting # 107                               		R4-2308586

Incheon, KR, May 22 – May 26, 2023

Agenda item:	7.32.3

Source: 	ZTE Corporation

Title: 	Discussion on UE RF requirements for the satellite L-/S-band

Document for:	Approval

Introduction

In RAN#98e meeting, the work item [RP-223485] on Introduction of the satellite L-/S-band was approved. Part of the objective is as follows:

-	Specify a new NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz;

-	Support channel bandwidths and SCS as presented in Table 4.1-1 below;

-	Support channel raster points at step of 100kHz;

NOTE: Support for channel raster points at step of 5kHz can be considered later based on the outcome of the corresponding RAN WG4 for potential channel raster enhancements.

-	Support UE Power class 3 (+23dBm);

NOTE: Work on UE Power class 2 can commence based on the RAN#99 decision.

-	Introduce the corresponding SAN and UE RF core requirements;

-	Introduce the corresponding RRM requirements.

Table 4.1-1: Channel bandwidth and SCS system parameters.

| SCS (kHz) | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
| 15 | 5 | 10 | 15 |  |
| 30 |  | 10 | 15 |  |
| 60 |  | 10 | 15 |  |

- In this contribution, we want to share some initial views on UE RF requirements for Introduction of the satellite L-/S-band.

Discussion

## 2.1. Operating bands

The following agreement has been approved to reserve band n254 for WI NR_NTN_Lsband.

Agreement: - n254 is reserved for the band defined for WI NR_NTN_LSband

According to the regulation of band number that the NTN satellite bands are numbered in descending order from n256, the reserved band number n254 should be applied to the new NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz.

Table 2.1-1: NTN satellite bands in FR1

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n254 | 1610 MHz-1626.5MHz | 2483.5MHz-2500MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from n256. |  |  |  |

Proposal 1: The NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz should be numbered as Table 2.1-1.

## 2.2 Asymmetric channel bandwidths

The introduction of asymmetric channel bandwidth combinations can reduce the complexity of UE implementation. If the asymmetric channel combinations 5+10MHz, 5+15MHz, and 10+15MHz UL/DL are necessary, we can support the introduction of asymmetric channel bandwidth which under the same principle defined in section 5.3.6 in TS 38.101-1.

Proposal 2: The asymmetric channel bandwidth combinations 5+10MHz, 5+15MHz, and 10+15MHz UL/DL can be introduced under the same principle defined in section 5.3.6 in TS 38.101-1.

## 2.3 Synchronization raster entries for each operating band

The detailed spectrum status between NTN band and surrounding TN bands are shown as following Figure 2.1-1.

Figure 2.1-1 NTN band and surrounding TN bands

Based on the above adjacent spectrum for NTN band, the applicable SS Block SCS and SS Block pattern of TN band can be reused. For example, the applicable SS Block SCS and SS Block pattern of band n24 can be reused to n255. Similarly, the applicable SS Block SCS and SS Block pattern of band n41 can be reused to n254. The applicable SS Block SCS and SS Block pattern of TN band n24/41/66 and NTN band n256/255 are listed in Table 2.3-1 and Table 2.3-2.

Table 2.3-1: Applicable SS raster entries per operating band

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n24 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n41 | 15 kHz | Case A | 6246 – <3> – 6717 |
|  | 30 kHz | Case C | 6252 – <3> – 6714 |
| n66 | 15 kHz | Case A | 5279 – <1> – 5494 |
|  | 30 kHz | Case B | 5285 – <1> – 5488 |

Table 2.3-2: Applicable SS raster entries per operating band

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

Therefore, the synchronization raster for operating bands n254 is specified in Table2.3-3.

Table 2.3-3: Applicable SS raster entries per operating band

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6218 – <1> – 6241 |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

Proposal 3: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the synchronization raster of SS Block pattern Case C also should be added as Table 2.3-3.

## 2.4 TX–RX frequency separation

As a starting point, the default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for operating bands n254 can be specified as Table 2.4-1.

Table 2.4-1: UE TX-RX frequency separation

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n254 | 873.5 MHz |

Proposal 4: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the TX–RX frequency separation defined in Table 2.4-1 can be used as the starting point.

## 2.5 Reference sensitivity level

The following agreement has been approved at last meeting:

Agreement： - Reference sensitivity: - n53 REFSENS + "insertion loss" dB; where potential options for the insertion loss are as follows - Option 1: 0dB (resulting in same REFSENS as band n53); - Option 2: 0.5dB (resulting in same REFSENS as band n255); - Other options are not excluded.

For the reference sensitivity level of band n254, it can be derived based on current band n256 or n53. Considering the TX-RX frequency separation of band n254 is lager than band n256 and new RF filter can be used for band n254, we prefer to reuse the reference sensitivity of band n53 to define the reference sensitivity of band n254. Therefore, Option 1 defined as Table 2.5-1 is our preference and we can also accept the option 2 which is same REFSENS as band n256.

Table 2.5-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n254 | 15 | -100.0 | -96.8 | -95.0 |  |  |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 |  |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 |  |  |  |  |  |  |  |

Proposal 5: For the reference sensitivity of the new NTN FDD band, we prefer to option 1 which is same REFSENS as band n53.

## 2.6 Out-of-band blocking requirements

For out-of-band blocking requirements of n254, the general requirements for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz can be reused. Out-of-band blocking requirements for band n53 and band n255 also used this general requirements. Therefore, for band n254, the out-of-band blocking requirements can be specified in Table 2.6-1.

Table 2.6-1: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n254 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |

Proposal 6: For the new NTN FDD band, the out-of-band blocking requirements can be specified in Table 2.6-1.

Conclusions

In this contribution, we want to share some further views on UE RF requirements for Introduction of the satellite L-/S-band. Some proposals are made as following:

Proposal 1: The NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz should be numbered as Table 2.1-1.

Proposal 2: The asymmetric channel bandwidth combinations 5+10MHz, 5+15MHz, and 10+15MHz UL/DL can be introduced under the same principle defined in section 5.3.6 in TS 38.101-1.

Proposal 3: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the synchronization raster of SS Block pattern Case C also should be defined as Table 2.3-3.

Proposal 4: For the NTN FDD band with a UE transmitting at 1610-1626.5MHz and SAN transmitting at 2483.5-2500MHz, the TX–RX frequency separation defined as Table 2.4-1 can be used as the starting point.

Proposal 5: For the reference sensitivity of the new NTN FDD band, we prefer to option 1 which is same REFSEMS as band n53.

Proposal 6: For the new NTN FDD band, the out-of-band blocking requirements can be specified in Table 2.6-1.

References

- RP-223485, “New WID on Introduction of the satellite L-/S-ban”, Globalstar, Apple.