3GPP TSG-RAN WG4 Meeting #107 	R4-2308602

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Specific Issues Related to Use Cases for AI/ML

Agenda item:	8.22.2

Document for:	Discussion

Introduction

The first discussions on AI/ML for NR air interface started t RAN4#106-bis-e. The outcomes of the meeting are captured in the WF [1]. Some of the use case specific issues require further discussion, as follows:

- KPIs/Test Metrics for use cases

- Use case specific core and LCM (Life Cycle Management) related requirements

- Requirements/tests related to generalization/scalability

In this paper, we provide some additional views on the topics listed above.

More detailed analysis of general aspects of AI/ML and Interoperability and testing aspects are provided in our accompanying papers [2] and [3], correspondingly

Discussion

KPIs/ Test Metrics for use cases

KPIs/Test Metrics for CSI Feedback

From RAN WG4 Meeting # 106-bis-e WF [1] document we observe the KPIs identified are as below.

Following KPIs are to be considered in the RAN4 study for different use cases. - KPIs/Test Metrics for CSI prediction and compression - Throughput - Other options could also be considered depending on work in other work groups For e.g., SGCS/NMSE and accuracy of CSI prediction, latency of CSI feedback/prediction

In the WF document, throughput terminology can be confusing because absolute throughput as a KPI could not be a feasible indicator.

- The throughput KPI should be “the relative gain in Throughput”. It would also then be comparable to the legacy CSI framework.

Before we discuss the KPIs and Metrics to be considered from RAN4 perspective, we briefly present the model performance for the AIML based CSI feedback functionality.

Note: The visualizations and results tabulated below are from our RAN1 accompanying paper [4]. For more detailed information on the test parameters, methodologies etc., please refer to the accompanying paper.

AI/ML Enabled CSI Feedback Performance

CSI Compression

Some simulation results for CSI Compression use case using Transformer model are compared with Rel-16 Type II baseline in following Figures.

- (b)                			  (c)

Figure 1: Transformer model system-level performance compared to the Rel-16 Type II baseline, (a) SGCS; (b) mean user throughput gain; (c) cell-edge user throughput gain – MU-MIMO Rank 1 Full Buffer.

(a)            			          (b)         		                            (c)

Figure 2.  Transformer model performance compared to the Rel-16 Type II baseline, (a) SGCS; (b) mean user throughput gain; (c) cell-edge user throughput gain – MU-MIMO Rank 2 Full Buffer.

(a)            			     (b)         		                               (c)

Figure 3.  Transformer model performance compared to the Rel-16 Type II baseline, (a) SGCS; (b) mean user throughput gain; (c) cell-edge user throughput gain – MU-MIMO Rank 1 Bursty traffic with ~80% RU.

(a)            			          (b)         		                 (c)

Figure 4:  Transformer model performance compared to the Rel-16 Type II baseline, (a) SGCS; (b) mean user throughput gain; (c) cell-edge user throughput gain – MU-MIMO Rank 2 Bursty traffic with ~80% RU.

The transformer-based CSI compression model outperforms baseline Rel-16 eTypeII codebook performance based on both the SGCS metric and MU-MIMO throughput performance.  Gains up to the following are seen:

- 6.5%/2.5% in mean/cell edge user throughput in full buffer traffic with maximum rank of 1.

- 8.5%/3.5% in mean/cell edge user throughput in full buffer traffic with maximum rank of 2.

- 4.5%/10.0% in mean/cell edge user throughput in bursty traffic (~80% RU) with maximum rank of 1.

- 13.0%/22.5% in mean/cell edge user throughput in bursty traffic (~80% RU) with maximum rank of 2.

CSI Prediction

Some simulation results for CSI Prediction use case using 3-layer convolutional LSTM are shown in following Table 1.

|  |  | Source 1: Nokia |
| --- | --- | --- |
| AI/ML model description | AL/ML model backbone | 3-layer convolutional LSTM |
|  | [Pre-processing] | time domain interpolation / oversample factor 5 |
|  | [Post-processing] | Type II Release 16 |
|  | FLOPs/M | 129472 FLOPs |
|  | Parameters/M | 4368 parameters |
|  | [Storage /Mbytes] |  |
|  | Input type | complex CSI, 50 PRBs, 16 AP |
|  | Output type | complex CSI, 50 PRBs, 16 AP |
| Assumption | UE speed | 30 kmph |
|  | CSI feedback periodicity | 5 ms |
|  | Observation window (number/distance) | 4 / 5 ms |
|  | Prediction window (number/distance) | 1 / 4 ms |
|  | Whether/how to adopt spatial consistency | Track over 500 ms |
| Dataset size | Train/k | 200 UE tracks |
|  | Test/k | 100 UE tracks |
| Benchmark 1 |  | ZOH |
| Intermediate KPI #1 of Benchmark 1 |  | 8.65% < 0.9 SGCS [ideal CSI-RS] 80.7% < 0.9 SGCS [non-ideal CSI-RS] |
| Gain for intermediate KPI#1 over Benchmark 1 |  | 3.74% [ideal CSI-RS] 11.4% [non-ideal CSI-RS] |
| Intermediate KPI #2 of Benchmark 1 |  |  |
| Gain for intermediate KPI#2 over Benchmark 1 |  |  |
| Intermediate KPI #1 of Benchmark 1 |  |  |
| Gain for eventual KPI (Benchmark 1) | Mean UPT | 3 % [ideal CSI-RS], 2% [non-ideal CSI-RS] |
|  | 5% UPT | 6 % [ideal CSI-RS], 15% [non-ideal CSI-RS] |

Table 1: CSI Prediction Performance Results

System level simulations of AI/ML-based CSI prediction show gains of around 5% in mean user throughput with SU-MIMO in full buffer traffic.

As we can see from the results above, AIML enabled functionality brings in some gains which are comparable to the legacy functionality. Now to accurately measure the gains that the AIML enabled functionality for CSI Feedback (Compression / Prediction) brings in, some modifications to the existing KPIs and test metrics may be necessary.

A re-look of the existing KPIs is necessary due to the gains the AIML enabled functionality brings in with respect to the legacy functionality.

The KPIs and metrics for the AIML functionality, are indirectly dependent on the performance of different components of the AIML functionality. So, it becomes pertinent at RAN4 to define additional KPIs and metrics which helps test the performance of the AIML functionality completely.

Definition of new KPIs / Test metrics in addition to the functionality level KPIs and test metrics are essential to test the performance of AIML enabled functionality.

Some of the intermediate KPIs identified in other work groups like RAN1 can be good KPIs / Metrics at RAN4 such as data accuracy (SGCS), end to end execution latency.

In case of CSI feedback, data accuracy of the feedback is an important KPI. Especially when we use two-sided models. This KPI is a measure of how similar is the data that is recovered at the gNB to the data that was compressed using the model.

This can be measured in different ways. A common metric to measure this similarity is the normalized mean-square error (NMSE).  While the NMSE may be appropriate for assessing the performance of channel feedback, it is less appropriate for assessing the performance of eigenvector feedback where the primary concern is with the direction of the eigenvectors.  A more appropriate measure in this case is the cosine similarity which indicates how well the directions of the desired and estimated vectors match.  Cosine similarity may also be used with channel feedback by either tracking the cosine similarity of the actual channel and the channel recovered from feedback or by tracking the eigenvectors of the actual and recovered covariance matrix. Since the channel eigenvectors are the primary source used for calculating transmit coefficients, the cosine similarity of the eigenvectors is an appropriate KPI.  Cosine similarity can also be measure on conventional feedback methods under the assumption that the feedback is approximating the eigenvectors of the channel.

- RAN4 to study the feasibility of using the intermediate KPIs identified in RAN1/RAN2 as a KPI or metric at RAN4 requirements and procedures. Consider the cosine similarity between actual and recovered eigenvectors as the KPI for assessing the performance of ML-based CSI feedback.

KPIs/Test Metrics for beam management

From RAN WG4 Meeting # 106-bis-e WF [1] document we observe the KPIs identified are as below.

  - Beam prediction accuracy (absolute or relative)

  - other KPIs could also be considered: e.g., average system throughput, cell edge throughput, beam measurement accuracy, prediction confidence etc.

- From the above listed KPIs, average system throughput and cell edge throughput are the KPIs not feasible to validate in absolute terms. KPIs on the relative increase in average throughput or cell edge throughput seem more suitable to validate and monitor throughput aspects of the AI/ML-enabled BM use case.

Absolute Throughput KPIs are not feasible to validate throughput related aspects for AI/ML-enabled BM use case. Currently, RAN4 doesn’t have throughput-based requirements for beam management. In RAN1 evaluation method, it is shown that average throughput and cell edge throughput are important performance indicator.

- We suggest to have KPIs on the relative increase in average throughput or cell edge throughput to validate and monitor throughput aspects of the AI/ML-enabled BM use case. We further suggest to study in RAN4 the feasibility to have throughput related performance indicators for beam management use case.

To evaluate the performance of AI/ML in beam management, in the RAN1 #109-e meeting the agreement [1] was made to further study the following KPI options: Beam prediction accuracy-related KPIs, which may include the options specified in Table 2 and Table 3 for beam prediction related KPIs and system performance related KPIs respectively.

| KPIs | KPI description | Notes |
| --- | --- | --- |
| Beam prediction accuracy Top-1 (%) | the percentage of “the Top-1 genie-aided beam is Top-1 predicted beam” | ATop-1=1Nt n=1Ntl(bn=bn) Where bn is the index of the Top-1 predicted beam and bn is the index of the Top-1 genie-aided beam. Nt is the number of data points for obtaining the ML model performance, l() is the indicator function. If bn=bn, then l()=1. Otherwise, l()=0. Top-1 genie-aided beam index is selected based on bn=argmax(RSRPl) for l={1,…,M}, where M are all the beams in SetA. |
| Beam prediction accuracy Top-K/1 (%) | the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams” | ATop-K=1Nt n=1Ntk=1Kl(bk,n=bn) Where bk,n is the index of the k-th beam in the Top-K predicted beams and bn is the index of the Top-1 genie-aided beam. |
| Beam prediction accuracy (%) with 1dB margin for Top-1 beam | The beam prediction accuracy (%) with 1dB margin is the percentage of the Top-1 predicted beam “whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam” | ATop-1, 1 dB margin=1Nt n=1Ntl(RSRPbn-RSRPbn<1 dB) Where RSRPbn is the RSRP of the Top-1 genie-aided beam and RSRPbn is the RSRP of the Top-1 predicted beam. |
| Average L1-RSRP difference of Top-1 predicted beam | The difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam | eRSRP=1Nt n=1NtRSRPbn-RSRPbn |

Table 2: List of Beam prediction accuracy related KPIs to evaluate the performance of AI/ML in beam management.

| KPIs | KPI description | Notes |
| --- | --- | --- |
| UE throughput | CDF of UE throughput, Average throughput and 5%-tile UE throughput | A similar mechanism as in Rel-16/17 MIMO BM simulations |
| RS overhead reduction | RS overhead reduction at least for spatial-domain beam prediction at least for Top-1 beam | For the evaluation of the overhead for BM-Case1, adoption the Opt1 metric: RS overhead reduction [%]=1-NM • where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML • where M is the total number of beams (pairs) to be predicted For the evaluation of the overhead for BM-Case2, adoption the Opt3 metric: RS overhead reduction [%]=1-NLM • where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each time instance • where M is the total number of beams (pairs) to be predicted for each time instance • where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction |

Table 3: List of System performance related KPIs to evaluate the performance of AI/ML in beam management

From Table 2 above, Average L1-RSRP difference of Top-1 predicted beam is a KPI/Test metric to be considered in RAN4.

- We suggest to have Average L1-RSRP difference of Top-1 predicted beam in the list of KPIs/Test metrics selected for further study in RAN4.

In this paper, we provide our analysis on the use of the above listed KPIs. As, it is shown below different KPIs can be useful in different BM-Cases. We are mainly focusing on Spatial Domain BM case here.

#### Evaluation Results for Set B is a subset of Set A

In this section, the following BM-Case1DL TX alternative has been considered for evaluation:

BM-Case1DL TX Alt. 1-1 – Set B is a subset of Set A

- ML model input: Set B beam L1-RSRP

- ML model output: Set A best beam ID

- Model training and testing with the same Set B

| Assumptions | Fixed Set B (SetB subset of SetA) | Fixed Set B (SetB subset of SetA) | Fixed Set B (SetB subset of SetA) |
| --- | --- | --- | --- |
| Number of beams in Set A | 64 | 64 | 64 |
| Number of beams in Set B | F32 | F16 | F8 |
| [Pattern of Set B] | Tx ID=[ 0, 2, 4, … ] | Tx ID=[ 0, 4, 8, … , 61] | Tx ID=[6, 14, 20, 28, 34, 42, 48, 56] |
| [Rx beam assumption] | Best Rx Opt 1 | Best Rx Opt 1 | Best Rx Opt 1 |
| Baseline scheme | Option 2 | Option 2 | Option 2 |
| Model input, | Alt2: L1-RSRP, implicty Tx beam ID | Alt2: L1-RSRP, implicty Tx beam ID | Alt2: L1-RSRP, implicty Tx beam ID |
| Model output, | Probabilities of Top-1 beam for all Tx beam in Set A | Probabilities of Top-1 beam for all Tx beam in Set A | Probabilities of Top-1 beam for all Tx beam in Set A |
| Model label | Top-1 genie-aided beam ID | Top-1 genie-aided beam ID | Top-1 genie-aided beam ID |
| Training | ~38K | ~38K | ~38K |
| Testing | ~4K | ~4K | ~4K |
| model description | DNN | CNN | DNN |
| [Model complexity in a number of model parameters (M)] | ~50k | ~100K | ~50k |
| [Model complexity in a number of model size (e.g. Mbyte)] | NA | NA | NA |
| Computational complexity [FLOPs] | NA | NA | NA |
| Top-1(%) | NA | 85.2 | NA |
| Top-1(%) with 1dB margin | 99/- | 96/37 | 86/20 |
| Top-2/1(%) , Top-4/1(%) , other values | NA | Top-2/1(%) , Top-3/1(%) ,Top-5/1(%) [95.8, 97.9, 99.2] | NA |
| Top-1/2(%), Top-1/4(%), other values (Optional) | - | - | - |
| Average L1-RSRP diff (dB) | 0.03/- | 0.2/2.95 | 0.8/6.11 |
| [5%ile of L1-RSRP diff (dB)] | NA | 0.9 | NA |
| [e.g., Predicted L1-RSRP] (Optional) | - | - | - |
| RS overhead Reduction (%) (Opt 1/2/3 reported by companies) | 50 (Opt1) | 75 (Opt1) | 87.5 (Opt1) |
| [avg. UE throughput] | Average cell throughput ratio – Method/Ideal [%] 100%/98% | Average cell throughput ratio – Method/Ideal [%] 100%/85% | Average cell throughput ratio – Method/Ideal [%] 98%/73% |
| [5%ile UE throughput] | 5th percentile UE throughput ratio – Method/Ideal [%] 100%/97% | 5th percentile UE throughput ratio – Method/Ideal [%] 99%/97% | 5th percentile UE throughput ratio – Method/Ideal [%] 84%/75% |
| [UCI report] | NA | NA | NA |

Table 4: Evaluation results for BM-Case1 for DL Tx beam prediction Alt. 1-1.(SetB is subset of SetA)

From Table4, we can observe that ML-based 16 beams have high prediction accuracy (>95%) and the prediction RSRP error mean is lower than 1 dB, and its good prediction performance is also reflected in the throughput that it has similar system throughput compared to the ideal baseline. On the other hand, from ML-based 8 beams, the prediction accuracy drops by a significant margin, and the ML-based 8 beams start losing nonnegligible cell-edge UE throughput compared to the ideal case. Therefore, if the beam prediction functionality/model input ONLY uses a “sparse” Set B or a poor Set B pattern design for the UE, may cause throughput loss, especially for the cell-edge UE. By this point, we understand that in certain cases, Set B RSRP may not be sufficient for beam prediction input, and additional assistant info may be needed to improve the prediction performance.

- RAN4 further study to include Top-1(%) or Top-K(%) or Top-K/1(%) prediction accuracy requirements for AI/ML based BM use case.

#### Evaluation Results for Set B is Different to Set A

As mentioned in the accompanying RAN1 BM evaluation paper [5], for BM-Case1DL TX- Set B is different from Set A, we consider Set B to be a wide beam codebook, and Set A is a refined (narrow) beam codebook. In the following, we will try to compare the Set A prediction performance based on different choices of Set B wide beam codebook, and we consider the following wide beam construction methods:

- Wide beam codebook#1 - baseline

  - The wide beam codebook construction is described in our evaluation paper [5]

- Wide beam codebook#2

  - The wide beam codebook construction is adding circular shift operation on top of [5]

- Wide beam codebook#3

  - Each wide beam codebook is the summation of the randomly selected refined beam in Set A.

- Wide beam codebook#4

  - The wide beam codebook is constructed in a hybrid fashion - some wide beams are constructed based on the wide beam codebook#1 method while other wide beams are constructed based on the wide beam codebook#3 method. The hybrid wide beam codebook has the same coverage as Set A.

Figure 5: Examples design of wide beam codebook #1 #2 #3.

Examples of wide beam codebook#1/#2/#3 are shown in Figure 5.

All wide beam codebooks combine 4 refined beams into 1 wide beam. In the following, we consider Set A with 64 and 32 refined beams, and Set B will have 16 and 8 wide beams correspondingly. Table 5 shows the model performance KPI for different wide beam codebooks

| Assumptions |  | Widebeam and Narrow beam | Widebeam and Narrow beam | Widebeam and Narrow beam | Widebeam and Narrow beam |
| --- | --- | --- | --- | --- | --- |
| Number of beams in Set A |  | 64 narrow beam | 64 narrow beam | 32 narrow beam | 32 narrow beam |
| Number of beams in Set B |  | 16 widebeams (wide beam codebook #1) | 16 widebeams (wide beam codebook #4) | 8 widebeams (wide beam codebook #1) | 8 widebeams (wide beam codebook #4) |
| [Pattern of Set B] | All widebams | All widebams | All widebams | All widebams |
| [Rx beam assumption] | Best Rx Opt 1 | Best Rx Opt 1 | Best Rx Opt 1 | Best Rx Opt 1 |
| Baseline scheme | Option 2 | Option 2 | Option 2 | Option 2 |
| Model input, | Alt2: L1-RSRP, implicty Tx beam ID | Alt2: L1-RSRP, implicty Tx beam ID | Alt2: L1-RSRP, implicty Tx beam ID | Alt2: L1-RSRP, implicty Tx beam ID |
| Model output, | Probablities of Top-1 beam for all Tx beam in Set A | Probablities of Top-1 beam for all Tx beam in Set A | Probablities of Top-1 beam for all Tx beam in Set A | Probablities of Top-1 beam for all Tx beam in Set A |
| Model label | Top-1 genie-aided beam ID | Top-1 genie-aided beam ID | Top-1 genie-aided beam ID | Top-1 genie-aided beam ID |
| Training | ~33K | ~33K | ~33K | ~33K |
| Testing | ~4K | ~4K | ~4K | ~4K |
| model description | DNN | DNN | DNN | DNN |
| [Model complexity in a number of model parameters (M)] |  | ~0.03M | ~0.03M | ~0.007M | ~0.007M |
| [Model complexity in a number of model size (e.g. Mbyte)] |  | ~0.12M | ~0.12M | ~0.028M | ~0.028M |
| Computational complexity [FLOPs] |  | 0.132G | 0.132G | 35M | 35M |
| Top-1(%) | 80.5 | 84.19 | 68.5 | 88.1 |
| Top-1(%) with 1dB margin | 92.1 | 95.3 | 73 | 93.5 |
| Top-2/1(%) , Top-4/1(%) , other values | Top-2/1(%) , Top-3/1(%) ,Top-5/1(%) 93.57 96.6 98.5 | Top-2/1(%) , Top-3/1(%) ,Top-5/1(%) 95.64 97.8 99.4 | Top-2/1(%) , Top-3/1(%) ,Top-5/1(%) 85.52 93.19 97.83 | Top-2/1(%) , Top-3/1(%) ,Top-5/1(%) 96.5 98.6 99.6 |
| Top-1/2(%), Top-1/4(%), other values (Optional) |  | NA | NA | NA | NA |
| Average L1-RSRP diff (dB) |  | 0.334 | 0.17 | 3.007 | 0.4 |
| [5%ile of L1-RSRP diff (dB)] |  | 2.067 | 0.93 | 16.83 | 1.84 |
| [e.g., Predicted L1-RSRP] (Optional) |  | NA | NA | NA | NA |
| RS overhead Reduction (%) (Opt 1/2/3 reported by companies) |  | 75% (Opt 1) | 75% (Opt 1 | 75% (Opt 1) | 75% (Opt 1) |
| [avg. UE throughput] |  | NA | NA | NA | NA |
| [5%ile UE throughput] |  | NA | NA | NA | NA |
| [UCI report] |  | NA | NA | NA | NA |

Table 5: Evaluation results for BM-Case1 for DL Tx beam prediction Alt. 2 (SetB is different from SetA)

From Table 5, we have the following observations:

- For Set A with 64 beams, the designed wide beam codebooks #2/#3/#4 have marginal performance gain compared to the baseline wide beam codebook#1. This is mainly because of the large/oversampled beam number 64 in Set A for the current antenna configurations – with a 1 dB error margin, the wrong refined beam prediction may have less than 1 dB RSRP difference compared to the actual best beam since the refined beams now are too close to each other.

- For Set A with 32 beams, the designed wide beam codebooks #2/#3/#4 perform significantly better than the baseline wide beam. The idea behind the wide beam codebook #2/#3/#4 can be found in our accompanying RAN1 BM other aspects paper [8]. And as expected, randomly combining the refined beam to form the wide beam can provide the richest correlation between the refined beam and the wide beam codebook.

- From Table 5, one can see that the refined beam prediction with WB#1 has significantly worse throughputs than the baseline. On the other hand, the suggested WB#2, #3, #4 have comparable throughput to the baseline. One should notice that the assumption for WB#2, #3, #4 is using the wide beam codebook for SSB and using the SSB measurements to predict the refined beams, while the hierarchical search will additionally require UE-specific CSI-RS for P2 beam management. Also, as we point out in [5], WB#3 may not be good for SSB as it will degrade the cell coverage, and the throughput results also tell that the wide beam codebook with the best beam prediction results may not be beneficial for throughput. For Set B is different to Set A with Set B is wide beam, the KPI for the wide beam codebook design should be both prediction accuracy and throughput performance.

For Set B is different to Set A with Set B is wide beam, the KPI for the wide beam codebook design should be both prediction accuracy and throughput performance.

- RAN4 should further study the requirements and test mechanisms for both prediction accuracy and relative throughput performance when set B is wide beam and set A is narrow beam.

#### 2.1.2.3 Impact of Measurement Errors and Quantization Errors

##### L1-RSRP Measurement Errors

Both BM-Case 1 and BM-Case 2 sub use cases utilize L1-RSRP measurements for the input/output of the ML model. However, due to the RF impairments and other non-ideal components at the UE receiver, the L1-RSRP measurements are affected by errors. The range of measurement errors for FR2 is set by current L1-RSRP requirements defined in Clauses 10.1.20 of TS 38.133. Therefore, in the evaluation, we assume that the measurements error can be modelled with a normal distribution with zero mean and standard deviation that allows with a 95% probability to keep the measurement errors within a range. Then, we repeat the evaluation with different ranges of measurement errors.

All Tx beams are assumed with the same range of measurements error, nonetheless the measurement errors are independently generated between one and other beams because the UE may perform the beams’ measurements in different time instances.

For both BS and UE side models, the measurement errors affect the input L1-RSRPs for SetB beams. At the same time, during training, measurement errors also affect the output since the labels are determined based on the non-ideal L1-RSRP of SetA beams. Therefore, we use as model input the non-ideal L1-RSRP values for both training and inference operations. On the other hand, for the model output, during training, we determine the labels from the non-ideal L1-RSRP, whereas during inference we use the ideal L1-RSRP for assessing model performance. A summary is provided in Table 6. We use the BM-Case1 with fixed Set B pattern and 16 Tx beams input to predict SetA with 64 Tx beams.

|  | Model Input | Model Output |
| --- | --- | --- |
| Training | L1-RSRP with measurement errors | L1-RSRP with measurement errors |
| Inference | L1-RSRP with measurement errors | Based on ideal L1-RSRP (for assessing model performance) |

Table 6: Evaluation assumptions for model input/output with measurement errors.

Figure 6 shows the CDFs of the difference between the ideal L1-RSRP of predicted beam and the ideal L1-RSRP of the genie-aided beam. Several ranges including 95% of measurement errors are tested: ± 0 dB, ± 2 dB, ± 4 dB, ± 6 dB. Ideal quantization it is assumed to evaluate the measurement error effect only.

Figure 6: Tx beam prediction results for different ranges of measurement errors: ± 0 dB, ± 2 dB, ± 4 dB, ± 6 dB and ideal quantization (step size 0 dB).

The range of ± 6 dB introduces an RSRP difference of 3.6 dB at the 95%-tile of the CDF. This degrades the RSRP difference by 2.8 dB compared to the model that uses ideal measurements. Differently, with ± 4 dB and ± 2 dB ranges, the RSRP differences at the 95%-tile are 2.6 dB and 1.4 dB, respectively. Hence, the RSRP difference increases by 1.7 dB and by 0.5 dB wrt ideal measurements.

- For BM-Case1, RAN4 should study the requirements of non-ideal measurements considering values of measurement errors ranges tighter than the current L1-RSRP requirements.

##### L1-RSRP Quantization Errors

For the NW side model, quantization of L1-RSRP measurements may impact the L1-RSRP for SetB beams in both training and inference operations. At the same time, during the training, the model may use the L1-RSRP of the SetA beams, which may also be reported with quantized values. Therefore, for training, we assume that both the input samples of the model and the ground truth are affected by the quantization error. On the other hand, for inference we use quantized L1-RSRP as input whereas we use the ideal L1-RSRP to compute the RSRP error between the predicted and actual best beams. A summary is provided in Table 7. We use the BM-Case1with fixed Set B pattern and 16 Tx beams input to predict SetA with 64 Tx beams.

.

|  | Model Input | Model Output |
| --- | --- | --- |
| Training | Quantized L1-RSRP | Based on quantized L1-RSRP |
| Inference | Quantized L1-RSRP | Based on non-quantized L1-RSRP (for assessing model performance) |

Table 7: Evaluation assumptions for model input/output with quantized L1-RSRPs

Figure 7 shows the CDFs of the difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam. We repeat the evaluation with several quantization step sizes: 0 dB, 1 dB, 2dB, and 4 dB. Note that the ideal measurement error is assumed to evaluate the quantization error effect only.

Figure 7: Tx beam prediction results for different quantization step sizes: 0 dB, 1 dB, 2dB, 4 dB and ideal measurement error range ± 0 dB.

For the 1 dB step size, the quantization introduces a L1-RSRP difference of 0.2 dB at the 95%-tile of the CDF wrt the non-quantized measurements. Conversely, using the step sizes of 2 dB and 4 dB increases the L1-RSRP prediction difference to 0.65 dB and 2 dB at the 95%-tile of the CDF wrt the non-quantized measurements. Therefore, the 1 dB and 2 dB step sizes may be sufficient for the AI/ML beam management as they do not much affect the prediction performances.

- RAN4 might further consider the requirements of quantization steps of the measurements when evaluating AI/ML based BM use case.

##### Combined Measurement Errors and Quantization Errors

In this section, we study the combined effect of measurement errors and quantization errors. For the measurement errors, we use the range ± 2 dB including 95% of measurement errors and repeat the evaluation with quantization step sizes: 0 dB, 1 dB, and 2 dB. Figure 2-8 shows that for a 2 dB quantization step size, the L1-RSRP difference wrt the ideal measurement case is about 1 dB at the 95%-tile of the CDF. Reducing the quantization step size to 1 dB reduces the L1-RSRP difference wrt the ideal measurement to 0.6 dB. However, it can’t completely eliminate the L1-RSRP difference as the prediction errors are limited by the measurement errors.

The performance of the model trained with data affected by both measurement errors and quantization errors can’t be improved by only reducing the quantization step size as the prediction performances are limited by the measurement errors.

- RAN4 should study further the need of requirements for combined measurement and quantization errors.

Figure 9: Tx beam prediction results for different quantization step sizes: 0 dB, 1 dB, 2dB and measurement error range ± 2 dB.

KPIs/Test Metrics for positioning

From RAN WG4 Meeting # 106-bis-e WF [1] document we observe the KPIs identified are as below.

  - Measurement accuracy

  - FFS whether latency can also be considered

  - other KPIs could also be considered:

Benchmark/legacy method accuracy

Considering UE positioning, different legacy methods are defined and can be used as benchmark. However, the accuracy of the considered benchmark method should be accounted for, which depends on the selected method (e.g. TDOA), input parameters (e.g. Number of TRP measurements) as well as the considered environment (e.g. NLOS proportion).

Benchmark method for positioning accuracy can vary based on different parameters. The selection of the benchmark method should then be performed accounting for the expected achievable accuracy considering the impacting factors.

- Benchmark positioning method should be selected accounting for its achievable positioning accuracy

For measurement accuracy, we can provide input on TOA measurement accuracy and RSRP measurement accuracy.

ML based positioning performance can be sensitive to the input data accuracy. The tolerable accuracy threshold depends on the considered ML model parameters and input types (e.g. ToA and RSRP).

Following the analysis, some Observations and Proposals are made.

The impact of the input accuracy to the AI/ML positioning functionality/model, on the final performance (i.e., UE positioning accuracy) depends on the considered functionality/model parameters and inputs type.

- Input to the AI/ML positioning functionality/model, measurement accuracy (typically for ToA and RSRP) should be considered.

CSI testing baseline framework

From RAN WG4 Meeting # 106-bis-e WF [2] document we observe the KPIs identified are as below.

- PMI reporting framework (follow PMI vs. random PMI test, use of γ as criteria, etc.) to be taken as starting point for CSI related tests - Other KPI/framework is not precluded

Currently minimum performance requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reported PMI compared to the case when the transmitter is using random precoding, respectively. This ratio is referred to as γ (gamma). With the introduction of ML-enabled CSI compression we can foresee some changes in the CSI reporting framework for PMI. and that may impact the cases especially the PMI reporting requirements in terms of performance and the value of γ (gamma) can be different than the current minimum performance requirement.

The CSI use case impacts only PMI part of the CSI reporting requirements.

- RAN4 should further study the impacts of AI/ML-enabled CSI use cases on the UE performance requirements in TS 38.101-4. A specific new target value of γ (gamma) for AI/ML-enabled CSI use cases can be envisaged.

Other than the legacy γ (gamma), a potential new parameter can be introduced to measure the relative increase in throughput when the transmitter is configured according to the UE reported PMI (using AI/ML-enabled method) compared to the case when the transmitter is configured according to the UE reported PMI, respectively. This new ratio γAIML will show the performance gain of AI/ML enabled use cases in comparison with legacy mechanism.

γAIML is defined as below:

γAIML=tAIMLtue

Where:

- tue is as per the existing requirement (90 % of the maximum throughput obtained at  using the precoders configured according to the UE reports). It can be either type1 random, type1 or type2 precoding.

- tAIML is the throughput measured at using the precoders configured according to the CSI report when AIML based CSI feedback enabled.

A new relative throughput performance indicator can be introduced for AI/ML-enabled CSI use cases.

- RAN4 should further study if a new relative throughput performance indicator would be more suitable for AI/ML-enabled CSI use case, other than the legacy γ (gamma).

Note: Legacy performance can be considered as baseline only for the features/use-cases that are mandatorily supported by the device.

Core requirements for AI/ML / LCM Related Requirements

Agreements and pointers from RAN WG4 Meeting # 106-bis-e WF [1] as below.

### Core requirements for AI/ML - Definition of RAN4 core requirements for the following procedures will be studied based on progress in RAN1/RAN2: - Performance monitoring procedure, including performance evaluation and decision making procedure for AI/ML functionalities/models - Functionality/Model management procedure, including functionality/model selection/activation/deactivation, and functionality/model switching/fallback/transfer/delivery/update - Latency/interruption requirement for above procedures - FFS is any other aspects should be studied ### LCM Related requirements Agreements: - Following LCM related requirements are to be studied: - Model/Functionality select/switch/activate/deactivate/fallback - Model/Functionality monitoring - FFS if requirements for data collection (in particular for training) could/need be defined - FFS if requirements for transfer/delivery/update - NOTE: RAN4 study should be aligned with the agreements in other working groups. - Further study under LCM related tests if they are defined. - how the framework can address the possibility of updates/activation/deactivation/switching to the functionalities/models after the deployment of the devices in the field.

Currently there are no use case specific core requirements defined in other WGs.

- RAN4 should wait until the discussions in other WGs are concluded on the use-case specific topics to see if any core requirements need to be defined.

Due to the introduction of two-sided models for CSI feedback, it is necessary to define some requirements for interoperability of the models at the UE and the TE. Especially for validating the models before they are deployed in the field. e.g., making sure that the models selected at the DUT and the TE are interoperable and work seamlessly as an initial step in the test procedure. This can be covered as part of use case specific LCM requirements.

In case of AIML based CSI feedback use case, interoperability of the DUT and the TE side models is a very important step in the test procedure.

- RAN4 to study the possible ways of defining LCM requirements for CSI feedback specific use case to make sure the specific models of the two-sided model are interoperable. This can be done in RAN4 as a part of the pre-deployment validation of the corresponding models.

Generalization/scalability of requirements/tests

Pointers from RAN WG4 Meeting # 106-bis-e WF [1] as below.

- Further study whether it is needed/feasible to introduce some form of generalization and/or scalability related requirements for different scenarios/configurations based on RAN1 agreements - Whether this can be implicitly handled in the test case definition should be considered - Intention is to guarantee that performance will still be maintained in different environments/scenarios/configurations.

Generalization/scalability as a topic is being discussed at length in both RAN1 and RAN2. And most of the items in this are yet to be concluded.

- RAN4 to wait for the conclusion on the generalization and scalability aspects in RAN1/2 to start the feasibility study for defining use case specific generalization/scalability related requirements and tests.

Conclusion

In this paper we share our views on potential RAN4 impacts from selected AI/ML use cases in NR air interface. Specifically, we cover following aspects for selected use cases:

- KPIs/Test Metrics for use cases

- Use case specific core and LCM (Life Cycle Management) related requirements

- Requirements/tests related to generalization/scalability

In the paper, the following Observations and Proposals were made:

- In the WF document, throughput terminology can be confusing because absolute throughput as a KPI could not be a feasible indicator.

The throughput KPI should be “the relative gain in Throughput”. It would also then be comparable to the legacy CSI framework.

The transformer-based CSI compression model outperforms baseline Rel-16 eTypeII codebook performance based on both the SGCS metric and MU-MIMO throughput performance.  Gains up to the following are seen:

- 6.5%/2.5% in mean/cell edge user throughput in full buffer traffic with maximum rank of 1.

- 8.5%/3.5% in mean/cell edge user throughput in full buffer traffic with maximum rank of 2.

- 4.5%/10.0% in mean/cell edge user throughput in bursty traffic (~80% RU) with maximum rank of 1.

- 13.0%/22.5% in mean/cell edge user throughput in bursty traffic (~80% RU) with maximum rank of 2.

System level simulations of AI/ML-based CSI prediction show gains of around 5% in mean user throughput with SU-MIMO in full buffer traffic.

A re-look of the existing KPIs is necessary due to the gains the AIML enabled functionality brings in with respect to the legacy functionality.

Definition of new KPIs / Test metrics in addition to the functionality level KPIs and test metrics are essential to test the performance of AIML enabled functionality.

- RAN4 to study the feasibility of using the intermediate KPIs identified in RAN1/RAN2 as a KPI or metric at RAN4 requirements and procedures. Consider the cosine similarity between actual and recovered eigenvectors as the KPI for assessing the performance of ML-based CSI feedback.

Absolute Throughput KPIs are not feasible to validate throughput related aspects for AI/ML-enabled BM use case. Currently, RAN4 doesn’t have throughput-based requirements for beam management. In RAN1 evaluation method, it is shown that average throughput and cell edge throughput are important performance indicator.

- We suggest to have KPIs on the relative increase in average throughput or cell edge throughput to validate and monitor throughput aspects of the AI/ML-enabled BM use case. We further suggest to study in RAN4

From Table 2 above, Average L1-RSRP difference of Top-1 predicted beam is a KPI/Test metric to be considered in RAN4.

For Set B is different to Set A with Set B is wide beam, the KPI for the wide beam codebook design should be both prediction accuracy and throughput performance.

- RAN4 should further study the requirements and test mechanisms for both prediction accuracy and relative throughput performance when set B is wide beam and set A is narrow beam.

- RAN4 further study to include Top-1(%) or Top-K/1(%) prediction accuracy requirements for AI/ML based BM use case.

- For Set B is different to Set A with Set B is wide beam, the KPI for the wide beam codebook design should be both prediction accuracy and throughput performance.

- RAN4 should further study the requirements and test mechanisms for both prediction accuracy and relative throughput performance when set B is wide beam and set A is narrow beam.

- RAN4 might further consider the requirements of quantization steps of the measurements when evaluating AI/ML based BM use case.

The performance of the model trained with data affected by both measurement errors and quantization errors can’t be improved by only reducing the quantization step size as the prediction performances are limited by the measurement errors.

- RAN4 should study further the need of requirements for combined measurement and quantization errors.

Benchmark method for positioning accuracy can vary based on different parameters. The selection of the benchmark method should then be performed accounting for the expected achievable accuracy considering the impacting factors.

- Benchmark positioning method should be selected accounting for its achievable positioning accuracy

The impact of the input accuracy to the AI/ML positioning functionality/model, on the final performance (i.e., UE positioning accuracy) depends on the considered functionality/model parameters and inputs type.

- Input to the AI/ML positioning functionality/model, measurement accuracy (typically for ToA and RSRP) should be considered.

The CSI use case impacts only PMI part of the CSI reporting requirements.

- RAN4 should further study the impacts of AI/ML-enabled CSI use cases on the UE performance requirements in TS 38.101-4. A specific new target value of γ (gamma) for AI/ML-enabled CSI use cases can be envisaged.

A new relative throughput performance indicator can be introduced for AI/ML-enabled CSI use cases.

- RAN4 should further study if a new relative throughput performance indicator would be more suitable for AI/ML-enabled CSI use case, other than the legacy γ (gamma).

Currently there are no use case specific core requirements defined in other WGs.

- RAN4 should wait until the discussions in other WGs are concluded on the use-case specific topics to see if any core requirements need to be defined.

In case of AIML based CSI feedback use case, interoperability of the DUT and the TE side models is a very important step in the test procedure.

- RAN4 to study the possible ways of defining LCM requirements for CSI feedback specific use case to make sure the specific models of the two-sided model are interoperable. This can be done in RAN4 as a part of the pre-deployment validation of the corresponding models.

Generalization/scalability as a topic is being discussed at length in both RAN1 and RAN2. And most of the items in this are yet to be concluded.

- RAN4 to wait for the conclusion on the generalization and scalability aspects in RAN1/2 to start the feasibility study for defining use case specific generalization/scalability related requirements and tests.

- References

- R4-2306299, WF on AI/ML RAN4 studies, Qualcomm Incorporated, Electronic Meeting, RAN4#106bis-e, 17 April –26 April 2023.

- R4-2308030, On General Aspects of AI/ML for NR Air Interface, Nokia, Nokia Shanghai Bell, RAN4#107, Incheon, Korea, May 22 – 26, 2023.

- R4-2308031, On Interoperability and Testability Aspects of AI/ML for NR Air Interface, Nokia, Nokia Shanghai Bell, RAN4#107, Incheon, Korea, May 22 – 26, 2023.

- R1-2304681, Evaluation of ML for CSI feedback enhancement, Nokia, Nokia Shanghai Bell, RAN1 #113, Incheon, Korea, May 22 – 26, 2023.

- R1-2304683, Evaluation of ML for beam management, Nokia, Nokia Shanghai Bell, RAN1-113, Incheon Korea, May 22-26, 2023.

- R1-2304684, Other aspect of AI/ML for beam management, Nokia, Nokia Shanghai Bell, RAN1-113, Incheon Korea, May 22-26, 2023.