3GPP TSG-RAN WG4 Meeting #107	R4-2308604

Incheon, Republic of Korea, 22-26 May 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Beam type and DRX implications

Agenda item:	8.7.3.2

Document for:	Approval

Introduction

The Rel-18 NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 WI includes the following objectives for defining UE beam correspondence requirements for RRC_INACTIVE and initial access.

Beam correspondence requirements for RRC_INACTIVE and initial access - Specify UE beam correspondence requirements for initial access and RRC_INACTIVE state, for SSB-based beam correspondence without UL beam sweeping [RAN4 RF] - For RRC_INACTIVE specify at least requirements for Random Access SDT and Configured Grant SDT - Requirements for other transmission within RRC_INACTIVE state are not precluded. - For initial access, specify requirements and verification of beam correspondence requirements based on msg1 spherical coverage (at least) - Study the potential impact on testability aspects (i.e., test time).

In RAN4#106bis, we conclude on three topics in the WF. In this contribution we deal with the second topic:

- Topic #2: UE beam type and DRX implications ## Sub-topic 2-1 Minimum peak EIRP and spherical coverage Issue 2-1: Beam refinement - Proposals - Option 1: Since the UE has enablers such as Rel-17 TRS signals which can be used for beam refinement without impacting the system acquisition time, the minimum peak EIRP requirements for msg1 in IDLE and INACTIVE can be kept same as RRC_CONNECTED mode - Option 2: It would be beneficial to study and propose solutions on how the UE can do beam refinement in IDLE and INACTIVE modes for msg3. These would be helpful in a large number of practical network scenarios one of which we have stated in our discussion. - Option 3: The RF requirement should accommodate both rough beam and fine beam. - Option 4: RAN4 to conclude that no more discussion on beam type selection for beam correspondence requirements for initial access and RRC_INACTIVE state. - Option 5: Rough beam or Fine beam used in IA is up to UE implementation and the requirements should be implementation agnostic. Way forward/Agreements: - FFS

Beam refinement in RRC_INACTIVE and RRC_IDLE

One of the main concerns voiced by many companies in the ongoing 3GPP RAN 4 discussions for defining beam correspondence requirements in IDLE and INACTIVE mode is that the UE may use a rough beam for initial access.

This in general would then mean that the minimum peak EIRP that the UE can achieve would be lower than what it could achieve using a refined beam. For a UE to have a narrow beam, in the IDLE and INACTIVE modes, it sweeps the SSB beams. For it to do so, assuming it has 3 panels, the minimum time it would require is 20ms per SSB/ 0.125 ms per slot * 3 sweeps = 480 slots

Figure 1: UE beam refinement using SSB

For a narrow beam, the UE needs to sweep at least 3 times which translates to around 480 slots in FR2 assuming SSB periodicity is 20 ms and UE has 3 panels.

In the last 3GPP meeting, it was also stated that the UE chooses a rough beam to speed up the initial acquisition process[2].

In case the intent for the UE to use a rough beam is indeed to speed up the initial acquisition process, then the UE can use alternate reference signals in the IDLE and INACTIVE modes for beam refinement without compromising on the time taken for system acquisition.

3GPP Rel-17 has defined Tracking reference signal (TRS) signals in IDLE and INACTIVE modes. The highest configurable allocation of TRS is two consecutive slots with 2 symbols per slot TRS for IDLE mode and a periodicity of 10 slots, which amounts to 4 occasions to refine UE beam every 1.25 ms in FR2 120 KHz SCS. Hence as compared to the SSB which is periodically broadcasted very 20 ms, the UE has many more occasions to refine its beams using the IDLE/INACTIVE mode TRS

Beam refinement for msg1 in IDLE and INACTIVE modes can be done using Rel-17 Tracking reference signal (TRS) signals in IDLE and INACTIVE modes which have a periodicity of 10 slots. (1.25 ms in FR2 120 KHz SCS).

Figure 2: Tracking reference signal in IDLE/INACTIVE mode

This means that by using the Tracking reference signal instead of the SSB in IDLE and INACTIVE modes for beam refinement, the UE can still have a narrow beam for msg1 without major impacts on the overall system acquisition time.

Using the Tracking reference signal (TRS) signals in IDLE and INACTIVE modes for beam refinement UE can refine its beams for msg1 without compromising on system acquisition time.

Since the UE has enablers such as Rel-17 TRS signals which can be used for beam refinement without impacting the system acquisition time, the minimum peak EIRP requirements for msg1 in IDLE and INACTIVE can be kept same as RRC_CONNECTED mode

There could also be a case during the Random access procedure or during small data transmission procedures where the UE may need to refine its beam for msg3 especially if it is power limited. In such cases, the UE may have just enough PA power to send msg1 which consists of the preamble. However, this power will not suffice to successfully transmit msg3 of the 4-step Random Access procedure since msg3 consists of the payload as well. One way by which the UE can mitigate this and ensure msg3 reaches the gNB is by increasing the Tx antenna gain, i.e. refining its beam further such that it has a very narrow beam.

For the UE to refine its Tx beam for msg3 specifically, it needs sufficient time between msg1 and msg3 to further sweep its Rx beam during enough SS bursts, refine its Rx beam and deduce its best Tx narrow beam. The msg2 of the Random Access procedure carries the uplink grant which as per the current specifications can be a maximum of around 4.75 ms. However, assuming a default periodicity of 20 ms for the SSBs, even if a UE wants to refine its beam for msg3 by sweeping the SSBs, it will not be able to do so due to the reason stated above.

The alternative for such a UE would be to sweep some other reference signals similar to the TRS which has a lower periodicity between msg1 and msg3 to refine its beams for the successful transmission of msg3 which has the payload. Beam refinement procedures are currently only specified in RRC_Connected mode, where the network may schedule CSI-RS with the repetition ‘on’. Similar reference signals are not available in IDLE/INACTIVE modes.

Figure 3: Legacy beam refinement procedure

There are no defined procedures nor enablers which can help the UE to refine its beam for msg3 in IDLE/INACTIVE modes today. It would be worth studying how beam refinement can be enabled for UEs in IDLE and INACTIVE modes for msg3.

For the UE to refine its Tx beam for msg3 specifically, it needs sufficient time between msg1 and msg3 to further sweep its Rx beam during enough SS bursts, refine its Rx beam and deduce its best Tx narrow beam.

Current 3GPP specifications do not have defined procedures nor enablers which can help the UE to refine its beam in IDLE/INACTIVE modes for msg3.

It would be beneficial to study and propose solutions on how the UE can do beam refinement in IDLE and INACTIVE modes for msg3. These would be helpful in a large number of practical network scenarios where there could be a need for the UE to refine its beams for msg3

Conclusion

In the paper, the following Observations and Proposals were made:

Observation 1: For a narrow beam, the UE needs to sweep at least 3 times which translates to around 480 slots in FR2 assuming SSB periodicity is 20 ms and UE has 3 panels.

Observation 2: Beam refinement for msg1 in IDLE and INACTIVE modes can be done using Rel-17 Tracking reference signal (TRS) signals in IDLE and INACTIVE modes which have a periodicity of 10 slots. (1.25 ms in FR2 120 KHz SCS).

Observation 3: Using the Tracking reference signal (TRS) signals in IDLE and INACTIVE modes for beam refinement UE can refine its beams for msg1 without compromising on system acquisition time.

Proposal 1: Since the UE has enablers such as Rel-17 TRS signals which can be used for beam refinement without impacting the system acquisition time, the minimum peak EIRP requirements for msg1 in IDLE and INACTIVE can be kept same as RRC_CONNECTED mode

Observation 4: For the UE to refine its Tx beam for msg3 specifically, it needs sufficient time between msg1 and msg3 to further sweep its Rx beam during enough SS bursts, refine its Rx beam and deduce its best Tx narrow beam.

Observation 5: Current 3GPP specifications do not have defined procedures nor enablers which can help the UE to refine its beam in IDLE/INACTIVE modes for msg3.

Proposal 2: It would be beneficial to study and propose solutions on how the UE can do beam refinement in IDLE and INACTIVE modes for msg3. These would be helpful in a large number of practical network scenarios where there could be a need for the UE to refine its beams for msg3

- Annex: Text proposal to TR 38.891

## 6.2	Implementation impact to UE

One of the main characteristics of 5G NR beam management procedures is using directive beams hence requiring beam alignment between the UE and the base station. In FR2, the UE uses the beam refinement procedure to have narrow highly directive beams towards the gNodeB thereby increasing the uplink coverage. During a Random access procedure, the UE could use both the rough and fine beams. The UE could first transmit a broad beam and when RAR is not received, the UE in subsequent RA attempts could further refine its beams.

An example of UE simulated at 28 GHz (band n257) with 1x4 array placed on the side of a smart phone in free space (no user presence) is simulated in Figure 1 and Figure 2 to show the EIRP coverage curves.

Figure 1: EIRP spherical coverage for a broad beam or a single narrow beam.

Figure 2: EIRP spherical coverage for envelope of narrow beams

In this example, the current requirements in RRC_CONNETED mode of 11.5 dBm EIRP at 50MHz CBW and at 50%-ile is represented with the grey box. In Figure 1, the blue and red curves represent the simulated spherical coverage of a UE with a broad beam or a single narrow beam. The blue curve represents the single narrow beam while the red curve represents the broad beam. It is assumed that the PA deliver 23 dBm power to the antenna array and that the antenna array has a 6 dB antenna implementation loss. The broad beam is generated with a single element activation of the array, i.e., about 90 degrees radiation pattern in the azimuthal plane. In Figure 2, the blue curve represents the envelope of narrow beams with a single panel with 4 elements active. As illustrated above, the EIRP spherical coverage is sensitive to beam refinement.

During random access procedure for Initial access or even handovers, cell reselection and other scenarios where Random access is required, the 3GPP standards do not mandate the kind of beam the UE needs. This is up to UE implementation if a rough beam or a narrow beam is to be used and how to achieve beam refinement.

For a UE to have a narrow beam, in the IDLE and INACTIVE modes, it has to sweep the SSB beams. For it to do so, assuming it has 3 panels, the minimum time it would require is 20ms per SSB/ 0.125 ms per slot * 3 sweeps = 480 slots.

One of the ways, the UE can refine its beams in the IDLE mode is by using Tracking Reference Signals. 3GPP Rel-17 has defined Tracking reference signal (TRS) signals in IDLE and INACTIVE modes. The highest configurable allocation of TRS is two consecutive slots with 2 symbols per slot TRS for IDLE mode and a periodicity of 10 slots, which amounts to 4 occasions to refine UE beam every 1.25 ms in FR2 120 KHz SCS.

Hence as compared to the SSB which is periodically broadcasted very 20 ms, the UE has many more occasions to refine its beams using the IDLE/INACTIVE mode TRS. This means that by using the Tracking reference signal instead of the SSB in IDLE and INACTIVE modes for beam refinement, the UE can still have a narrow beam for msg1 without major impacts on the overall system acquisition time.

- References

- RP-222909, Revised WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3,  Rapporteur (Nokia, Xiaomi)

- R4-2302476: On BC requirement for IA/RA-SDT/CG-SDT

- R4- 2306600: WF on BC in RRC_INACTIVE and initial access