3GPP TSG-RAN WG4 Meeting #107 	R4-2308620

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations on mobile IAB coexistence

Agenda item:	8.34.2 Co-existence study	[NR_mobile_IAB-Core]

Document for:	Approval

Introduction

RAN4#106-bis meeting continued discussion on the work item for Mobile IAB [1]. The WI objectives specific for RAN4 were defined in work item description as follows:

RAN4 is expected to study impact on RF and RRM requirements: - Conduct co-existence study to assess the impact of moving cells. Based on the study outcome, specify RF and RRM requirements and mechanisms for the mobile IAB-node to enable co-existence, if needed. - Specify RRM requirements for the mobile IAB-node to enable IAB-node mobility, if needed.

Agreements from RAN4#106-bis meetings are captured in agreed way forwards [2]. In this contribution we elaborate further on the assumption for the co-existence evaluation.

Discussion

Interference scenarios

As the starting point, WF lists the interference scenario to be considered for co-existence. Tables from [2] copied in Appendix A for convenience.

As indicated, there are certain cases that are already covered in the existing specified requirements, namely:

- IAB ßà NR (Table 1): Cases 1, 4, 6 and 8.

Proposal 1. Interference scenarios 1, 4, 6 and 8 in [2] can be excluded from the mobile IAB co-existence.

Following agreement was also made by RAN4#106bis-e:

- RAN4 to focus on antenna installation with IAB-MT antenna outside (e.g., rooftop) the vehicle and IAB-DU antenna inside the vehicle.

- FFS on the MT /DU antenna modeling

Regarding the IAB-MT installation, the earlier assumptions used for co-existence can apply just that the antenna hight can be mobile IAB specific.

Proposal 2. Assumed assumptions for IAB-MT antenna structure and modeling can be applied for mobile IAB except the modified antenna hight.

IAB-DU antenna when assumed to be inside the vehicle possibly would require some modification. There could be different options to model the installation e.g. adjusting the IAB-DU TX power and RX level, or adding a specified attenuation factor reflecting the isolation between the inside and outside cells. Other options may also exist. The isolation factor needs to be discussed whether some relevant value(s) exist.

Proposal 3. RAN4 to discuss suitable options for modeling the mobile cells and how the installation inside the vehicle can be considered.

Conclusion

In this contribution, we elaborated further the assumptions for co-existence evaluations, and we ended up with following proposals:

Proposal 1. Interference scenarios 1, 4, 6 and 8 in [2] can be excluded from the mobile IAB co-existence.

Proposal 2. Assumed assumptions for IAB-MT antenna structure and modeling can be applied for mobile IAB except the modified antenna hight.

Proposal 3. RAN4 to discuss suitable options for modeling the mobile cells and how the installation inside the vehicle can be considered.

- References

- RP-222671, Mobile IAB (Integrated Access and Backhaul) for NR

- R4-2305935, WF on mIAB RF requirements and coexistence, Qualcomm Incorporated

- Appendix A

Interference scenarios as defined in the WF [2].

Table 1:  Scenarios simulated for layout 1&2 between IAB <-> NR

| Deployment Scenario (Aggressor->Victim) | Case No. | Baseline | Aggressor | Victim | Comemnts (related RF parameter) |
| --- | --- | --- | --- | --- | --- |
| IAB -> NR Macro | 1 | NR DL active:UE receiving IAB (unenabled): access link ( IAB-DU) DL inactive, backhaul link ( IAB-MT) inactive | IAB: access link (IAB-DU) inactive, backhaul link ( IAB-MT) DL active : IAB donor transmitting | NR DL: UE receiving | IAB donor ACLR is the same with normal BS, this case could be skipped. |
|  | 2 | NR DL active: UE receiving IAB (unenabled): access link ( IAB-DU) DL inactive, backhaul link ( IAB-MT) inactive | IAB: access link (IAB-DU) inactive, backhaul link ( IAB-MT) inactive : IAB-DU transmitting | NR DL active: UE receiving | IAB-DU Tx power IAB-DU ACLR |
|  | 3 | NR UL active: BS receiving, UE transmitting IAB(unenabled) : access link (IAB-DU) DL inactive, backhaul link (IAB-MT) inactive | IAB: access link (IAB-DU) inactive, backhaul link (IAB-MT) UL active: IAB-MT transmitting | NR, UL | IAB-MT Tx power IAB-MT ACLR |
|  | 4 | NR UL active: BS receiving, UE transmitting IAB(unenabled) : access link (IAB-DU) DL inactive, backhaul link (IAB-MT) inactive | IAB: access link (IAB-DU) active, backhaul link (IAB-MT) UL inactive: IAB-DU receiving | NR, UL | This is legacy scenario, could be skipped. |
| NR Macro -> IAB | 5 | IAB: access link ( IAB-DU) DL inactive, backhaul link DL( IAB-MT) active: IAB-MT receiving NR DL and UL inactive: | NR DL active: BS transmitting | IAB backhaul link: IAB-MT receiving | IAB-MT ACS |
|  | 6 | IAB: access link (IAB-DU) inactive, backhaul link (IAB-MT) UL active : IAB-Donor receiving, IAB-MT transmitting NR UL and DL inactive; | NR UL active: UE transmitting, BS receiving | IAB backhaul link: IAB-Donor receiving | IAB donor ACS is the same with normal BS, this case could be skipped. |
|  | 7 | IAB: access link (IAB-DU) UL active, backhaul link (IAB-MT) inactive: IAB-DU receiving NR UL inactive; | NR UL active: UE transmitting, BS receiving | IAB access link : IAB-DU receiving | IAB-DU ACS |
|  | 8 | IAB: access link ( IAB-DU) DL active, backhaul link DL( IAB-MT) inactive: IAB-DU transmitting, UE receiving NR DL and UL inactive: | NR DL active: BS transmitting | IAB access link: UE receiving | UE and BS are legacy, this scenario can be skipped. |

Table 2:  Scenarios simulated for layout 1&2 between IAB <-> IAB

| Deployment Scenario (Aggressor->Victim) | Case No. | Baseline | Aggressor | Victim | Comemnts (related RF parameter) |
| --- | --- | --- | --- | --- | --- |
| IAB Agressor -> IAB Victim | 9 | IAB victim DL,backhaul link active: IAB donor transmitting, IAB-MT receiving IAB aggressor (unenabled): access link ( IAB-DU) DL inactive, backhaul link ( IAB-MT) inactive | IAB Agressor: access link (IAB-DU) inactive, backhaul link ( IAB-MT) DL active : IAB donor transmitting | IAB victim DL: IAB-MT receiving | IAB donor ACLR is the same with normal BS IAB-MT ACS |
|  | 10 | IAB Victim DL active; IAB agressor (unenabled): access link ( IAB-DU) DL inactive, backhaul link ( IAB-MT) inactive | IAB: access link (IAB-DU) active: IAB-DU transmitting backhaul link ( IAB-MT) inactive : IAB-DU transmitting | IAB victim DL: IAB-MT receiving | IAB-DU Tx power IAB-DU ACLR IAB-MT ACS |
|  | 11 | IAB victim UL: IAB-MT transmitting, IAB-Donor receiving IAB agressor (unenabled) : access link (IAB-DU) DL inactive, backhaul link (IAB-MT) inactive | IAB: access link (IAB-DU) inactive, backhaul link (IAB-MT) UL active: IAB-MT transmitting | IAB victim UL: IAB-MT transmitting, IAB-Donor receiving | IAB-MT Tx power IAB-MT ACLR |
|  | 12 | IAB victim UL: IAB-DU receiving IAB agressor (unenabled) : access link (IAB-DU) DL inactive, backhaul link (IAB-MT) inactive | IAB: access link (IAB-DU) inactive, backhaul link (IAB-MT) UL active: IAB-MT transmitting | IAB victim UL: IAB-DU receiving | IAB-MT Tx power IAB-MT ACLR IAB-DU ACS |