3GPP TSG-RAN WG4 Meeting #107 	R4-2308628

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RAN1 LS reply

Agenda item:	8.20.2.3

Document for:	Discussion

Introduction

RAN1 sent LS R1-2304182 [1] to RAN4 asking about the Noise Figure model agreed in RAN4#106 R4-2302885 [2]:

Working assumption: For SLS of duplex evaluation in RAN1, the BS noise figure is modelled as piece wise linear based on the total received power (P) as NF dB= C , P<A (D- C)(B-A)P-A+C, A≤P<B - For FR1, A = -43dBm, B = -25dBm, C = 5dB, D = 14dB - P is in dB scale. The linear value of total received power is the linear sum of all received power, including wanted signal, co-channel and adjacent-channel UE-gNB interference, self-interference, co-channel and adjacent-channel co-site inter-sector interference and co-channel and adjacent-channel inter-site gNB-gNB interference. - adjacent-channel interference is only used for SBFD deployment case 4 - If P is larger than B, the receiver will be blocked. - Send LS to RAN4 to ask the following questions: - Whether the above values of A, B, C and D can be used for all the BS classes in FR1? If not, what are the values of A, B, C and D for each of BS classes in FR1? - Whether fixed noise figure can be used for FR2-1 in RAN1 evaluation? If not, what are the values of A, B, C and D for BS classes in FR2-1? - The feasibility and applicable scenarios of improved noise figure, e.g., by introducing additional interference reduction techniques like subband filtering.

We also note that RAN4 has sent another LS to RAN1 in RAN4 #106-bis-e (R4-2306004), which clarifies some of the questions asked by RAN1.

Furthermore, RAN4 made the following agreement on noise figure model (i.e., noise figure over average total input power, captured in RAN4 LS R4-2302885): - For typical FR1 WA BS implementation, companies have different views on sub-band filter, and based on companies’ proposal on the implementation with sub-band filter, the blocking performance can be improved around 10 dB. The following values of A and B can apply to the noise figure model if sub-band filter adopted. - The values of A, B, C and D: - A = -35dBm - B = -17dBm - C = 5dB - D = 14dB - Note: RAN4 has not reach consensus on the implementation feasibility of sub-band filter. RAN4 will further evaluate and update to RAN1 if needed. - For FR1 MR BS class, the following noise figure model can be used as starting point: - (A, B, C, D) = (-38, -20, 10, 19) - Point A is suggested to follow the in-band blocking requirement as specified in TS 38.104, i.e. -38dBm. - Point C is suggested to follow the assumed noise figure, i.e. 10dB. - For FR1 LA BS class, the following noise figure model can be used as starting point: - (A, B, C, D) = (-35, -17, 13, 22) - Point A is suggested to follow the in-band blocking requirement as specified in TS 38.104, i.e. -35dBm. - Point C is suggested to follow the assumed noise figure, i.e. 13dB. - For FR2-1 BS, the following noise figure model can be used as starting point: - The values of A, B, C and D: - A = [-58] dBm - B = [-40] dBm - C = 10dB - D = [19] dB

In this document, we present our views regarding the LS received from RAN1.

Discussion

In this Section we discuss the questions asked by RAN1.

In our view, the first question “Whether the above values of A, B, C and D can be used for all the BS classes in FR1? If not, what are the values of A, B, C and D for each of BS classes in FR1?” was already answered by RAN4 in the LS sent in RAN4 #106bis-e. RAN4 provided values for different types of BS: WA, MR and LA, and no further answer is needed.

We discuss the third question in the next section.

Question 1 in RAN1 LS was already answered by RAN4.

- 2.1 Implementation feasibility of sub-band filtering

The third question asked by RAN1 “The feasibility and applicable scenarios of improved noise figure, e.g., by introducing additional interference reduction techniques like subband filtering” has been partially answered in the RAN4 LS. However, even though RAN4 provided values for NF model assuming sub-band filtering in R4-2306004, we would like to discuss the implementation feasibility of subband filtering.

Even though RAN4 agreed, sub-band filtering before or after the LNA is not seen feasible as it will have the effect of:

  - High insertion loss before the LNA will increase the receiver noise figure and negate any system gains of SBFD.

  - High insertion loss placed after the LNA does not reduce LNA linearity requirements. Improved linearity LNAs are needed that add to cost and power consumption, which leads to thermal management issues. Other challenges include:

    - Significant insertion loss → NF increase that can hardly be accommodated in the 1 dB desensitization budget.

    - Considerable transition band and temperature dependency of the passband position → larger guard band between the sub-bands needed and less BW is usable for UL

    - Group delay distortion close to the cut-off frequencies

    - Incompatibility with a typical multi-carrier gNB design

    - Increased complexity as switches are needed for by-pass in UL slots for full BW

    - Additional space needed in RX chain that is not available in typical gNB design

    - Overall additional power consumption which leads to thermal management issues

    - Frequency drift over temperature that will impact filter insertion loss and rejection performance, hence impacts the RX lineup performance

It is proposed to send LS to RAN1 and clarify on the feasibility and applicable scenarios of improved noise figure.

Conclusion

In this document, we present our views regarding the LS received from RAN1. In the paper, the following Observation and Proposal were made:

Observation 1: Question 1 in RAN1 LS was already answered by RAN4.

Proposal 1: It is proposed to send LS to RAN1 and clarify on the feasibility and applicable scenarios of improved noise figure.

- References

- R1-2304182, Draft LS on BS noise figure model for duplex evolution

- R4-2302885, Reply LS on interference modelling