3GPP TSG-RAN WG4 Meeting #107	R4-2308637

Incheon, KR, 22 – 26 May, 2023

Title: 	Discussion on remaining issues in Rel-17 MGE

Source: 	Huawei, HiSilicon

Agenda item:	5.2.10.3

Document for:	Discussion

Introduction

Remaining issues for Rel-17 MG enhancement are discussed in RAN4#106, and outcomes are captured in WF [1]. Based on [1] the following issues are to be further discussed.

- Association for MO that can be measured without MG

- Measurement of LTE MOs with con-MG

In this paper we will provide our views on the remaining issues in Rel-17 MG enhancement.

Discussion

Association for MO that can be measured without MG

Issue 2-1: MO associated with a MG but measured without MG - Wayforward - For an MO that can be measured outside MG and associated with an MG, FFS - Option 1: the MO is always measured within the associated MG - Option 2: the MO is measured within the associated MG only when it needs to be measured with MG

We support option 2.

RAN4 has categorised MOs into measurement with MG and without MG. In cl. 9.2.1 it is specified when intra-frequency MOs are measured without MG and when with MG. For example, when intra-frequency MO is within active BWP, or when UE indicates via NFG or NCSG signalling that it is capable to measure the MO without MG without interruption or NCSG, the MO is measured without MG. Otherwise, the MO is measured with MG or NCSG. In cl. 9.3.1 similar is specified for inter-frequency measurements.

Since Rel-15, MO that can be measured without MG is measured outside MG, no matter whether MG is configured or not. Of course, there is one exception when the MO’s SMTC is fully overlapped with MG, in which case the MO is measured within MG. When the SMTC is partially overlapped with MG, the MO is still measured outside MG and scaling factor Kp is introduced to account for the lost SMTC occasions.

When it comes to con-MG, we understand the same principle should be used. An MO being associated with a MG does not mean it should be measured within MG. The association may be configured considering the SSB is outside some BWP, and it is same case as in Rel-15 when a MG is configured. In Rel-15 only single MG can be configured, and it can be regarded as associated with all MOs.

Proposal 1: An MO that can be measured without MG is measured outside MG unless its SMTC is fully overlapping with the union of con-MGs, regardless of if the MO is associated to a MG or not.

Issue 2-2: MO fully overlapping with the union of the MG1 and MG2 - Wayforward - the MO should be measured within the associated MG in the following cases, - When a MO is fully overlapping with the associated MG, or - FFS: When a MO is partially overlapping with the associated MG and fully overlapping with the union of the ConMGs. - Whether to clarify this in spec is FFS

As discussed above, for an MO that can be measured without MG there is one exception when the MO’s SMTC is fully overlapped with MG, in which case the MO is measured within MG. When the SMTC is fully overlapping with the associated MG, it is agreed last meeting that the MO should be measured within the associated MG.

When the SMTC is partially overlapping with the associated MG and fully overlapping with the union of the con-MGs, we see it straightforward that the MO should still be measured within the associated MG.

Proposal 2: For an MO that can be measured without MG, when its SMTC is fully overlapping with the union of con-MGs, the MO is measured within the associated MG.

Measurement of LTE MOs with con-MG

Issue 2-3: E-UTRAN measurement with con-MGs - Wayforward - FFS whether all E-UTRAN measurement objects are expected to be associated with a single concurrent gap pattern or not when both E-UTRAN measurement objects and other type of measurements are configured - Option 1: Yes. - Option 2: No. - Option 3: Up to UE capability (e.g. concurrentMeasGapEUTRA-r17) - For UE supporting the capability, different LTE MOs can be associated with multiple MGs. - For UE not supporting the capability, all LTE MOs can be associated with only a single MG.

In last meeting companies have different understanding on whether UE should support measuring LTE MOs with multiple con-MGs. We understand UE capability concurrentMeasGapEUTRA-r17 also applies when both LTE MOs are MOs of other RATs are configured.

It is true that in WF [2] the issue title is when only LTE MOs are configured, but we understood the issue is common for both cases where only LTE MOs are configured and where both LTE and NR MOs are configured, and this is why in the UE capability description there is no limit on the applicable scenario.

Issue 2-1-1: Whether concurrent gaps are allowed in the case when only E-UTRAN measurement objectives are configured <Agreement in May 9th GTW session>: Agree on Option 3 - Up to UE capability - For UE supporting the capability, different LTE MOs can be associated with multiple MGs. - For UE not supporting the capability, all LTE MOs can be associated with only a single MG. - Issue 2-1-2: UE capability for concurrent gap with only E-UTRAN measurement objectives < Agreement>: The detail UE capability Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs. Concurrent measurement gap; 19-2-1; Concurrent measurement gaps for E-UTRAN measurement objectives; Capability of supporting configurations of E-UTRAN measurement objectives associated with more than 1 concurrent measurement gaps; 19-2; yes; no. Consequence if the feature is not supported by the UE; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. All configured E-UTRAN measurement objectives shall be associated with a single measurement gap; per UE; No; No; Optional with capability signalling.

Technically, from UE side, the concern is that existing LTE measurements are based on single MG, and if multiple MGs are used, the UE implementation for LTE measurement needs to be enhanced in the same way as for NR measurement, and this will add additional cost and complexity. This is same case no matter if NR MOs are configured with LTE MOs or not.

From NW side, LTE can be measured at any time (this is different from NR measurements which are based on SSB and CSI-RS), so all LTE MOs can be measured with one MG, and the motivation to use multiple MGs is unclear. We do not see additional motivation to have LTE MOs measured in different MGs just because there are NR MOs configured.

It is noted that when UE does not support this capability, it means all LTE MOs should be associated to a single MG, but NR MOs can still be associated to different MGs.

Proposal 3: UE capability concurrentMeasGapEUTRA-r17 also applies when both LTE MOs are MOs of other RATs are configured.

Conclusions

In this paper we provided our views on remaining issues in Rel-17 MG enhancement.

Proposal 1: An MO that can be measured without MG is measured outside MG unless its SMTC is fully overlapping with the union of con-MGs, regardless of if the MO is associated to a MG or not.

Proposal 2: For an MO that can be measured without MG, when its SMTC is fully overlapping with the union of con-MGs, the MO is measured within the associated MG.

Proposal 3: UE capability concurrentMeasGapEUTRA-r17 also applies when both LTE MOs are MOs of other RATs are configured.

Reference

- R4-2303206, WF on R17 NR MG enhancements RRM maintenance, Huawei, HiSilicon, ZTE

- R4-2210585, WF on R17 NR MG enhancements – multiple concurrent MGs, MediaTek inc.