3GPP TSG-RAN4 Meeting #107	R4-2308638

# Incheon, KR, 22 – 26 May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3258 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on NCSG related requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-Core |  |  |  |  |  | Date: |  |  | 2023-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the deriveSSB-IndexFromCellInter-r17 tolerance requirements, cells on the target carrier and reference carrier are assumed to be in sync. Cells should be in sync with the reference cell rather than the reference carrier. Besides, the definition of reference cell is unclear. - It was agreed that NCSG cannot be used for 2G/3G or PRS measurement, but the current wording of applicability of NCSG may be mis-interpreted as NCSG requirements do not apply when these measurements are configured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the deriveSSB-IndexFromCellInter-r17 tolerance requirements by replacing reference carrier with reference cell, and clarify the definition of reference cell. - Update the applicability of NCSG so that the requirements for 2G/3G or PRS measurement do not apply when UE is configured with NCSG. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NCSG related requirements are incomplete or incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.9.1, 9.1.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

7.9.1	Minimum requirements

When deriveSSB-IndexFromCellInter-r17 is enabled, the UE assumes frame boundary alignment (including half frame, subframe and slot boundary alignment) across cells on the target carrier and reference cell is within a tolerance not worse than min(2 SSB symbols of target carrier, 1 PDSCH symbol of the reference cell) and the SFNs of all cells on the target carrier and reference cell are the same. The reference cell is the serving cell which is used for SSB indexes derivation as indicated via deriveSSB-IndexFromCellInter-r17.

<End of Change 1>

<Start of Change 2>

#### 9.1.9.2	Requirements applicability

Requirements in clause 9.1.9 apply for UE capable of NCSG in standalone NR in both FR1 and FR2 (including FR1+FR2 CA), provided UE is configured with only NCSG and no other measurement gap is configured, and UE is configured with

SSB based intra-frequency measurement (including measurement on de-activated SCC and measurement on dormant SCell), and/or

SSB based inter-frequency measurement, and/or

Inter-RAT E-UTRAN measurement.

Requirements for the following measurements do not apply if UE is configured with NCSG.

Inter-RAT GSM measurement, or

Inter-RAT UTRAN measurement, or

PRS measurement.

<End of Change 2>