3GPP TSG-RAN WG4 Meeting #107	R4-2308663

Incheon, KR, 22 – 26 May, 2023

Title: 	Discussion on collision handling for MUSIM gaps

Source: 	Huawei, HiSilicon

Agenda item:	8.26.2.2

Document for:	Discussion

Introduction

Collision handling related to MUSIM gaps are discussed in RAN4#106-bis-e and the outcomes are captured in [1]. Based on [1] the following issues need to be further discussed.

- MUSIM gap priority

- Collision between different MUSIM gaps

- Collision between MUSIM and legacy MG

- Collision between MUSIM gaps and NW A signals

In this paper we will provide our views on open issues in collision handling related to MUSIM gaps.

Discussion

MUSIM gap priority

Issue 2-1-4-1: Constraints on MUSIM gap priority configuration from NW A - Proposals - P1: NW A maintains the same relative priorities requested by the UE; The configured priority level may or may not be the same as that requested by UE. (vivo Apple MTK Xiaomi Huawei Qualcomm Nokia Charter ZTE) - P1-a: Based on P1, NW A is not required to keep the relative priority order for a particular MUSIM gap when the MGRP of that particular MUSIM gap is less than a threshold, in this scenario NW A will still keep the same relative order of the other MUSIM gaps except for that particular MUSIM gap (vivo) - P2: When MUSIM gaps with equal priority is allowed, if UE requests two MUSIM gaps with the same priority X and if the network configures both gaps, then both gaps must be assigned a common priority X’. X’ may or may not be equal to X. (vivo Apple Qualcomm oppo) - P4: If the network cannot fulfill the UE priority requests the network may chose not to assign the requested MUSIM gaps (Nokia Qualcomm) - P6: In the special case when both one MUSIM gap and one other MG gap has set the highest priority level in gapPriority-r17 IE, then we propose that MUSIM gap has the ability to signal with an extra 1-bit to indicate higher priority than the highest level in gapPriority-r17 IE (Charter) Agreements: No Candidate options: Recommendations: Continue discussion

We think it is reasonable that NW A follows the relative priority among MUSIM gaps as indicated by UE when configuring priority for MUSIM gaps. Otherwise it does not make much sense for UE to indicate the preferred priority for MUSIM gaps. It is out of NW A’s responsibility to decide the relative priority among MUSIM gaps.

Proposal 1: NW A should maintain the relative priority among MUSIM gaps as indicated by UE when configuring priority for MUSIM gaps.

Issue 2-1-4-2: Constraints on MUSIM gap priority indication from UE side - Proposals - P1: When UE requests the MUSIM gaps, the MGRP of highest priority gap should be larger than 160ms; When UE requests only one MUSIM gap, the MGRP should be larger than 80ms (Ericsson ZTE) Agreements: No Candidate options: Recommendations: Continue discussion

We prefer not to define constraints on MUSIM gap priority indication from UE side.

The assumption in P1 is that UE will suggest MUSIM gap used for paging as highest priority, and paging cycle is always larger than 160ms. However, UE may request one MUSIM gap for multiple purposes, and it may suggest one MUSIM gap with lower MGRP with highest priority. On the other hand, it is always up to NW A to decide whether to configure the requested MUSIM gap or not, i.e. if NW A considers the MGRP for a MUSIM gap is too small and it may impact the use of legacy MG for NW A mobility, NW A can just object the MUSIM gap request.

Proposal 2: RAN4 not to define constraints on MUSIM gap priority indication from UE side.

Issue 2-1-5: Priority setting for aperiodic MUSIM gaps Agreements from GTW - Option 1 (QC, Nokia, vivo, Charter, Xiaomi) - The priority level of aperiodic MUSIM gap can be configured by NW A - If the priority level is not configured by NW A then the aperiodic MUSIM gap by default has the highest priority level - The aperiodic MUSIM gap priority level can be optionally requested by UE from NW A - Option 2 (MTK, Huawei, Apple, Ericsson, OPPO): - Aperiodic MUSIM gap by default has the highest priority level. - The gap priority level is not explicitly configured by the NW Agreements: No Candidate options: Recommendations: try to down select between option 1 and 2

We support option 2.

In our view, AP gap by its nature should be always prioritized. It is noted that AP gap is still configured by NW A, i.e. NW A can choose to configure the requested AP gap or not. If NW A chooses to configure the AP gap, it does not make much sense to associate it with a lower priority than legacy MG, as it will cause the AP gap to be dropped, and it is same as not configuring the AP gap since AP gap has only one occasion. Although option 1 can provide some flexibility, based on discussion last meeting, there seems to be little use case for such flexibility from either NW A or UE perspective.

Proposal 3: Aperiodic MUSIM gap by default has the highest priority level. No need for UE to request or for NWA to configure a priority for aperiodic gap.

Issue 2-1-6: Order for applying the priority when number of colliding MGs is larger than 2 - Proposals: - P1: Collisions between gaps are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority (vivo xiaomi Qualcomm MTK oppo Huawei Apple) - P2: RAN4 to postpone multiple gap collision issue until RAN4 has a clear understanding on MUSIM gaps’ priority. (Ericsson Nokia Chapter ZTE) Agreements: No Candidate options: Recommendations: Discuss at the next meeting

With MUSIM gaps, the number of colliding gaps can be larger than 2, e.g. two MUSIM gaps and one legacy MG. For this case, RAN4 has to discuss the order for applying the priority when the 3 gaps are configured with 3 different priorities. Similar issue has been discussed in RAN4#103-e for concurrent MGs in Issue 2-2-2 in R4-2210478, and we think P1 is a reasonable solution.

Proposal 4: Collisions between gaps are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority.

Collision between different MUSIM gaps

Issue 2-2-1: Definition of the collision between different MUSIM gaps - Proposals - Option 1: The gap proximity condition for the Rel-17 concurrent gap collision should be reused for the collision between different MUSIM gap when priority rules are used to handle the collision between MUSIM gaps (Qualcomm Apple MTK xiaomi Ericsson CMCC ZTE oppo vivo) - Option 2: Postpone the discussion till issue 2-2-2 is stable (Huawei Qualcomm ZTE) - Option 3: A collision between MUSIM gaps means a physical overlap in time domain between two MUSIM gaps and RAN4 does not define ‘proximity’ for collisions between MUSIM gaps. (Nokia) Agreements: No Candidate options: Recommendations: Continue discussion.

Whether a definition of the collision between different MUSIM gaps is needed depends on the solution used to handle the collision. As discussed next, if keep solution is used then there is no need for such definition. Otherwise, we agree that option 1 is reasonable. We suggest to postpone the definition for collisions between MUSIM gaps after RAN4 has conclusion on collision handling between MUSIM gaps.

Proposal 5: Postpone the definition for collisions between MUSIM gaps after RAN4 has conclusion on collision handling between MUSIM gaps.

Issue 2-2-2: Solutions for collision between different MUSIM gaps - Proposals - Option 1: Priority based solution is used for collision between different MUSIM gaps when these collided MUSIM gaps are assigned with different priority levels (vivo Apple Qualcomm) - Option 2: Priority based solution is used for collision between different MUSIM gaps (MTK xiaomi oppo Charter) - Option 2-1: The priority handling rule shall be used if it collides with the periodic gaps (except the paging gap) (ZTE) - Option 3: Keep solution (keep all collided MUSIM gap) is used when MUSIM gaps have same priority (Apple Qualcomm oppo vivo) - Option 3a: The overlapping MUSIM gap occasions under scenario of option 3 shall be merged. (Apple) - Option 3b: whether multiple MUSIM gaps can be assigned the same priority level or not subject to UE capability (Qualcomm vivo) - Option 3c: Up two periodic MUSIM gaps can be configured with the same priority and inform such the configuration to RAN2 (oppo) - Option 4: MUSIM gap ‘keep rule’ will be applied in some certain scenarios, such as Paging monitoring and AGC (Ericsson) - Option 4-1: The kept/merged solution is used if the second gap in collision is paging gap (ZTE) - Option 5: Keep solution (keep all collided MUSIM gap) is used when different MUSIM gaps collide (Huawei) - Option 6: For keep solution, UE shall under defined conditions not drop a colliding MUSIM gap of lower priority, provided the UE perform all actions related to the colliding MUSIM gaps of higher priority or priorities. RAN4 shall define these conditions. (Nokia) Agreements: No Recommendations: Consider and down-select from the following options at the next meeting: - Option 1: Priority based solution is used for collision handling between different MUSIM gaps - Option 2: Keep solution (keep all collided MUSIM gap) is used for collision handling between different MUSIM gaps - Option 3a: Use both Priority based solution and Keep solution for collision handling between different MUSIM gaps - Priority based solution is used when collided MUSIM gaps have different priority levels - Conditions when Keep solution is used are FFS. - Option 3b: Use both Priority based solution and Keep solution for collision handling between different MUSIM gaps - Conditions when priority based solution is used and conditions when Keep solution is used are FFS.

Our first preference is option 2, i.e. all colliding MUSIM gaps should be kept.

All the MUSIM gaps are requested by the UE, and if two MUSIM gaps are colliding based on UE request, it would mean that UE needs to use both of them. For example, one MUSIM gap may be used for serving cell measurement (sync/AGC) and another MUSIM gap may be used for paging, and depending on the distance between PO and SSB in NW B, it could happen that those two gaps are colliding, and in this case no gap should be dropped. We think it is a typical use case and should be supported.

Some companies raised the concern that two colliding MUSIM gaps may be used for accessing different frequency layers in NW B, or for other operations that cannot be done at same time. We understand UE would not request such colliding MUSIM gaps when it cannot use both of them, as it will only lead to less scheduling opportunity for this UE in NW A without any benefit for its operation in NW B. UE can make MUSIM gaps non-colliding by using larger gap periodicity or by utilizing different SMTC occasions with different offsets in an DRX cycle.

Therefore, we understand in most cases where UE requests colliding MUSIM gaps, UE would prefer to use all of them for NW B operation, and it is rare case where UE cannot avoid the collision and can only use one of the MUSIM gaps. We cannot agree to only use priority solution as in option 1, as it is too restrictive for UE implementation and precludes typical use cases.

Proposal 6: Keep solution (keep all collided MUSIM gap) is used for collision handling between different MUSIM gaps.

If Proposal 5 is not agreeable, then a compromise is both keep solution and priority solution are supported, and RAN4 to define applicability conditions for two solutions.

Proposal 7: If Proposal 5 is not agreeable, use both Priority based solution and Keep solution for collision handling between different MUSIM gaps. Conditions when Priority based solution is used and conditions when Keep solution is used are FFS.

Issue 2-2-3: Conditions on “keep solution” is used during collision between different MUSIM gaps - Proposals - Keep solution (keep all collided MUSIM gap) is used when - P1: RAN4 needs to further discuss the solution to indicate to the NW when gap keep rule will be applied within two MUSIM gaps. - P2: Both keep solution and priority solution could be supported. Whether keep solution or priority solution is used is based on UE request, i.e. UE can indicate whether all MUSIM gaps can be kept or not when there is collision between MUSIM gaps. FFS how to apply the priority solution when colliding MUSIM gaps are with equal preferred priority or without preferred priority - P3: Definition of colliding MUSIM gaps must be defined before agreement on the keep solution and related conditions can be agreed - P4: If RAN4 agree to consider kept solution during collision between different MUSIM gaps, we propose to take the condition that - the MUSIM gaps are regarded as collision based the collision definition, and - the collided MUSIM gaps are for paging reception, SSB measurement, or SI reading in the same frequency layer. - P5: Conditions when “keep solution” are used - when the collided MUSIM gaps are not physically overlapping and the distance between them is less than 4ms; - UE has the capability to handle the two collided MUSIM gaps when they are not overlapped however the distance between them is less than 4 ms - These “kept” MUSIM gaps measure Mos at the same frequency layer - P6: (from issue 2-2-2) Keep solution (keep all collided MUSIM gap) is used when these collided MUSIM gaps have the same priority Agreements: No Candidate options: Recommendations: continue discussion

On the exact condition, since whether UE can use all of the colliding MUSIM gaps depends on the usage of each colliding MUSIM gap, which is up to UE implementation, a reasonable way to select between keep and priority solutions is to base on UE request, i.e. UE can indicate whether all MUSIM gaps can be kept or not when there is collision between MUSIM gaps. This can be supported by introducing an explicit signalling in MUSIM gap request. For example, one extra bit can be added to the MUSIM gap request, and ‘1’ indicates the colliding MUSIM gaps can be used (Keep solution should be used), and ‘0’ indicates only one of the colliding MUSIM gaps can be used (Priority based solution should be used).

In last meeting, some companies suggest to use equal priority as the condition to apply Keep solution. In our view, priority is used to indicate the importance of the MUSIM gaps, and it may not be coupled with whether colliding MUSIM gaps can be kept or not where there is a collision. For example, MUSIM gaps for paging and SSB measurement may be suggested with different priority, but they can be used at same time. Thus, an explicit indication from UE side is more preferable.

Proposal 8: Whether keep solution or priority solution is used is based on UE request.

- Introduce explicit signalling in MUSIM gap request to allow UE to indicate whether all MUSIM gaps can be kept or not when there is collision between MUSIM gaps.

Collision between MUSIM and legacy MG

Issue 2-3-2: Solutions for collision between MUSIM gap and Type-1 MG or gap configured without priority - Proposals - P1: When a MUSIM gap collides with a legacy MG, requirements shall not apply if any one of the collided gaps is not assigned a priority. (vivo Apple xiaomi oppo) - Against: (Nokia) - P2: No requirements apply if the two gaps are configured with same MGRP (Huawei Ericsson) - Against: (Nokia) - P3: RAN4 to prioritize the gap with longer MGRP for the following MUSIM collision scenarios: Any of the collision gaps is Type-1 MG; NW-A doesn’t configure a priority associated with any of the collision gaps. (Ericsson Huawei ZTE MTK) - P4: MUSIM gaps are assumed to have higher priority than a Type-1 MG when either MUSIM gaps or Type-1 MG (or both) are not assigned priorities by the network. (Qualcomm) - Against: (Huawei xiaomi) - P5: Introduce priority for Type-1 MG when MUSIM gaps are configured when also having Type-1 measurement gaps allocated. (Nokia) - Against: (Qualcomm Huawei xiaomi ) - P6: The sharing rule solution could be considered. (xiaomi) - Against: (Qualcomm Huawei) Agreements: No Candidate options: Recommendations: Suggest to focus on P1, P2 and P3 since they have relative more supporters and less objections.

In last meeting, some companies raised the concern that some NW may not support the signaling to configure type-2 MG (e.g. NW does not support con-MG) but support configuring MUSIM gaps, so the collision handling between MUSIM gaps and type-1 MG should be considered. We think it is a valid concern, and prioritizing gap with longer MGRP is also reasonable because otherwise gap with longer MGRP will be completely dropped.

The next question is whether to extend the principle to scenario where MUSIM gaps are not configured with a priority. Since it was agreed in last meeting (Issue 2-1-2) that NW A assigns priority levels to all configured periodic MUSIM gaps, and aperiodic MUSIM gap has either configured priority or by default the highest priority, there is no need to consider the extension.

The next question is what happens when the colliding gaps are with same MGRP. In our view, no requirements shall apply because the collision cannot be resolved in an easy way. Basically, when a legacy MG and a MUSIM gap collide, NW A should either configure priority for all colliding gaps, or to configure different MGRPs for all colliding gaps.

Proposal 9: When a MUSIM gap collides with a Type-1 MG, prioritize the gap with longer MGRP. No requirements apply if the two gaps have same MGRP.

Collision between MUSIM gaps and NW A signals

Issue 2-4-3: Collision between SMTC and MUSIM gaps for handover and Scell activation - Proposals - P1: Collisions between other RRM procedures and MUSIM gaps are handled in the same way as collisions between RRM procedures and legacy MG, i.e., no special handling solution is defined. (Qualcomm MTK xiaomi Huawei Nokia vivo oppo) - P2: RAN4 to define requirements for the collision between MUSIM gaps with Handover, Scell activation and SI update. When NW-A’s RS resources for one-shot RRM procedure (Handover, Scell activation, SI update) collide with MUSIM gaps, MUSIM gaps should have lower priority; Periodic paging monitoring or one-shot procedure in NW-B Idle mode, such as On-demand SI reading have higher priority than Measurements procedures for both NW-A and NW-B (Ericsson) - P3: When NW-A’s uplink signals for one-shot RRM procedure(Handover, Scell activation) collide with MUSIM gaps, MUSIM gaps should have lower priority, such as NW-A’s PRACH and CSI-RS reporting for Scell activation should be prioritized. (Ericsson) - P4: Add a high-level clarification in RAN4 spec that during one-shot procedure such as Scell activation, SI update and so on, UE is not expected to enable MUSIM gaps unless existing RRM requirement for the corresponding one-shot procedure can be met. (Apple Ericsson) Agreements: No Candidate options: Recommendations: Discuss at next meeting

We do not support to define special collision between MUSIM gaps and SMTC for handover and SCell activation. While we can understand the point, we are not sure if RAN4 needs to specify such details for MUSIM gaps when the corresponding prioritization has not been specified for legacy MGs, i.e. it is already possible to have legacy MG colliding with those procedures, but RAN4 has not defined how to handle the collision between NW A MG and all the procedures.

For example, the SCell activation delay can be several SMTC periods, and when MG is configured, it could happen that MG is colliding with SMTC of the SCell. It is clear that UE cannot perform MG based RRM measurement and SCell activation at the same time, and as such the requirements for one of the procedures need to be relaxed, but so far RAN4 has not defined the what happens in this case. We believe collision with legacy MG is more relevant than collision with MUSIM gaps in real deployment.

In addition, besides legacy MG and MUSIM gaps, there are also other sources of interruption, e.g. measurement of deactivated SCell, SRS carrier or antenna switching, BWP switching, etc. The question is whether RAN4 should define requirements or prioritization for all collisions between all interruption sources and all important RRM procedures/important transmissions or receptions.

Based on above, we suggest not to define specific handling for collision between MUSIM gaps and specific RRM procedures / transmissions or receptions in NW A.

Proposal 10: No special handling is defined for collision between SMTC and MUSIM gaps for handover and SCell activation.

Conclusions

In this paper we provided our views on collision handling related to MUSIM gaps.

Proposal 1: NW A should maintain the relative priority among MUSIM gaps as indicated by UE when configuring priority for MUSIM gaps.

Proposal 2: RAN4 not to define constraints on MUSIM gap priority indication from UE side.

Proposal 3: Aperiodic MUSIM gap by default has the highest priority level. No need for UE to request or for NWA to configure a priority for aperiodic gap.

Proposal 4: Collisions between gaps are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority.

Proposal 5: Postpone the definition for collisions between MUSIM gaps after RAN4 has conclusion on collision handling between MUSIM gaps.

Proposal 6: Keep solution (keep all collided MUSIM gap) is used for collision handling between different MUSIM gaps.

Proposal 7: If Proposal 5 is not agreeable, use both Priority based solution and Keep solution for collision handling between different MUSIM gaps. Conditions when Priority based solution is used and conditions when Keep solution is used are FFS.

Proposal 8: Whether keep solution or priority solution is used is based on UE request.

- Introduce explicit signalling in MUSIM gap request to allow UE to indicate whether all MUSIM gaps can be kept or not when there is collision between MUSIM gaps.

Proposal 9: When a MUSIM gap collides with a Type-1 MG, prioritize the gap with longer MGRP. No requirements apply if the two gaps have same MGRP.

Proposal 10: No special handling is defined for collision between SMTC and MUSIM gaps for handover and SCell activation.

Reference

- R4-2306357, WF on NR Dual TxRx Multi-SIM, vivo